3GPP TSG-RAN WG2 Meeting #128	R2-2410948

# Orlando, USA, 18 - 22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5198 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scope of interFreqL1-MeasConfig-r18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For each supported BC, the UE can indicate whether it supports interFreqL1-MeasConfig-r18 defined as: interFreqL1-MeasConfig-r18 Indicates whether UE supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s). This definition does not indicate in which band(s) the candidate cell(s) can be, so it can be any band where candidate cells(s) can be. However, If the SSBs of candidate cells are all in the serving BC, the UE can take advantage of its CA capability to measure them, while if they are not in the serving BC, this is not possible so extra resources must be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new per UE capabilty that indicates that, for each BC in which the UE indicates support of interFreqL1-MeasConfig-r18, the UE only supports inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) in the BC. Impact Analysis Impacted 5G architecture options: NR, NR-DC Impacted functionality: Inter-frequency L1 measurements for LTM Inter-operability: If the network is implemented according to the CR and UE is not, there is no inter-operability issue. If UE is implemented according to the CR and the network is not, the network might configure inter-frequency L1 measurements of SSB of candidate cells that are not supported by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs that indicate support inter-frequency L1 measurements for LTM cannot indicate that they only support such measurements for SSB of candidate cells in the BC(s) in which they indicate support of inter-frequency L1 measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.306 CR 1222 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.3	UE capability information elements

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE {

concurrentPerUE-OnlyMeasGap-r17         ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17    ENUMERATED {supported}

}                                                                               OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

nr-NeedForGapNCSG-Reporting-r17         ENUMERATED {supported}                  OPTIONAL,

eutra-NeedForGapNCSG-Reporting-r17      ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-1 per FR Network controlled small gap (NCSG)

ncsg-MeasGapPerFR-r17                   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-2 Network controlled small gap (NCSG) supported patterns

ncsg-MeasGapPatterns-r17                BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-1-3 Network controlled small gap (NCSG) supported NR-only patterns

ncsg-MeasGapNR-Patterns-r17             BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL,

rrm-RelaxationRRC-ConnectedRedCap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 25-3: Parallel measurements with multiple measurement gaps

parallelMeasurementGap-r17              ENUMERATED {n2}                         OPTIONAL,

condHandoverWithSCG-NRDC-r17            ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-r17              ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-ENDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NEDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NRDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NPN-r17          ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R4 25-1: Parallel measurements on multiple SMTC-s for a single frequency carrier

parallelSMTC-r17                        ENUMERATED {n4}                         OPTIONAL,

-- R4 19-2-1 Concurrent measurement gaps for EUTRA

concurrentMeasGapEUTRA-r17              ENUMERATED {supported}                  OPTIONAL,

serviceLinkPropDelayDiffReporting-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-4 Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter

ncsg-SymbolLevelScheduleRestrictionInter-r17  ENUMERATED {supported}            OPTIONAL

]],

[[

eventD1-MeasReportTrigger-r17           ENUMERATED {supported}                  OPTIONAL,

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               OPTIONAL

]],

[[

interSatMeas-r17                            ENUMERATED {supported}              OPTIONAL,

deriveSSB-IndexFromCellInterNon-NCSG-r17    ENUMERATED {supported}              OPTIONAL

]],

[[

-- R4 31-1 Enhanced L3 measurement reporting for unknown SCell activation if the valid L3 measurement results are available

l3-MeasUnknownSCellActivation-r18           ENUMERATED {supported}              OPTIONAL,

-- R4 31-3 Shorter measurement interval for unknown SCell activation

shortMeasInterval-r18                       ENUMERATED {supported}              OPTIONAL,

nr-NeedForInterruptionReport-r18            ENUMERATED {supported}              OPTIONAL,

measSequenceConfig-r18                      ENUMERATED {supported}              OPTIONAL,

cellIndividualOffsetPerMeasEvent-r18        ENUMERATED {supported}              OPTIONAL,

eventD2-MeasReportTrigger-r18               ENUMERATED {supported}              OPTIONAL,

-- R4 32-1: Concurrent gaps with Pre-MG in a FR

concurrentMeasGapsPreMG-r18                 ENUMERATED {supported}              OPTIONAL,

-- R4 32-2: Support for dynamic collisions

dynamicCollision-r18                        ENUMERATED {supported}              OPTIONAL,

-- R4 32-3: Concurrent gaps with NCSG in a FR

concurrentMeasGapsNCSG-r18                  ENUMERATED {supported}              OPTIONAL,

-- R4 32-4: Inter-RAT EUTRAN measurements without gap and outside active DL BWP

eutra-NoGapMeasurementOutsideBWP-r18        ENUMERATED {supported}              OPTIONAL,

-- R4 32-5: Inter-RAT EUTRAN measurement without gap and within active DL BWP

eutra-NoGapMeasurementInsideBWP-r18         ENUMERATED {supported}              OPTIONAL,

-- R4 32-6: Effective measurement window for inter-RAT EUTRAN measurements

eutra-MeasEMW-r18                           BIT STRING (SIZE(6))                OPTIONAL,

-- R4 32-7: Simultaneous reception of NR data and EUTRAN CRS with different numerology

concurrentMeasCRS-InsideBWP-EUTRA-r18       ENUMERATED {supported}              OPTIONAL,

-- R4 39-2a: SSB based inter-frequency L1-RSRP measurements with measurement gaps

ltm-InterFreqMeasGap-r18                    ENUMERATED {supported}              OPTIONAL,

dummy-ltm-FastUE-Processing-r18             SEQUENCE {

fr1-r18                                    ENUMERATED {ms10, ms15},

fr2-r18                                    ENUMERATED {ms10, ms15},

fr1-AndFR2-r18                             ENUMERATED {ms20, ms30}

}                                                                                OPTIONAL,

rach-LessHandoverInterFreq-r18              ENUMERATED {supported}               OPTIONAL,

enterAndLeaveCellReport-r18                 ENUMERATED {supported}               OPTIONAL,

bestCellChangeReport-r18                    ENUMERATED {supported}               OPTIONAL,

secondBestCellChangeReport-r18              ENUMERATED {supported}               OPTIONAL

]],

[[

ltm-InterFreq-r18                           ENUMERATED {supported}               OPTIONAL,

ltm-MCG-NRDC-r18                            ENUMERATED {supported}               OPTIONAL,

ltm-RACH-LessDG-r18                         ENUMERATED {supported}               OPTIONAL,

ltm-RACH-LessCG-r18                         ENUMERATED {supported}               OPTIONAL,

ltm-Recovery-r18                            ENUMERATED {supported}               OPTIONAL,

ltm-ReferenceConfig-r18                     ENUMERATED {supported}               OPTIONAL,

ltm-MCG-NRDC-Release-r18                    ENUMERATED {supported}               OPTIONAL,

-- R4 39-7: Faster UE processing time during cell switch

ltm-FastUE-Processing-r18                   SEQUENCE {

fr1-r18                                    ENUMERATED {ms10, ms15}          OPTIONAL,

fr2-r18                                    ENUMERATED {ms10, ms15}          OPTIONAL,

fr1-AndFR2-r18                             ENUMERATED {ms20, ms30}          OPTIONAL

}                                                                                OPTIONAL,

ntn-NeighbourCellInfoSupport-r18            ENUMERATED {supported}               OPTIONAL

]],

[[

ltm-interFreqL1-OnlyInBC-r18                ENUMERATED {supported}               OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=        SEQUENCE {

intraAndInterF-MeasAndReport            ENUMERATED {supported}                  OPTIONAL,

eventA-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

...,

[[

handoverInterF                          ENUMERATED {supported}                  OPTIONAL,

handoverLTE-EPC                         ENUMERATED {supported}                  OPTIONAL,

handoverLTE-5GC                         ENUMERATED {supported}                  OPTIONAL

]],

[[

sftd-MeasNR-Neigh                       ENUMERATED {supported}                  OPTIONAL,

sftd-MeasNR-Neigh-DRX                   ENUMERATED {supported}                  OPTIONAL

]],

[[

dummy                                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                  ENUMERATED {supported}              OPTIONAL,

dummy                                       ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                       ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}          OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL,

-- R4 6-2: Support of beam level Early Measurement Reporting

idleInactiveNR-MeasBeamReport-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

increasedNumberofCSIRSPerMO-r16             ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersFR2-2-r17 ::=           SEQUENCE {

handoverInterF-r17                          ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC-r17                         ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC-r17                         ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r17               ENUMERATED {supported}              OPTIONAL,

...

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP