3GPP TSG-RAN2 Meeting # 128	R2-2411106

# Orlando, USA,  18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2002 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to multi-PUSCH configured grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm, Apple, vivo, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | First, in the current MAC specification, the definition of multi-PUSCH configured grant is captured as follows: Multi-PUSCH configured grant: A configured grant configuration that includes multiple consecutive configured uplink grants within a single periodicity.. While for R16 NR-U, similar mechanism has been supported that multiple UL grants can occur within a single CG periodicity. Specifically, the mechanism is configured with cg-nrofSlots and cg-nrofPUSCH-InSlot, which indicate a number of consecutive slots within a CG period and a number of consecutive PUSCH allocations within a slot. The main differences between the NR-U mechanism and the R18 multi-PUSCH CG are listed as following: - NR-U can support mini-slot/symbol level CG occasion configuration. And there should be no gap betwee the CG occasions within a slot to avoid the resource to be grabbed by another LBT - NR-U can support both retransmission and new transmission on the multiple PUSCH CG resources. While, the multi-PUSCH CG introduced in XR can only support new transmission - CG-UCI is sent over NR-U multiple CG, while for XR, UTO-UCI is sent for indicating whether the resource is used. The current definition of multi-PUSCH configured grant is not clear to distinguish multi-PUSCH CG and the similar NR-U mechanism on the above aspects of differences. This should be further clarified. Next, in clause 5.8.2 in the following sentence. The MAC entity shall not include the UL-SCH resource of a configured uplink grant not available for use in its procedures (e.g. in clause 5.4.4).. Clause 5.4.1 should also be referenced as it is more related to UL-SCH resource determination and allocation. |
| Summary of change: |  | 1/ Clarify what is multi-PUSCH configured grant to distinguish it from the similar mechaniem introduced for NR-U. 2/ Add 5.4.1 as the referred clause about MAC UL-SCH resource related procedures in clause 5.8.2. For both changes: Impact analysis: Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: Multi-PUSCH configured grant Inter-operability: There is no inter-operability issue for the CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the first change the definition of multi-PUSCH configured grant is not clear. Without the second change, the spec is confusing for the readers with an incomplete reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===================================CHANGE BEGINS====================================

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

A2X communication: A communication to support A2X services leveraging PC5 reference points, as defined in TS 23.256 [31]. A2X services are realized by various types of A2X applications, e.g., BRID or DAA.

Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000 m and take-off/landing altitudes down to 3000 m.

BWP for SRS for positioning Tx frequency hopping: For SRS for positioning Tx frequency hopping, separate BWP configuration outside BWP configuration for data transmission.

Dedicated SL-PRS resource pool: A sidelink resource pool which can be used for the transmission of SL-PRS and cannot be used for the transmission of PSSCH.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signaling. In the dormant BWP, the UE stop monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured.

DRX group: A group of Serving Cells that is configured by RRC and that have the same DRX Active Time.

eRedCap UE: A UE with enhanced reduced capabilities as specified in clause 4.2.22.1 of TS 38.306 [25].

HARQ information: HARQ information for DL-SCH, for UL-SCH, or for SL-SCH transmissions consists of New Data Indicator (NDI), Transport Block Size (TBS), Redundancy Version (RV), and HARQ process ID.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes.

Listen Before Talk: A procedure according to which transmissions are not performed if the channel is identified as being occupied, see TS 37.213 [18].

LTM candidate cell: A candidate cell configured for LTM as defined in TS 38.331 [5].

Msg3: Message transmitted on UL-SCH containing a C-RNTI MAC CE or CCCH SDU, submitted from upper layer and associated with the UE Contention Resolution Identity, as part of a Random Access procedure.

Multi-path: Mode of operation of a UE in RRC_CONNECTED configured with one direct path on which the UE connects to gNB using NR Uu, and one indirect path on which the UE connects to the same gNB via another UE using PC5 unicast link or non-3GPP connection (N3C).

Multi-PUSCH configured grant: A configured grant configuration configured with nrOfSlotsInCG-Period (see TS 38.331 [5]). It includes multiple consecutive configured uplink grants within a single periodicity.

N3C indirect path: In Multi-path, the indirect path using Non-3GPP Connection between remote UE and relay UE.

NCR-Fwd: NCR-node function, which performs amplifying-and-forwarding of UL/DL RF signals between gNB and UE. The behavior of the NCR-Fwd is controlled according to the side control information received by the NCR-MT from a gNB.

NCR-MT: NCR-node entity which communicates with a gNB via a control link to receive side control information. The control link is based on NR Uu interface.

NCR-node: RAN node comprising NCR-MT and NCR-Fwd.

Non-terrestrial network: An NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [19] and ProSe communication (including ProSe non-Relay, UE-to-Network Relay and UE-to-UE Relay communication (including ProSe UE-to-UE Relay communication with integrated discovery)) as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay discovery, ProSe UE-to-Network Relay discovery and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink transmission: Any NR Sidelink-based transmission, including transmission for NR sidelink discovery, transmission for NR sidelink communication, transmission for Ranging/Sidelink Positioning, and transmission for A2X communication.

PDCCH occasion: A time duration (i.e. one or a consecutive number of symbols) during which the MAC entity is configured to monitor the PDCCH.

Positioning SRS Bandwidth Aggregation: Transmission of positioning SRS on multiple carriers in RRC_CONNECTED and RRC_INACTIVE where the positioning SRS resources are linked in RRC configuration as defined in TS 38.331 [5].

PRS Processing Window: A time window during which UE may perform PRS measurement inside the active DL BWP with the same numerology as the active DL BWP without measurement gap.

Ranging/Sidelink Positioning: AS functionality enabling ranging-based services and sidelink positioning as specified in TS 23.586 [30].

RB set: A RB set refers to a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum as defined in TS 37.213 [18].

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [25].

Serving Cell: A PCell, a PSCell, or an SCell in TS 38.331 [5].

Shared SL-PRS resource pool: A sidelink resource pool which can be used for the transmission of both SL-PRS and PSSCH.

Sidelink transmission information: Sidelink transmission information included in an SCI for an SL-SCH transmission or SL-PRS transmission with or without SL-SCH transmission on Shared SL-PRS resource pool as specified in clause 8.3 and 8.4 of TS 38.212 [9] consists of Sidelink HARQ information including NDI, RV, Sidelink process ID, HARQ feedback enabled/disabled indicator, Sidelink identification information including cast type indicator, Source Layer-1 ID and Destination Layer-1 ID, and Sidelink other information including CSI request, SL-PRS request, SL-PRS resource ID, a priority, a communication range requirement and Zone ID and COT sharing information.

SL-PRS delay budget: Delay budget before which the SL-PRS is expected to be transmitted by the Tx UE.

SL-PRS transmission information on Dedicated SL-PRS resource pool: SL-PRS transmission information on Dedicated SL-PRS resource pool is included in an SCI for an SL-PRS transmission on Dedicated SL-PRS resource pool, as specified in TS 38.212 [9], consisting of

-	SL-PRS identification information, including cast type indicator, source ID and destination ID;

-	SL-PRS transmission other information, including SL-PRS priority, SL-PRS request, SL-PRS resource ID and resource reservation period.

SRS for positioning Tx frequency hopping: Transmit frequency hopping of positioning SRS in RRC_INACTIVE and RRC_CONNECTED.

SRS positioning validity area: An area consisting of a list of cells within which the corresponding positioning SRS configuration is considered as valid.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG depending on if the MAC entity is associated to the MCG or the SCG, respectively. Otherwise the term Special Cell refers to the PCell. A Special Cell supports PUCCH transmission and contention-based Random Access, and is always activated.

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

UE-gNB RTT: For non-terrestrial networks, the sum of the UE's Timing Advance value (see TS 38.211 [8] clause 4.3.1) and kmac.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [20], between nearby UEs, using E-UTRA technology but not traversing any network node.

NOTE 1:	A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until it is stopped or expires (e.g. due to BWP switching). When the MAC entity applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

NOTE 2:	In this version of the specification, the SRS in the procedural description includes Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE as in clause 5.26. Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE is treated the same as SRS by the UE unless explicitly stated otherwise.

===================================NEXT CHANGE====================================

### 5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

Only configured grant Type 1 can be configured for CG-SDT or for RACH-less LTM cell switch or for RACH-less handover. CG-SDT can only be configured on initial BWP.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-CS-RNTI: CS-RNTI for CG-SDT retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-RRC-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for RACH-less handover;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration;

-	timeReferenceH-SFN: H-SFN used for determination of the offset of a resource in time domain. The UE uses the closest H-SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

RRC configures the following parameter when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process;

-	cg-SDT-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial CG-SDT transmission with CCCH message when the UE shall not autonomously retransmit the HARQ process;

-	cg-RRC-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial transmission of RACH-less handover and RACH-less LTM cell switch when the UE shall not autonomously retransmit the HARQ process.

RRC configures the following parameter when a multi-PUSCH configured grant is configured:

-	nrofSlotsInCG-Period: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

RRC configures the following parameter when UTO-UCI (as specified in clause 9.3 in TS 38.213 [6]) is configured for a configured grant:

-	nrofBitsInUTO-UCI: number of bits in a UTO-UCI bitmap.

For a configured uplink grant, the MAC entity shall:

1> if its associated configured grant is configured with UTO-UCI and it has not been indicated to the lower layers as unused for PUSCH transmission; or

1> if its associated configured grant is not configured with UTO-UCI:

2> if it is associated with a multi-PUSCH configured grant and meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]; or

2> if it is not associated with a multi-PUSCH configured grant:

3> consider it available for use.

The MAC entity shall not include the UL-SCH resource of a configured uplink grant not available for use in its procedures (e.g. in clause 5.4.1 and 5.4.4).

For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI.

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

If cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, after an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

If cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is configured, after an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	((timeReferenceH-SFN × numberOfSFNperH-SFN + timeReferenceSFN)
	× numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × 1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For a multi-PUSCH configured grant Type 1, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

3>	if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27 (i.e., initial transmission for CG-SDT):

4>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured uplink grant:

5>	select this SSB.

4>	else if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is not above cg-SDT-RSRP-ThresholdSSB:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	if SSB is selected above:

4>	indicate the SSB index to the lower layer;

4>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

2>	if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received:

3>	if there is data available for transmission for at least one RB configured for SDT:

4>	initiate Random Access procedure in clause 5.1.

NOTE 1:	Void.

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the TCI state ID field in LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

NOTE 1a:	When there is an ongoing RACH-less LTM cell switch, the configured grant Type 1 which is not specifically configured for LTM (see cg-LTM-Configuration in TS 38.331 [5]) is not used.

NOTE 1b:	After completion of LTM cell switch, the UE stops using the grant configured for RACH-less LTM cell switch (see cg-LTM-Configuration in TS 38.331 [5]).

For the uplink grant configured for configured grant Type 1 for RACH-less handover, if the configured uplink grant is valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after the initial transmission of RACH-less handover has been performed according to clause 5.4.1 and 5.33, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for the initial transmission of RACH-less handover (i.e., retransmission of initial transmission of RACH-less handover):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB corresponding to the configured uplink grant with SS-RSRP above cg-RRC-RSRP-ThresholdSSB is available:

2>	select an SSB with SS-RSRP above cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant;

2>	indicate the selected SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid;

2>	initiate Random Access procedure in clause 5.1.

NOTE 1A:	When the UE determines if there is an SSB with SS-RSRP above cg-RRC-RSRP-ThresholdSSB or cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

For a multi-PUSCH configured grant Type 2, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within the same periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

If cg-nrofPUSCH-InSlot or cg-nrofSlots is configured for a configured grant Type 1 or Type 2, the MAC entity shall consider the uplink grants occur in those additional PUSCH allocations as specified in clause 6.1.2.3 of TS 38.214 [7].

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured uplink grants.

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer, cg-RRC-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

====================================CHANGE ENDS====================================