3GPP TSG-RAN WG2 Meeting #128	R2-2411111

# Orlando, Florida, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1954 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SCS for Timing Advance Report MAC CE for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Qualcomm Inc., Huawei, HiSilicon, CMCC, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The unit of symbols used for the Timing Advance field in Timing Advance Report MAC CE for ATG is based on the SCS. As the BWP and by extension the SCS may change, it may not be clear which SCS that the UE should apply for computing the Timing Advance field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the Timing Advance field that the UE applies the SCS of the active BWP. Impact analysis: Impacted 5G architecture options: NR SA Impacted functionality: NR ATG Timing Advance Report Inter-operability: If the UE is implemented according to this CR but the network is not, there is a possible misalignment on the SCS applied by the UE, thus the network may misunderstand the Timing Advance that the UE applies. If the network is implemented according to this CR but the network is not, there is a possible misalignment on the SCS applied by the UE, thus the network may misunderstand the Timing Advance that the UE applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear what SCS that the UE shall apply when computing the Timing Advance field in SCS, and unclear for the network what is the Timing Advance that the UE applies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.56 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In principle agreed CR from RAN2#127bis in R2-2408807 |  |  |  |  |  |  |  |  |

NEXT CHANGE

### 6.1.3	MAC Control Elements (CEs)

. . . OMITTED . . .

#### 6.1.3.56	Timing Advance Report MAC CE

The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: Except for ATG in FR1, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1).

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarrier spacing of the active BWP the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

The length of the field is 14 bits.

Figure 6.1.3.56-1: Timing Advance Report MAC CE