3GPP TSG-RAN WG3#126 	R3-247103

Orlando, USA, November 18-22, 2024

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-CU LTM Dual Connectivity

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-242356 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC o Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint • Specify RRM requirements related to the above objectives as necessary [RAN4]

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. RAN3#125bis: Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered. WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The LTM Configuration IDs are allocated by the source CU. WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS. Continue to work on the solutions comparison and details…

This paper discusses additional considerations to take into account for the Inter-CU LTM procedure and support for NR-DC.

# 2	Discussion on Inter-CU LTM DC

## 2.1	General Aspects

Scenarios for NR-DC with LTM

The following are NR-DC scenarios applicable for Rel-19 LTM DC work scope:

- Inter-CU SCG LTM with unchanged MCG

- Inter-CU MCG LTM with SCG retained or released

Similarly, RAN2 has confirmed the support of coexistence of a) Inter-MN LTM and Intra-SN LTM and b) Inter-SN LTM and Intra-MN LTM, which RAN3 should study the necessary signaling.

Simultaneous Inter-CU MCG LTM and inter-CU SCG LTM Configuration

There was an agreement at RAN3#123bis meeting that inter-CU MCG LTM and inter-CU SCG LTM are not to be configured simultaneously.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Given that LTM is configured by a gNB-CU based on L3 measurements, there is no guarantee that e.g., the SN is aware that LTM has been configured at the MN (MCG), since the SN addition may have occurred prior to LTM being configured. Thus, the SN may independently decide to configure LTM in the SCG unless it is explicitly aware that LTM is already configured at the MN. A similar scenario can occur in the opposite direction. Therefore, RAN3 should study whether and how to avoid LTM being configured at both MCG and SCG.

To avoid simultaneous inter-CU LTM configuration at the MN and SN, one approach is to have an indication over Xn that informs the other node that inter-CU LTM is to be configured. With this information, the receiving node can avoid configuring inter-CU LTM simultaneously at both the Cell Groups. Likewise, a similar indication would be signalled upon inter-CU LTM being released.

Proposal 1: In case of NR-DC, MN indicates to SN about inter-CU MCG LTM configuration/de-configuration.

Proposal 2: In case of NR-DC, SN indicates to MN about inter-CU SCG LTM configuration/de-configuration.

## 2.2	Inter-CU SCG LTM DC

Support for Scenario: Inter-CU SCG LTM with unchanged MCG

RAN2#127 agreed to support only SN initiated Inter-CU SCG LTM in Rel-19. Further, regarding coexistence of Intra-CU MCG LTM and Inter-CU SCG LTM, it is likely that each candidate SN may prepare different SCG configuration with reference to SCG configuration at the source, this may result in bearer remapping (e.g. remapping of SCG bearers to the MCG) in some inter-CU LTM PSCell change scenarios. However, in our view the existing Rel-18 framework should be able to handle this.

Observation 1: The coexistence of Intra-CU MCG LTM and inter-CU SCG LTM, may result in bearer remapping (e.g. remapping of SCG bearers to the MCG) in some inter-CU LTM PSCell change scenarios, which can be handled by the existing Rel-18 framework.

## 2.3	Inter-CU MCG LTM DC

Support for Scenario: Inter-CU MCG LTM with SCG retained or released

With regard to coexistence of intra-CU SCG LTM with Inter-CU MCG LTM, the Inter-CU LTM PCell change will result in security configuration update at the SN (via SN Reconfiguration); this requires the UE to perform reconfiguration with sync with the PSCell, even if the PSCell remains unchanged.

Observation 2: Inter-CU LTM PCell change results in security configuration update at the SN and requires UE to perform reconfiguration with sync with the PSCell, even if the PSCell remains unchanged.

Given that, it is the decision of candidate MN during HO preparation to retain or release the SCG, there is a chance that some of the inter-CU MCG LTM candidate nodes during HO preparation may prepare MCG + SCG configuration (if source SN is retained) or MCG only configuration (if source SN is released). This may result into a combination of inter-CU MCG LTM preparation with SCG retained and SCG released.

For such a scenario, if the Inter-CU LTM PCell change results in PCell HO with SCG release at the target, then SCG may be added during the subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes either full SCG configuration or SCG reference + delta configuration.

Observation 3: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes either full SCG configuration or SCG reference + delta configuration.

Possible alternatives to handle the scenario above include the following:

- Option 1: Not including an inter-CU MCG LTM candidate that has prepared MCG only configuration. This requires RAN3 signaling enhancements.

- Option 2: Include all the inter-CU MCG LTM candidate nodes in the subsequent cell change configuration, irrespective of whether a candidate node has prepared MCG + SCG configuration or MCG only configuration. This would allow dual connectivity converted to single connectivity if the Inter-CU LTM PCell change results in PCell HO with SCG release at the target. This option also requires RAN3 signaling enhancements.

- Option 3: To enable SCG addition after SCG release, the candidate configuration for inter-CU MCG LTM must either include full SCG configuration or SCG reference + delta configuration. Such that, UE can perform RACH to the target PSCell and add SCG, if the inter-CU MCG candidate configuration stored at the UE for the target cell includes SCG full or SCG Reference + delta configuration. such that, if the candidate target cell is prepared with MCG and SCG configuration.

Proposal 3: RAN3 to further discuss handling of Inter-CU MCG LTM with SCG retained or released based on the options above as basis.

Support for scenario: Inter-CU MCG LTM - candidate node preparing different PSCell

In Inter-CU MCG LTM, we think, it is likely that a candidate MN may prepare a different PSCell even if it retains the source SN. Thus, we think the following scenarios are relevant to be considered in the scope of inter-CU MCG LTM:

- Scenario 1: Candidate MN retains the source PSCell

- Scenario 2: Candidate MN prepares a different PSCell

Accordingly, the handling upon Inter-CU MCG LTM execution also varies as follows:

- In scenario 1 though the PSCell is unchanged, the UE performing reconfiguration with sync as SN security key changes due to MN change.

- In scenario 2, PSCell HO shall be triggered.

Observation 4: In inter-CU MCG LTM, a candidate MN may prepare a different PSCell, which may result in PSCell HO during inter-CU MCG LTM execution.

Proposal 4: RAN3 to discuss solutions for supporting candidate MN preparing a different PSCell during inter-CU MCG LTM preparation.

## 2.4	Other Issues

Inter-CU LTM when NR-DC is Configured

In TS 37.340, different dual connectivity scenarios and their respective signalling for PSCell addition, change, release or retention are described. These cell changes or reconfigurations can be MN or SN initiated. Likewise, in Rel-18 various issues related to inter-SN PSCell change were discussed and agreements were made under S-CPAC feature that could be considered for LTM work in Rel-19.

In Rel-18 S-CPAC, data forwarding may be realized either as direct data forwarding or as indirect data forwarding. The same principles and framework may be reused for inter-CU SCG LTM scenarios. However, to enable data forwarding for inter-CU SCG LTM with subsequent cell change, since the new serving SN is not aware of configuration of other candidate SNs, the following should be considered:

- Transferring the Admitted PDU session resources configuration of each prepared candidate node to all the prepared candidate inter-CU SCG LTM nodes via Xn signaling to enable data forwarding after each inter-CU SCG LTM PSCell change.

  - Option 1: At LTM preparation, the source node signals the Admitted PDU session resources configuration of all the prepared candidate nodes to all the prepared candidate SNs.

  - Option 2: At the time of inter-CU SCG LTM PSCell change execution, the source node signals the Admitted PDU session resources configuration of all the prepared candidate nodes only to the next serving SN. The new serving SN would carry out the same procedure at the next inter-CU SCG LTM mobility execution.

Observation 3: The data forwarding framework and principles of Rel-18 S-CPAC may be re-used for inter-CU SCG LTM.

Proposal 5: To support direct and indirect data forwarding for inter-CU SCG for subsequent LTM, the configuration of Admitted PDU session resources of the prepared inter-CU SCG LTM candidate SNs is signalled to all the prepared candidate SNs.

Inter-CU SCG LTM preparation

In both MN initiated and SN initiated inter-CU SCG LTM preparation scenarios, the MN triggers the preparation of candidate SN by requesting the candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure. The MN may prepare one or multiple candidate target SNs. Likewise, the candidate SN prepares the LTM candidate configuration including SCG configuration and LTM CSI Resource configuration.

As part of existing S-CPAC functionality, the S-CPAC Request IE was introduced in the SN ADDITION REQUEST message to indicate that the SN change is for S-CPAC. A similar approach can be used to enable inter-CU SCG LTM, where a specific IE within the SN ADDITION REQUEST message indicates to the candidate target SN that the preparation is for inter-CU SCG LTM.

Proposal 6: Introduce a new IE in SN ADDITION REQUEST message to indicate that the SN change is for inter-CU SCG LTM.

# 3	Conclusions

Simultaneous Inter-CU MCG LTM and inter-CU SCG LTM Configuration

Proposal 1: In case of NR-DC, MN indicates to SN about inter-CU MCG LTM configuration/de-configuration.

Proposal 2: In case of NR-DC, SN indicates to MN about inter-CU SCG LTM configuration/de-configuration.

Support for Scenario: Inter-CU SCG LTM with unchanged MCG

Observation 1: The coexistence of Intra-CU MCG LTM and inter-CU SCG LTM, may result in bearer remapping (e.g. remapping of SCG bearers to the MCG) in some inter-CU LTM PSCell change scenarios, which can be handled by the existing Rel-18 framework.

Support for Scenario: Inter-CU MCG LTM with SCG retained or released

Observation 2: Inter-CU LTM PCell change results in security configuration update at the SN and requires UE to perform reconfiguration with sync with the PSCell, even if the PSCell remains unchanged.

Observation 3: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes either full SCG configuration or SCG reference + delta configuration.

Proposal 3: RAN3 to further discuss handling of Inter-CU MCG LTM with SCG retained or released based on the options above as basis.

Support for scenario: Inter-CU MCG LTM - candidate node preparing different PSCell

Observation 4: In inter-CU MCG LTM, a candidate MN may prepare a different PSCell, which may result in PSCell HO during inter-CU MCG LTM execution.

Proposal 4: RAN3 to discuss solutions for supporting candidate MN preparing a different PSCell during inter-CU MCG LTM preparation

Inter-CU LTM when NR-DC is Configured

Observation 3: The data forwarding framework and principles of Rel-18 S-CPAC may be re-used for inter-CU SCG LTM.

Proposal 5: To support direct and indirect data forwarding for inter-CU SCG for subsequent LTM, the configuration of Admitted PDU session resources of the prepared inter-CU SCG LTM candidate SNs is signalled to all the prepared candidate SNs.

Inter-CU SCG LTM preparation

Proposal 6: Introduce a new IE in SN ADDITION REQUEST message to indicate that the SN change is for inter-CU SCG LTM.

# References

- 3GPP TS 38.401

- RP-242356 NR mobility enhancements Phase 4, Intel (moderator)