3GPP TSG-RAN WG3 Meeting #126	R3-247143

Orlando, USA, 18-22 Nov 2024

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Discussion Continuous MDT collection

Document for:	Discussions & Approval

- Introduction

During the previous meeting, continuous MDT collection enhancements was agreed to be solved during Rel-19.

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Further discussions are needed on how to define a temporary identifier for Continuous MDT.

Further discussions are needed on the solution for measurement correlation in Continuous MDT.

In this contribution, we provide our understanding on the motivation and solutions of continuous MDT collection.

2	Discussion

During Rel-19 phase, the Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

To resolve this issue above, there seems several kinds of solutions on the table:

- Solution #1: Allocate the persistent “UEID” from CN/AMF

- Solution #2: Reuse the method proposed in SON/MDT (UE pointer solution). Transmitting a 'continuous flag' to the UE. The UE will then store this indication and include it in the report sent back to the RAN.

- Solution #3: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information.

Regarding Solution #1, this approach is associated with the RAN-based solution. In this method, the AMF can assign a UEID (or a random ID) to the UE, which is then communicated to the RAN. Consequently, the RAN can effectively identify that the measurement report originates from the same UE, even when transitioning between RRC states. However, there are several drawbacks that still need to be addressed:

- The CN is required to consistently allocate a persistent UE ID for each UE. This allocation process can potentially place an additional burden on the CN or OAM.

- The use of a persistent UE ID may not be suitable for scenarios involving inter-AMF handovers, which still can not keep the continuous measurement report from the same UE.

- The Solution #1, RAN-based solution, is not an ideal approach for resolving the issue of continuous data collection due to its significant burden on the CN and its infeasibility in scenarios involving inter-AMF handovers.

Regarding Solution #2, this approach involves the introduction of a 'continuous MDT' flag within the logged MDT configuration. When the UE transitions to idle mode, it collects the logged MDT measurements. Then, upon moving from the IDLE/INACTIVE to the CONNECTED state, the UE signals to the RAN, as part of the RRC Setup procedure, that it is configured for Continuous MDT. This enables the RAN to promptly configure the UE for Immediate MDT measurements.

However, it's worth noting that the same value of continuous MDT flag conflict arises in scenarios involving mobility, particularly when different vendor OAM are utilized. There may be concerns regarding the consistency of the configuration by the new RAN side compared to the previous configuration which means the continuous data collection for AI/ML purpose still can not be guaranteed.

- The Solution #2, a kind of UE-based solution, is also not an ideal approach for resolving the issue of continuous data collection.

For the solution #4, the approach is to introduce a "continuous MDT" indication, and the logged MDT configuration can also include a measurement duration parameter optionally. When the UE moves to idle mode, it can start collecting the logged MDT measurements based on this configuration. And then when the UE moves from IDLE/INACTIVE to CONNECTED, UE continues to collect the measurements by itself based on the configuration of measurement duration. Once the measurement duration has elapsed, the UE can stop collecting the measurements and report them to the RAN. In addition, supporting company brings another solution to support the continuous data collection.

Figure. Continuous collection indication in logged MDT measurement

- The Solution #3 to introduce an indication (i.e., continuous MDT indication) in the logged MDT measurement configuration to indicate UE to let UE collect measurement across various RRC state is the most straightforward and simple solution with minor standard impact.

Several companies raised concerns that the approach of collecting continuous historical information for AI/ML model training and inference would burden the UE since it may store a large amount of measurement data. However, these concerns are not justified.

Firstly, the measurement duration is used to determine the time range for a measurement. This means that UE will not store a large quantity of measurements as the duration would specify the amount of data collected for each measurement. Therefore, this means that the UE will only store measurements for the measurement duration specified in the MDT configuration.

Secondly, this approach is based on logged measurements. When the UE enters the connected state, the measurements stored in the UE can be reported to the RAN directly via available indication to RAN, thus there is no buffer issue, and if the continuous collection indication is available, the UE will continue to collect the measurements.

The only change is that the current logged MDT measurement should be allowed to be performed by UE in RRC Connected mode which is a stage 2 enhancement, there is no additional burden introduced in UE side compared with reusing current immediate MDT mechanism to perform the corresponding AI/ML related measurements.

- The Solution #3 does not bring additional burdens to the UE compared with other options.

In the context of AI/ML-based use cases, it's crucial to emphasize that measurements from a single UE hold significant value. A data-set comprising complete and continuous measurements plays a pivotal role in training effective AI/ML models and generating informed decisions. Looking ahead, there will likely be additional UE measurements required to support various AI/ML-based use cases.

And during the discussion for AI/ML based use cases in RAN2, RAN2 also found the necessity to enhance the air-interface to support continuous data collection for temporal beam prediction. Therefore, while this may impose some limitations on the UE, the trade-off is generally acceptable.

- For the sake of AI/ML RAN function, it is allowed to enhance MDT functions to resolve the issue of continuous data collection in R19.

- Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information across RRC states.

- The UE based approach can resolve both problem A and problem B.

3	Conclusion

- The Solution #1, RAN-based solution, is not an ideal approach for resolving the issue of continuous data collection due to its significant burden on the CN and its infeasibility in scenarios involving inter-AMF handovers.

- The Solution #2, a kind of UE-based solution, is also not an ideal approach for resolving the issue of continuous data collection.

- The Solution #3 to introduce an indication (i.e., continuous MDT indication) in the logged MDT measurement configuration to indicate UE to let UE collect measurement across various RRC state is the most straightforward and simple solution with minor standard impact.

- The Solution #3 does not bring additional burdens to the UE compared with other options.

- For the sake of AI/ML RAN function, it is allowed to enhance MDT functions to resolve the issue of continuous data collection in R19.

- Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information across RRC states.

- The approach proposed by us can resolve both problem A and problem B.