3GPP TSG-RAN3 Meeting #126	R3-247148

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	(TP to 38.769) Conclusion for Topology 2

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on the open issues for Topology 2 of Ambient IoT based on the progress in the previous RAN3 meetings.

# Discussion

## Radio Resource management

In the TR, the following is captured.

In NAS/UP based solutions, A-IoT radio resources can be requested in advance to the NAS/UP based communication with the A-IoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger A-IoT radio resource allocation, e.g., upon CN request or upon UE request, etc. Some aspects of A-IoT radio allocation may be preconfigured by OAM.

In the sub sections, we provide few call flows and options to show the Resource allocation can work in case of NAS/UP based solutions.

### NAS-based solution

In case of NAS based solution, two options are considered:

Option 1: A-IoT assistance information can be provided to the A-IoT enabled gNB by A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs

Option 2: A-IoT assistance information can be provided to the A-IoT enabled gNB by the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs.

In case of NAS based solution, A-IoT enabled gNB can allocate/coordinate AIoT radio resources, upon receiving “AIoT assistance info”, either

  - from the the A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs  (“UE request”)

  - from the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs (“CN request”)

### User-plane based solution

In case of User plane solution, three options are considered.

Option 1: A-IoT assistance information can be provided to the A-IoT enabled gNB by A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane

Option 2: A-IoT assistance information can be provided to the A-IoT enabled gNB by UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request

Option 3: A-IoT assistance information can be provided to the A-IoT enabled gNB by the AMF via the NGAP (e.g., after receiving assistance information from the AIoT CN node)

In case of User plane solution, A-IoT enabled gNB can allocate/coordinate AIoT radio resources, upon receiving “AIoT assistance info”, either from:

- Option 1: A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane (“UE request”)

- Option 2: UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request (“CN request”)

- Option 3: AMF via the NGAP after receiving assistance information from the AIoTF

For RRC/NAS based solution, capture in TR the different options via which the A-IoT enabled gNB in T2 can allocate/coordinate the A-IoT radio resources among AIoT enabled UE(s) e.g., upon receiving request/AIoT assistance info from A-IoT enabled UE or from AMF in NGAP or from UPF via GTP-U header (in case of UP based solution)

There were some doubts raised by some companies about the UE-Request based resource allocation citing this would need A-IoT enabled gNB to “learn” about the A-IoT enabled UE(s) and the A-IoT service messages should therefore be routed from AIoTF to AIoT-enabled gNB “in-band”. We don’t think it is an issue and what matters is that the A-IoT enabled gNB eventually knows that it must allocate A-IoT radio resources and for that it needs to know about the A-IoT service request (e.g., inventory or command), estimated number of devices

There are no drawbacks for UE-request based resource allocation compared to CN-request based resource allocation. In both cases, the A-IoT enabled gNB can allocate/coordinate A-IoT radio resources among readers as long it knows about the A-IoT service operation (e.g., type of A-IoT service request, estimated number of devices)

All solutions (RRC-based, NAS-based, UP-based) allow the A-IoT enabled gNB to allocate/coordinate of the usage of the A-IoT radio resources among UE Readers

Irrespective of RRC/NAS/user-plane based solution, the A-IoT enabled UE should be able to provide some A-IoT assistance information to the A-IoT enabled gNB e.g., requesting to release the previously provided A-IoT resources, requesting to provide more resources, to provide the A-IoT enabled UEs’ capabilities or interests (e.g., if it is no longer interested in performing A-IoT communication). We therefore propose the following:

Irrespective of RRC/NAS/user-plane based solution, in Topology 2, A-IoT enabled UE can provide assistance information to A-IoT enabled gNB e.g., can request the A-IoT enabled gNB to provide resources or release the previously provided A-IoT resources.

## Reader selection

In the last meeting, there was a discussion on how AIoTF can select UE Readers in RRC_IDLE and whether there are any impacts to the Paging function to support this. Before we discuss solutions, we should first confirm the requirement that UE Reader can be in any RRC state at the time of Reader selection and therefore the following is proposed:

A-IoT enabled UE can be in any RRC state at the time of Reader selection.

In Topology 2, A-IoT enabled UEs which are UE Readers are “discoverable” at the AIoTF e.g., via knowledge of A-IoT enabled UE’s location (via existing LCS procedures or ULI) or upon request from a person pushing a button on a hand-held UE Reader or based on implementation. Thereby, the AIoTF can select suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command)

In Topology 2, A-IoT enabled UEs which are UE Readers are “discoverable” at the AIoTF e.g., via knowledge of A-IoT enabled UE’s location (via existing LCS procedures or ULI) or upon request from a person pushing a button on a hand-held UE Reader or based on implementation

There were proposals to also enable A-IoT enabled gNB(s) to select A-IoT enabled UEs to act as UE Reader but that has several drawbacks e.g., A-IoT enabled gNB cannot select UE Reader(s) in RRC_IDLE (as UE context is released) whereas AIoTF can select UE Readers in RRC_IDLE as well (as CN entities/AMF have UE context even in RRC_IDLE). Further, this would lead to different solutions for T1 and T2 since if we agree that AIoTF selects A-IoT RAN node and BS Readers in T1.

Drawbacks of A-IoT enabled gNB selecting UE Reader(s) in T2

  - A-IoT enabled gNB cannot select UE Reader(s) in RRC_IDLE (as it doesn’t have the UE context)

  - Leads to different solutions for T1 and T2 (since AIoTF selects A-IoT RAN node in T1)

  - In T2, UE Reader is mainly “selected” based on external request (e.g., from a worker pushing a button in the UE Reader) and not based on UE Reader selection as per a certain criteria

In Topology 2, AIoTF can “select” suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command)

## UE Reader mobility

In our view, Topology 2 is primarily intended for the use case of handheld UE Readers (e.g., in a factory) where workers might be holding AIoT-enabled UEs and are interested in performing inventory over a certain portion of the factory (or send commands to the AIoT devices). These handheld UE Readers can be moving and therefore the architecture/procedures should support the mobility of UE Readers in Topology 2. Therefore, the following is proposed:

Topology 2 should support the use case of handheld UE Readers and therefore the mobility of UE Readers should also be considered.

In case of UE Reader mobility (e.g., handover), the source gNB might need to do a few things during handover preparation e.g., select a suitable target cell for the handover of UE Reader, forward certain UE context information (e.g., whether the UE Reader is authorized, AIoTF information so that the target gNB know where to send the received Inventory Reports). Therefore, the following is proposed:

In case of UE Reader mobility, the source AIoT enabled gNB (serving the UE Reader) might coordinate with the target AIoT enabled gNB to forward AIoT specific information (e.g., the UE Reader’s stored context). Details of coordination over Xn can be discussed in the normative phase.

## Aggregation of AIoT service operation results

SA2 has the following captured in the interim conclusions in the TRL

The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

Editor’s Note:    For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor’s Note:    How the aggregation can be done is FFS.

For RRC based solution, following options are possible:

- Option 1: A-IoT enabled gNB performs aggregation of the service operation results (including the removal of the duplicated devices records) from UE Readers (and BS Readers) before sending it to AIoTF

- Option 2: A-IoT enabled gNB sends service operation results without performing aggregation and AIoTF performs aggregation

In case of RRC based solution, RAN3 should discuss whether A-IoT enabled gNB needs to perform aggregation of the service operation results (including the removal of the duplicated devices records) from UE Readers (and BS Readers) before sending it to AIoTF, or whether it should send the service operation results to AIoTF without aggregation.

## Split RAN architecture aspects

In the TR, the following is captured for T1:

For Topology 1, architecture and protocol aspects of split RAN architecture are not studied.

Considering RAN3 deprioritized the split RAN architecture aspects and the lack of time in the study, the it is proposed to capture the same for T2 as well.

Split RAN architecture aspects are not studied for Topology 2 as well.

## Comparison of T2 transport options

In the Table below, we compare the three options by discussing the protocol stack, interface impacts and their pros vs. cons.

NAS-based

UP-based

|  | RRC-based solution | NAS-based solution | User plane based solution |
| --- | --- | --- | --- |
| Radio Resource management | A-IoT enabled gNB is aware of the contents of the A-IoT messages as it is received explicitly over XXAP and RRC and can perform Radio Resource management based on the contents of the A-IoT messages. However, it adds unnecessary complexity to RRC ASN.1 and need explicit signalling in XXAP. | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is received as a NAS container For Radio Resource management, A-IoT enabled gNB can be provided assistance information by AIoTF in NGAP (e.g., in Downlink NAS Transport) or by the A-IoT enabled UE over Uu about the A-IoT service option | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is sent over user plane For Radio Resource management, A-IoT enabled gNB can be provided assistance information by A-IoT enabled UE over Uu or via UPF (e.g., in GTP-U header) or via AMF |
| Protocols | No end-to-end protocol between the A-IoT enabled UE and the AIoTF | There is an end-to-end protocol (AIoT-AP) between the A-IoT enabled UE and the AIoTF. | There is an end-to-end protocol (AIoT-AP) between the A-IoT enabled UE and the AIoTF |
| Similar examples | RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC NAS-SM like solution This is like how NAS-SM is defined between UE and SMF and carried over NAS-MM/NGAP/RRC | SUPL like solution This is like how SUPL (Secure User Plane Location Protocol) is used to carry positioning measurements between a UE and location server PC3a like solution This is like how PC3a is used to transfer ProSe messages between UE and DDNMF |
| UE Reader selection | A-IoT enabled UE selection can be performed by A-IoT enabled gNB or AIoTF | A-IoT enabled UE selection performed by AIoTF | A-IoT enabled UE selection performed by AIoTF |
| Impacts to CN functions | Overload on AMF if there are lot of A-IoT devices | Overload on AMF if there are lot of A-IoT devices | UPF can handle IP traffic better. |
| Aggregation | A-IoT enabled gNB might need to perform aggregation of the Inventory/Command Reports from UE Readers (and BS Readers) before sending it to AIoTF | A-IoT enabled gNB can forward the Inventory/Command Reports to AIoTF without performing aggregation and AIoTF performs aggregation | A-IoT enabled gNB can send Inventory/Command Reports to AIoTF without performing aggregation and AIoTF performs aggregation |
| RAN2 impacts | RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of A-IoT related messages from A-IoT enabled gNB to A-IoT enabled UE RAN2 would need to define procedures when the A-IoT enabled UE’s serving gNB changes (e.g., due to mobility) or RRC state changes | RAN2 impact: No/minimal | RAN2 impacts: No/minimal |
| RAN3 impacts | RAN3 impact: High RAN3 would need to define explicit signalling over N2 for the A-IoT messages In case the serving gNB of the A-IoT enabled UE changes (e.g., due to mobility) or the RRC state of the A-IoT enabled UE changes, Xn coordination is needed to provide A-IoT enabled UE’s context from source gNB to target gNB | RAN3 impact: No/minimal No new signalling needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state | RAN3 impact: No/minimal No new signalling needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state. Existing handling of transfer of PDU session context upon UE mobility can be reused. |
| SA2 impacts | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoTF (Nac interface) | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoTF (Nac interface) and support the end-to-end AIoT-AP protocol | SA2 impact: Medium SA2 needs to define the interface between UPF and AIoTF (Nuc interface) and support the end-to-end AIoT-AP protocol |

RAN3 to capture in TR the “Table 1” which compares the three options (RRC/NAS/UP based) for transporting the AIoT upper layer information in Topology 2, lists their pros and cons along with their WG impacts.

## Conclusion

There is significant spec work needed in RAN2 and RAN3 to support RRC-based solution as listed in the Table in section 2.6 and copied below for quick reference:

- RAN2 would need to define new RRC signaling to support the reception/transmission of A-IoT related messages from A-IoT enabled gNB to A-IoT enabled UE

- RAN3 would need to define explicit signaling over XX for the A-IoT messages

- RAN2 would need to define procedures when the A-IoT enabled UE’s serving gNB changes (e.g., due to mobility) or RRC state changes

- In case the serving gNB of the A-IoT enabled UE changes (e.g., due to mobility) or the RRC state of the A-IoT enabled UE changes, Xn coordination is needed to provide A-IoT enabled UE’s context from source gNB to target gNB

- UE Reader would need to report this delay tolerant AIoT data over control plane RRC signaling to AIoT enabled gNB, creating burden and overload to CP signaling

Considering there are high impacts to the A-IoT enabled UE, AIoT enabled gNB, as well as the specification efforts for RAN2/RAN3 with RRC-based solution should not be pursued any more.

Given the high impacts to RAN2/RAN3, RRC-based solution is not considered in Rel-19.

NAS based solution may overload the AMF (e.g., if there are a lot of AIoT devices) and is also not yet considered by SA2 in the interim conclusions.

In contrast, to support User-plane based solution, existing session management procedures can be reused, and no dedicated signalling needs to be specified over RRC and XXAP. Also, there is no need to upgrade RAN and can be easily deployed by the operators without the need for RAN upgrade. Therefore, the following is proposed:

RAN3 recommends a user-plane based solution for UE Reader to communicate with the AIoTF in Topology 2

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Radio Resource management

2.1.1	NAS-based solution

Proposal 1: In case of NAS based solution, A-IoT enabled gNB can allocate/coordinate AIoT radio resources, upon receiving “AIoT assistance info”, either

a. from the the A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs  (“UE request”)

b. from the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs (“CN request”)

2.1.2	User-plane based solution

Proposal 2:	In case of User plane solution, A-IoT enabled gNB can allocate/coordinate AIoT radio resources, upon receiving “AIoT assistance info”, either from:

 Option 1: A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane (“UE request”)

 Option 2: UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request (“CN request”)

 Option 3: AMF via the NGAP after receiving assistance information from the AIoTF

Proposal 3:	For RRC/NAS based solution, capture in TR the different options via which the A-IoT enabled gNB in T2 can allocate/coordinate the A-IoT radio resources among AIoT enabled UE(s) e.g., upon receiving request/AIoT assistance info from A-IoT enabled UE or from AMF in NGAP or from UPF via GTP-U header (in case of UP based solution)

Observation 1:	There are no drawbacks for UE-request based resource allocation compared to CN-request based resource allocation. In both cases, the A-IoT enabled gNB can allocate/coordinate A-IoT radio resources among readers as long it knows about the A-IoT service operation (e.g., type of A-IoT service request, estimated number of devices)

Proposal 4:	All solutions (RRC-based, NAS-based, UP-based) allow the A-IoT enabled gNB to allocate/coordinate of the usage of the A-IoT radio resources among UE Readers

Proposal 5:	Irrespective of RRC/NAS/user-plane based solution, in Topology 2, A-IoT enabled UE can provide assistance information to A-IoT enabled gNB e.g., can request the A-IoT enabled gNB to provide resources or release the previously provided A-IoT resources.

2.2 Reader selection

Proposal 6:	A-IoT enabled UE can be in any RRC state at the time of Reader selection.

Proposal 7:	In Topology 2, A-IoT enabled UEs which are UE Readers are “discoverable” at the AIoTF e.g., via knowledge of A-IoT enabled UE’s location (via existing LCS procedures or ULI) or upon request from a person pushing a button on a hand-held UE Reader or basedon implementation

Observation 2:	Drawbacks of A-IoT enabled gNB selecting UE Reader(s) in T2

-	A-IoT enabled gNB cannot select UE Reader(s) in RRC_IDLE (as it doesn’t have the UE context)

-	Leads to different solutions for T1 and T2 (since AIoTF selects A-IoT RAN node in T1)

-	In T2, UE Reader is mainly “selected” based on external request (e.g., from a worker pushing a button in the UE Reader) and not based on UE Reader selection as per a certain criteria

Proposal 8:	In Topology 2, AIoTF can “select” suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command)

2.3 UE Reader mobility

Proposal 9:	Topology 2 should support the use case of handheld UE Readers and therefore the mobility of UE Readers should also be considered.

Proposal 10:	In case of UE Reader mobility, the source AIoT enabled gNB (serving the UE Reader) might coordinate with the target AIoT enabled gNB to forward AIoT specific information (e.g., the UE Reader’s stored context). Details of coordination over Xn can be discussed in the normative phase.

2.4 Aggregation of AIoT service operation results

Proposal 11:	In case of RRC based solution, RAN3 should discuss whether A-IoT enabled gNB needs to perform aggregation of the service operation results (including the removal of the duplicated devices records) from UE Readers (and BS Readers) before sending it to AIoTF, or whether it should send the service operation results to AIoTF without aggregation.

2.5 Split RAN architecture aspects

Proposal 12:	Split RAN architecture aspects are not studied for Topology 2 as well.

2.6 Comparison of T2 transport options

Proposal 13:	RAN3 to capture in TR the “Table 1” which compares the three options (RRC/NAS/UP based) for transporting the AIoT upper layer information in Topology 2, lists their pros and cons along with their WG impacts.

2.7 Conclusion

Proposal 14:	Given the high impacts to RAN2/RAN3, RRC-based solution is not considered in Rel-19.

Proposal 15:	NAS based solution may overload the AMF (e.g., if there are a lot of AIoT devices) and is also not yet considered by SA2 in the interim conclusions.

Proposal 16:	RAN3 recommends a user-plane based solution for UE Reader to communicate with the AIoTF in Topology 2

# Annex 1 : TP to 38.769

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF, UPF, AIoTF and A-IoT related functions, which is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

In Topology 2, the A-IoT enabled gNB may enable the coordination of the usage of the A-IoT radio resources among readers e.g., upon receiving “UE request” or upon receiving “CN request” from AIoTF/AMF/UPF and/or OAM (see section 6.5.1.3 for more details).

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter.

For Topology 2, architecture and protocol aspects of split RAN architecture is not studied.

#### 6.4.2.1 Reader selection

A-IoT enabled UE can be in any RRC state at the time of Reader selection.

In Topology 2, A-IoT enabled UEs which are UE Readers are “discoverable” at the AIoTF e.g., via knowledge of A-IoT enabled UE’s location (via existing LCS procedures or ULI) or upon request from a person pushing a button on a hand-held UE Reader or based on implementation

In Topology 2, AIoTF can select suitable A-IoT enabled UE(s) as UE Readers for performing A-IoT operations (e.g., Inventory/Command)

#### 6.4.2.2 Mobility of AIoT enabled UEs

Topology 2 supports the use case of handheld UE Readers, and the mobility of UE Readers is considered.

In case of UE Reader mobility, the source AIoT enabled gNB (serving the UE Reader) might coordinate with the target AIoT enabled gNB to forward AIoT specific information (e.g., the UE Reader’s stored context). Details of coordination over Xn will be discussed in the normative phase.

#### 6.5.1.3	A-IoT radio resource allocation in case of NAS/UP based solutions

In NAS/UP based solutions, A-IoT radio resources can be requested in advance to the NAS/UP based communication with the A-IoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger A-IoT radio resource allocation, e.g., upon CN request or upon UE request, etc. Some aspects of A-IoT radio allocation may be preconfigured by OAM.

In case of NAS-based solution, the A-IoT enabled gNB can allocate/coordinate AIoT radio resources, either upon receiving:

- “UE request” from the the A-IoT enabled UE via Uu after receiving the Inventory/Command Request NAS PDUs

- “CN request” from the AMF as part of Downlink NAS Transport in NGAP along with the Inventory/Command NAS PDUs.

In case of User plane solution, A-IoT enabled gNB can allocate/coordinate AIoT radio resources, either upon receiving:

- “UE request” from A-IoT enabled UE via Uu after receiving the Inventory/Command Request over user plane

- “CN request” from UPF as part of the GTP-U header in DL PDU Session Information (38.415) carrying the Inventory/Command Request

- “CN request” from AMF via the NGAP after receiving “assistance information” from the AIoTF.

### 6.4.2	Comparison of transport options for Topology 2

|  |  | RRC-based solution |  | NAS-based solution |  |  | User plane based solution |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Radio Resource management |  | A-IoT enabled gNB is aware of the contents of the A-IoT messages as it is received explicitly over XXAP and RRC and can perform Radio Resource management based on the contents of the A-IoT messages. However, it adds unnecessary complexity to RRC ASN.1 and need explicit signalling in XXAP. |  | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is received as a NAS container For Radio Resource management, A-IoT enabled gNB can be provided assistance information by AIoTF in NGAP (e.g., in Downlink NAS Transport) or by the A-IoT enabled UE over Uu about the A-IoT service option |  |  | A-IoT enabled gNB is unaware of the contents of the A-IoT messages as it is sent over user plane For Radio Resource management, A-IoT enabled gNB can be provided assistance information by A-IoT enabled UE over Uu or via UPF (e.g., in GTP-U header) or via AMF |
| Protocols |  | No end-to-end protocol between the A-IoT enabled UE and the AIoTF |  | There is an end-to-end protocol (AIoT-AP) between the A-IoT enabled UE and the AIoTF. |  |  | There is an end-to-end protocol (AIoT-AP) between the A-IoT enabled UE and the AIoTF |
| Similar examples |  | RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC |  | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC NAS-SM like solution This is like how NAS-SM is defined between UE and SMF and carried over NAS-MM/NGAP/RRC |  |  | SUPL like solution This is like how SUPL (Secure User Plane Location Protocol) is used to carry positioning measurements between a UE and location server PC3a like solution This is like how PC3a is used to transfer ProSe messages between UE and DDNMF |
| UE Reader selection |  | A-IoT enabled UE selection can be performed by A-IoT enabled gNB or AIoTF |  | A-IoT enabled UE selection performed by AIoTF |  |  | A-IoT enabled UE selection performed by AIoTF |
| Impacts to CN functions |  | Overload on AMF if there are lot of A-IoT devices |  | Overload on AMF if there are lot of A-IoT devices |  |  | UPF can handle IP traffic better. |
| Aggregation |  | A-IoT enabled gNB might need to perform aggregation of the Inventory/Command Reports from UE Readers (and BS Readers) before sending it to AIoTF |  | A-IoT enabled gNB can forward the Inventory/Command Reports to AIoTF without performing aggregation and AIoTF performs aggregation |  |  | A-IoT enabled gNB can send Inventory/Command Reports to AIoTF without performing aggregation and AIoTF performs aggregation |
| RAN2 impacts |  | RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of A-IoT related messages from A-IoT enabled gNB to A-IoT enabled UE RAN2 would need to define procedures when the A-IoT enabled UE’s serving gNB changes (e.g., due to mobility) or RRC state changes |  | RAN2 impact: No/minimal |  |  | RAN2 impacts: No/minimal |
| RAN3 impacts |  | RAN3 impact: High RAN3 would need to define explicit signaling over N2 for the A-IoT messages In case the serving gNB of the A-IoT enabled UE changes (e.g., due to mobility) or the RRC state of the A-IoT enabled UE changes, Xn coordination is needed to provide A-IoT enabled UE’s context from source gNB to target gNB |  | RAN3 impact: No/minimal No new procedures needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state |  |  | RAN3 impact: No/minimal No new procedures needed when the A-IoT enabled UE’s serving gNB changes or the A-IoT enabled UE changes its RRC state |
| SA2 impacts |  | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoTF (Nac interface) |  | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoTF (Nac interface) and support the end-to-end AIoT-AP protocol |  |  | SA2 impact: Medium SA2 needs to define the interface between UPF and AIoTF (Nuc interface) and support the end-to-end AIoT-AP protocol |

### 6.4.2	Conclusion for Topology 2

Given the high impacts to RAN2/RAN3, RRC-based solution is not to be considered in the normative phase.

NAS based solution may overload the AMF (e.g., if there are a lot of AIoT devices) and is also not yet considered by SA2 in the interim conclusions. Therefore, NAS-based solution is also deprioritized by RAN3.

RAN3 recommends a user-plane based solution for A-IoT enabled UE to communicate with the AIoTF in Topology 2.

In Topology 2, all solutions (RRC-based, NAS-based, UP-based) allow the A-IoT enabled gNB to allocate/coordinate of the usage of the A-IoT radio resources among UE Readers.

Irrespective of RRC/NAS/user-plane based solution, in Topology 2, A-IoT enabled UE can provide assistance information to A-IoT enabled gNB e.g., can request the A-IoT enabled gNB to provide resources or release the previously provided A-IoT resources.