3GPP TSG-RAN3 Meeting #126	R3-247150

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	16.4

Source: 	Qualcomm Incorporated

Title: 	(TP to 38.769) Locating Ambient IoT devices

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on locating an AIoT device based on the discussion and open issues identified in the previous meeting.

# Discussion

## Involvement of AIoTF in locating A-IoT devices

In our view, the new core network entity agreed by SA2 (referred as the AIoTF), that is responsible for forwarding AIoT specific messages (e.g., inventory/commands) from/to AF, can also be involved in locating the AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports). The following is therefore proposed:

AIoTF is involved in locating an AIoT device (e.g., collecting A-IoT location reports), for both Topology 1 and 2.

## Signalling details for A-IoT location reporting

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

In NR, to track UEs in RRC_CONNECTED, AMF can send a location report configuration to the gNB and the gNB sends the UE Location Information (ULI) by including the NR CGI (MCC + MNC + gNB ID + Cell Identity) and/or the Tracking Area Identity (TAI) to AMF.

In order to track the AIoT devices at reader granularity, we think we can define a procedure similar to locating reporting procedure, but a procedure between the AIoTF and Reader is defined (instead of AMF and gNB as in NR).

The AIoTF can send a AIoT location report config (e.g., indicating the AIoT device(s) for which location is requested, location reporting conditions) to a Reader and the Reader can send the AIoT device’s “location” (e.g., the “Reader ID” to which the AIoT device was last associated with). An example procedure is shown below:

Step 1: Reader receives a message from the AIoTF requesting the location information of one or more AIoT device(s) via an AIoT location report config.

- The AIoT devices for which the location information is requested can be identified by a AIoT device ID. FFS on the format of AIoT device ID (pending SA2 progress).

- The AIoT location report config can include when/how to report location information e.g., location to be reported just “one-time” or periodically or upon some other event being met. FFS on the type of events.

- AIoT location report config can be sent as part of Inventory Request/Command Request or can be sent separately.

Step 2: Reader stores the AIoT location report config and determines whether the condition to report the location information is satisfied.

Step 3: Upon determining that the condition to report location is satisfied, Reader sends an AIoT Location Report to the AIoTF with the Reader ID(s) associated to the requested AIoT device(s).

For both T1 and T2, Inventory Request and/or Command Request can include a “AIoT location reporting configuration”, indicating the AIoT device(s) for which location is requested and the location reporting conditions (e.g., periodicity).

For both T1 and T2, upon determining that the condition to report location is satisfied, the Reader can send the AIoT device’s “location” (e.g., the “Reader ID”) to the AIoTF in Inventory Report and/or Command Report.

- The AIoTF might want to obtain the Reader location independently, not necessarily along with  inventory or command. Further, the AIoTF might want to obtain location with a different periodicity compared to the periodicity with which Inventory/Command Reports are sent to AIoTF. We therefore propose to support an independent procedure for AIoTF to request location from Reader.

For both T1 and T2, AIoTF should be able to request Reader to provide the location of an AIoT device and obtain the requested location in a dedicated message (other than Inventory and Command)

## Reader location

Editor’s Note 1:	How to know the “reader” location is FFS.

For T1, the following options were discussed in the last meeting:

In Topology 1, the AIoTF knows the location of a reader supported by an AIoT RAN node by one of the following methods:

- Option 1a: AIoTF is configured by O&M about the readers supported by an AIoT RAN node and their location.

- Option 1b: AIoTF receives the list of readers supported by an AIoT RAN node and their location in an XXAP Setup/Configuration Update message received beforehand.

In our view, Option 1b (via the XX signaling procedure) is preferred compared to an OAM based solution (which is usually proprietary). This also aligns with our proposal in the companion paper where we propose that the AIoT RAN node indicates its list of supported readers to the AIoTF in a XX signaling procedure. The location of the readers can also be indicated via this XX signaling procedure.

In case of T1, the location of the Readers supported within an AIoT RAN node can be known via a XX signaling procedure, wherein the AIoT RAN node indicates its list of supported readers and their location to the AIoTF.

For T2, the following options were discussed in the last meeting:

In Topology 2, AIoTF can determine the location of the AIoT-enabled UE in the following ways:

- Option 2a: AIoT-enabled UE knows its location (e.g. if AIoT-enabled UE has GNSS info) and this is reported to the AIoTF in the Inventory/Command Report

- Option 2b: an identifier of the AIoT-enabled UE is reported to the AIoTF in the Inventory/Command Report and the AIoTF can determine the location of the identified UE using legacy LCS procedure(s).

- Option 2c: an identifier of the AIoT-enabled UE is reported to the AIoTF in the Inventory/Command Report and the AIoTF has determined the location of the identified UE via User location information reported by the AIoT-enabled gNB

In our view, it is easy to support Option 2b and Option 2c. AIoTF can obtain the location of the A-IoT enabled UE (acting as UE Reader) via existing LCS/ULI procedures (e.g., AIoTF can subscribe to AMF to obtain UE location obtained via ULI or location reports from LMF)

In case of T2, AIoTF can obtain the location of the A-IoT enabled UE (acting as UE Reader) via existing LCS/ULI procedures (e.g., AIoTF can subscribe to AMF to obtain UE location). The feasibility is to be confirmed via SA2.

## Multi-Reader based locating the A-IoT device

Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

- The AIoTF can select multiple A-IoT RAN nodes (in case of T1) or multiple UE Readers (in case of T2) to “triangulate” A-IoT devices better. In case an A-IoT device is in the coverage of multiple selected Readers, both the Readers may send their Reader ID or Reader location or signal strength measurements at the Reader to the AIoTF.

It is up to AIoTF implementation whether to select one or more readers for locating the A-IoT devices

## Finer granularities for locating AIoT device (besides Reader granularity)

The following Editor’s Notes are captured in the TR:

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

There are several Inventory/Command use cases listed in 22.840 which might potentially need to locate the AIoT device at a 3-5 m granularity.

Whether the Reader ID or Reader location is sufficient to locate the A-IoT device at a 3-5 m granularity indoors depends on the Reader coverage and the deployment of readers within an area of interest.

For example, if there is a single Reader in a whole factory which can inventory/command all the A-IoT devices in the factory e.g. for a 50 m coverage, then knowing just the reader ID or reader location won’t suffice to locate the A-IoT device at finer granularity e.g., within 3-5 m granularity.

In case there are multiple readers deployed in the factory and all these readers are used for locating the A-IoT devices, we can locate the A-IoT devices at a better granularity e.g., by knowing that the A-IoT device is in the intersection of the coverage of two readers.

Further, it is to be noted that RAN1 studied proximity determination and captured the following two solutions in the TR. In case of solution 2, measurements at reader side can also be utilized as shown below:

6.9.x	Proximity determination

Editor’s note: Proximity determination may be in a 6.9.x sub-clause, or another arrangement, depending on how the study proceeds.

Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item.

Solution 1:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

Solution 2:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Proximity determination based on device-side measurements is not considered.

Even for locating the A-IoT devices, reader side measurements can be utilized if such support is provided by RAN1.

If RAN1 supports reader-side measurements (being considered for proximity determination), reader can forward these “reader side measurements” to the AIoTF for locating the AIoT device at a finer granularity.

Agree the TP to 38.769 in Anex.

# Summary

2.1	Involvement of AIoTF in locating A-IoT devices

Proposal 1:	AIoTF is involved in locating an AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports), for both Topology 1 and 2.

2.2	Signalling details for A-IoT location reporting

Proposal 2:	For both T1 and T2, Inventory Request and/or Command Request can include a “AIoT location reporting configuration”, indicating the AIoT device(s) for which location is requested and the location reporting conditions (e.g., periodicity).

Proposal 3:	For both T1 and T2, upon determining that the condition to report location is satisfied, the Reader can send the AIoT device’s “location” (e.g., the “Reader ID”) to the AIoTF in Inventory Report and/or Command Report.

Proposal 4:	For both T1 and T2, AIoTF should be able to request Reader to provide the location of an AIoT device and obtain the requested location in a dedicated message (other than Inventory and Command)

2.3	Reader location

Proposal 5:	In case of T1, the location of the Readers supported within an AIoT RAN node can be known via a XX signaling procedure, wherein the AIoT RAN node indicates its list of supported readers and their location to the AIoTF.

Proposal 6:	In case of T2, AIoTF can obtain the location of the A-IoT enabled UE (acting as UE Reader) via existing LCS/ULI procedures (e.g., AIoTF can subscribe to AMF to obtain UE location). The feasibility is to be confirmed via SA2.

2.4	Multi-Reader based locating the A-IoT device

Proposal 7:	It is up to AIoTF implementation whether to select one or more than readers for locating the A-IoT devices

2.5	Finer granularities for locating AIoT device (besides Reader granularity)

Proposal 8:	If RAN1 supports reader-side measurements (being considered for proximity determination), reader can forward these “reader side measurements” to the AIoTF for locating the AIoT device at a finer granularity.

Proposal 9:	Agree the TP to 38.769 in Anex.

# 4 Annex (TP to 38.769)

## 6.9	Locating ambient IoT devices

### 6.9.1	General

Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g. by AIoTF sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

In case of Topology 1, the location of the Readers supported within an AIoT RAN node can be known via a XX signaling procedure, wherein the AIoT RAN node indicates its list of supported readers and their location to the AIoTF.

In case of Topology 2, AIoTF can obtain the location of the A-IoT enabled UE (acting as UE Reader) via existing LCS/ULI procedures (e.g., AIoTF can subscribe to AMF to obtain UE location). The feasibility is to be confirmed via SA2.

It is up to AIoTF implementation whether to select one or more than readers for locating the A-IoT devices.

If RAN1 supports reader-side measurements (being considered for proximity determination), reader can forward these “reader side measurements” to the AIoTF for locating the AIoT device at a finer granularity.

### 6.9.2 Signalling details for A-IoT location reporting

The study assumes that the AIoTF can be involved in locating an AIoT device (e.g., determining A-IoT location configuration, collecting A-IoT location reports), for both Topology 1 and 2.

The signalling details are shown in the Figure below.

Step 1:

- Reader receives a message from the AIoTF requesting the location information of one or more AIoT device(s) via an AIoT location report config.

- The AIoT devices for which the location information is requested can be identified by a AIoT device ID. FFS on the format of the AIoT device ID (pending SA2 progress).

- The AIoT location report config can include when/how to report location information e.g., location to be reported just “one-time” or periodically or upon some other event being met. FFS on the type of events.

- The AIoT location report config is sent as part of Inventory Request/Command Request or can be sent separately.

Step 2: Reader stores the AIoT location report config and determines whether the condition to report the location information is satisfied.

Step 3: Upon determining that the condition to report location is satisfied, Reader sends an AIoT Location Report to the AIoTF with the Reader ID(s) associated to the requested AIoT device(s).