3GPP TSG-RAN WG3 Meeting #126	R3-247159

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP for BLCR for SON for 38.300, 38.401,38.473, 38.413, 37.340, 38.423) MRO enhancement for R18 mobility mechanisms

Agenda Item:	10.2

Source:	Huawei

Document for:	other

# 1	Introduction

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms.

# 2	Discussion

## 2.1	MRO for LTM

In previous meeting, the progress was as follows:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change“successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

### 2.1.1	Failure scenarios

In last meeting we agreed that the CU identifies which node to send the information to. There was an agreement that the CU determine the failure type and send to the DU.

Reagrding the failure type determination, one crucial part of information needed for the node to distinguish the exact error type is the T_storeUEcontext. This is currently configured to the CU which means that the DU currently does not have access to this information.

The purpose of the timer is to easily define a threshold for what a successful handover is. This in turn depends on how rapid the assumed mobility out of the cell is. For example, in the legacy case, it may be assumed that a failure happening within 5 seconds after a completed handlover is considered as a failure in the source cell. But the difference with LTM is that the mobility decisions are based on L1 measurements, all candidates are already prepared, and the interruption time is much shorter. Therefore, the strategy may be completely different for LTM mobility. A stay time of 5 seconds may be considered as perfectly acceptable and the failure should in this case be considered as belonging to the target cell (too late). Using the same threshold for MRO decisions may limit the design of the RRM and MRO for LTM. It is also very likely that the thresholds used for ping pong is very different from the thresholds used for legacy handover but this is currently in the scope of the specification.

Therefore, it probably makes sense to consider the possibility that we specify a new, separate timer for LTM cell switch.

Add a new timer for LTM cell switch for LTM in the OAM requirements.

For failures shortly after successful handover, the source cell needs to identify the UE context based on the C-RNTI used in the source to perform the MRO analysis. It is assumed that the gNB-CU that supports this MRO feature will be able to store the association between the C-RNTI used in the target cell (last serving cell) and the corresponding C-RNTI used in the source cell. gNB-CU translates the C-RNTI in the target cell to the C-RNTI used in the source cell and convey it outside the RLF report when forwarding the information to the source gNB-DU.

In legacy MRO a similar solution was used for the same use case. The target gNB can store the C-RNTI coming in the HO preparation message and include it when sending the Handover report message. In order to facilitate this, the following stage2 text was added to 38.300:

If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure.

Without this stage2 text, there is nothing forcing/explaining when the C-RTNI should be included and therefore the intended functionality will not work, since implementations may choose to not include this information. Therefore, it is proposed to include a similar text in the stage2 for LTM. But since the behaviour is for F1AP, it makes more sense to capture this in 38.401 as previously proposed but not agreed in last meeting:

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the gNB-DU responsible for the failure. In case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure, the gNB-CU shall include the C-RNTI used in the source cell, if available.

Add stage2 text to specify when CRNTI should be included

### 2.3.2	Near failure scenarios

RAN2 has excluded the RACH-based access and HO interruption time as new trigger conditions for SHR. There is no doubt to resue the the timer related trigger, i.e. T310/T312/T304, similar to legacy L3 handover. Once the timer related trigger is satisfied, the network would better know whether the handover is a LTM cell switch or L3 handover, in order to adjust the parameter precisely and efficiently, e.g. whether it is the CU or DU to optimize accordingly. Therefore, we propose, an indication of LTM cell switch is introduced to SHR report.

An indication of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

In last meeting, there is a discussion on the number of RLC re-transmission as a new trigger for SHR report. However, there is no RLC re-transmission for the first UL data transmission on the target cell when the UE is attempting to access the target cell, since RLC re-transmission at the UE side is triggered by the feedback from the network, i.e RLC status report from the target cell. It is up to the UE implementation to retransmit in case the DCI received within T304. Therefore, it is not valueable to report the number of retransmissions, since this is not controlled by network.

Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Regarding the analysis of the SHR associated to an LTM mobility, there is also a role split between the source DU and target DU. Similar to the successful L3 handover analysis mechanism, the CU may forward to the DU which configures the timers causing the SHR to be generated. If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

RACH-less LTM cell switch has attracted a lot of attention. Some companies propose to study the following scenarios after RACH-less LTM cell switch execution failure:

- RACH-less LTM failure followed by successful RACH-based LTM recovery;

- RACH-less LTM failure followed by RACH-based LTM recovery failure;

- RACH-less LTM failure followed by reestablishment (if recovery is not configured).

Even though the RLF report already contains all the information required to distinguish the above three sub-cases, e.g. the presence of LTM recovery cell ID and the reestablishment cell ID, we do not see the necessity to discuss the sub-cases individually. In our understanding, what kind of failure handling performed by UE after RACH-less cell switch execution failure does not impact the detection mechanism and anlysis logic, similar to the failure handling after L3 handover failure. Therefore, we propose:

No need to divide the sub-cases for different failure handling performed by the UE after RACH-less LTM cell switch execution failure.

### 2.3.3	Successful scenarios

Also, a potential risk has been raised during the Rel-18 discussion of supporting subsequent LTM feature, which is about the ping-pong issue. The L1 measurement may be unstable and leading the handover decision directing UE to access to the same cell back and forth in a limited time. Therefore, we suggest to consider the UHI enhancement for LTM to identify the intra-gNB ping-pong issue. The CU should be responsible to collect the UHI and further provide the latest UHI to the new DU timely. Specifically, the CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU.

The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

For enabling the CU to pass the UHI to DU, a new IE over the existing F1AP signalling or a new F1AP signalling should be introduced. We believe, the existing Acess and Mobility Indication message would be the best choice to deliver UHI from CU to DU, since this message is designed for the MRO purpose to pass list of RA reports, RLF reports and SHR reports.

A new IE over the Acess and Mobility Indication message is introduced to sent UHI from CU to DU.

The UHI for DU to perform ping-pong analysis caused by LTM mobility events could be a simplified version of UHI. Because there is no need for DU to know the UHI caused by the legacy L3 handover. For example, the CU could collect the UHI, filter the UHI associcated to LTM cell switch, and passed the filtered LTM UHI to the DU.

The CU could filter the LTM related UHI and pass to DU via the new IE.

At least, we further discuss when the CU pass the UHI to DU for ping-pong purpose. The ping-pong purpose could be dived into two sub-cases: ping-pong event detection and mobility parameter optimization, ping-pong avoidance. For the former sub-case, CU forwards the UHI to source DU after the source DU makes the cell switch command. For the latter sub-case, CU forwards the UHI to the target DU as assistance information for target DU’s further handover decision once the UE accesses to the target DU. RAN3 discuss which case is the ping-pong purpose.

RAN3 discuss which case for ping-pong purpose: source DU detects the ping-pong optimization, and/or, target DU does the ping-pong avoidance for subsequent LTM execution.

If the CU propagates the UHI to the target CU duing connected UE’s mobility, an LTM indicator to distinguish LTM from the L3 handover should be added to legacy UHI. Besides, the Rel-19 LTM feature is discussing the inter-gNB LTM scenario. Such LTM indicator associated to the previously visited cells could be taken into consideration by the target CU for future mobility decision.

an LTM indicator to distinguish LTM from the L3 handover should be added to legacy UHI.

## 2.2	MRO for CHO with candidate SCGs

The agreements in last RAN3 meeting were:

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

The last RAN2 meeting agreed on the following enhancement to the RLF report:

Agreements

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

- Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

### 2.3.2	RLF report

RAN3 has agreed to report the PCell or PSCell that fulfilled the corresponding execution condition in the RLF report. Whether to report the PCell-PSCell association becomes an issue, if there is another type of execution condition that has not been met. For each candidate PCell, there could be multiple candidate PSCells, i.e a list of candidate PSCells. When UE is configured with CHO with candidate SCGs, the UE could recive at most 8 candidate configurations, each of which is for one PCell-PSCell association or one PCell only. If the RLF report indicates all the possible PCell-PSCell association, at most 8 pairs of cell IDs are needed. Therefore, we suggest to let the network acquire the PCell-PSCell association information from the UE context instead of the UE reporting method.

Besides, we also notice that each of the multiple PCell-PSCell associations could have its own elapsed time between the point in time of the first fulfilled condition and RLF, as agreed by RAN2. We do not intend to record such a time for every PCell-PSCell association. In our understanding, the first fulfilled condition means the first fulfilled condition among all the PCell-PSCell associations. Therefore, the RLF report only needs to log the time info once, i.e. earliest triggering point of PCell/PSCell till the RLF.

Only include one time information, namely between the earliest triggering point of PCell/PSCell till the RLF.

RAN3 has agreed to report the fulfilled cell both before and when the RLF is encountered. There comes a case, in which one cell met the execution for a while and then no longer met until the RLF. RAN discuss whether to differentiate if the execution condition is met before or when the RLF occurs.

RAN3 discuss whether to differentiate if the execution condition is met before or when the RLF occurs.

The FFS is whether to indicate the fulfilled events before RLF is encountered, especially when one trigger event of the execution condition is met and the other is not met. We think, it is left to network implementation to analyse which one of the trigger events is easy or hard based on the measurement result and UE context.

No need to indicate the fulfilled events before the RLF is encountered.

### 2.3.3	Near failure scenarios

For near failure case as below illustrated we assume the legacy mechanism can be re-used to the maximum extent. For near failure in PCell mobility and PSCell mobility, the UE generate SHR and SPR.

In order for the source node to correlate the SHR and SPR reports, the UE should also report the time between the execution of event and the reporting. With this information, the current solution would be able to combine the information from SHR and SPR associated to the same mobility event.

Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR.

In order for the receiving node to understand that this is from this new mobility event, the UE may also indicate the time between PCell triggered and associated PSCell triggered in SHR/SPR.

Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type.

To conclude, at the moment it seems that the impact on RAN3 is limited but that information is required form the UE. We propose to send an LS to RAN2 to request the information mentioned above.

For the SHR and SPR caused by CHO with candidate SCGs, the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

## 2.3	MRO for subsequent CPAC

### 2.3.1	Failure scenarios

New did not spot any further need for udates for the failure scenarios.

### 2.3.2	Near failure scenarios

For near failure cases, we assume the legacy mechanism and the above enhancements can be re-used.

### 2.3.3	Successful scenarios

Similar as the root causes of the failure, we’d like to address two main aspects related to S-CPAC. The first one is that the expired UE information. As standardized in Rel-18, the target SN(s) receive the SCG UHI from the MN via SN Addition Request message for CPC, which is at the preparation phase of the subsequent CPAC. Hence, for further CPC, the subsequent target SNs are not able to access the latest SCG UHI, which may hinder the timely and precise operations of target SNs, e.g., parameter optimization, resource allocation, etc. Notice that updated SCG UHI may be sent to MN from SN via SN MODIFICATION REQUEST ACKNOWLEDGE message when UE leaves this SN in S-CPAC, i.e., MN is already aware of the latest SCG UHI. Thus, MN is responsible to send such information to the candidate SNs if needed. Symmetrically, MN can use SN MODIFICATION REQUEST message to inform the candidate SNs about SCG UHI.

Add UHI to SN MODIFICATION REQUEST

Another similar information exchange that may be beneficial is that the initiating node may be informed of the outcome of the S-CPAC configuration. S-CPAC requires reservation of resources in advance. The initiating node (MN or SN) could evaluate the selection of candidate cells and possibly the time spent in each cell to evaluate if S-CPAC was beneficial or not.

In case of MN-initiated S-CPAC, we believe the MN already can get this information, since MN is already responsible for collecting the UHI. But in the case of SN initiated S-CPAC, the source SN may not be aware of what is happening to this UE, since SN is not responsible for collecting this information. Herefore the MN has to provide this to the SN.

The MN could be informaed also about intra SN changes since the MN:

- can get the updated UHI after the UE leaves the SN, or

- can subscribe to UHI updates from the SN.

Then, MN could inform the initiating SN of the UHI via the SN MODIFICATION REQUEST.

RAN3 agree that it is beneficial to inform the source SN about the outcome of mobility events for SN initiated S-CPAC by sending the UHI to the initiating SN using the SN MODIFICATION REQUEST.

# 3	Conclusion

This document proposes:

Proposal 1:	Add a new timer for LTM cell switch for LTM in the OAM requirements.

Proposal 2:	Add stage2 text to specify when CRNTI should be included

Proposal 3:	An indication of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

Proposal 4:	Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Proposal 5:	If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

Proposal 6:	No need to divide the sub-cases for different failure handling performed by the UE after RACH-less LTM cell switch execution failure.

Proposal 7:	The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

Proposal 8:	A new IE over the Acess and Mobility Indication message is introduced to sent UHI from CU to DU.

Proposal 9:	The CU could filter the LTM related UHI and pass to DU via the new IE.

Proposal 10:	RAN3 discuss which case for ping-pong purpose: source DU detects the ping-pong optimization, and/or, target DU does the ping-pong avoidance for subsequent LTM execution.

Proposal 11:	an LTM indicator to distinguish LTM from the L3 handover should be added to legacy UHI.

Proposal 12:	Only include one time information, namely between the earliest triggering point of PCell/PSCell till the RLF.

Proposal 13:	RAN3 discuss whether to differentiate if the execution condition is met before or when the RLF occurs.

Proposal 14:	No need to indicate the fulfilled events before the RLF is encountered.

Proposal 15:	Include a time between the execution of the event and the reporting in the SHR/SPR for the correlation of SHR and SPR.

Proposal 16:	Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type.

Proposal 17:	the existing SHR mechanism and the existing SPR mechanism in case of MN-initiated CPAC is capable of supporting the analysis, respectively.

Proposal 18:	Add UHI to SN MODIFICATION REQUEST

Proposal 19:	RAN3 agree that it is beneficial to inform the source SN about the outcome of mobility events for SN initiated S-CPAC by sending the UHI to the initiating SN using the SN MODIFICATION REQUEST.

# Annex	TP for LTM for 38.300

#### 15.5.2.5	O&M Requirements

All automatic changes of the HO and/or reselection parameters for mobility robustness optimisation shall be within the ranges allowed by OAM and specified below.

The following control parameters shall be provided by OAM to control MRO behaviour:

-	Maximum deviation of Handover Trigger: this parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation defined by the parameter values assigned by OAM.

-	Minimum time between Handover Trigger changes: this parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm.

Furthermore, in order to support the solutions for detection of mobility optimisation, the parameter Tstore_UE_cntxt and the Tstore_UE_cntxt_LTM shall be configurable by the OAM system.

# Annex	TP for LTM for 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the gNB-DU responsible for the failure. In case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure, the gNB-CU shall include the C-RNTI used in the source cell, if available.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU in case the SHR was triggered by source cell timer values (e.g. T310/T312) or the target gNB-DU in case the SHR was triggered by target cell timer values (e.g. T304).

# Annex	TP for LTM for 38.473

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ... |  |  |  |  |  |  |
| LTM UE History | O |  | 9.3.1.x |  | YES | Ignore |

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ... |  |  |  |  |  |  |
| LTM UE History | O |  | 9.3.1.x |  | YES | Ignore |

#### 9.3.1.x	LTM UE History

This IE includes information on previously visited cells associated with LTM mobility.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM UE History List |  | 1 |  |  |
| >LTM UE History Item |  | 1..<maxnoofCellsinLTMUEHistoryInfo> |  |  |
| >>NR CGI |  |  | 9.3.1.12 |  |
| >>Time UE Stayed in Cell |  |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinLTMUEHistoryInfo | Maximum no. of cells in the LTM UE history information. Value is 16. |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMUEHistory

[snip]

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMUEHistory,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

[snip]

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs,

maxnoofCellsinLTMUEHistoryInfo

[snip]

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

[snip]

{ ID id-LTMUEHistory							CRITICALITY ignore	TYPE LTMUEHistory										PRESENCE optional	}|

}

[snip]

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

[snip]

{ ID id-LTMUEHistory							CRITICALITY ignore	TYPE LTMUEHistory										PRESENCE optional	}|

}

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

LTMCFRAResourceConfig ::= OCTET STRING

LTMUEHistory = 	::= SEQUENCE {

lTMUEHistoryList 	LTMUEHistoryList,

iE-Extensions				ProtocolExtensionContainer { { LTMUEHistory-ExtIEs} }	OPTIONAL,

...

}

LTMUEHistory-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMUEHistoryList ::= SEQUENCE (SIZE(1..maxnoofCellsinLTMUEHistoryInfo)) OF LTMUEHistoryItem

LTMUEHistoryItem ::= SEQUENCE {

nRCGI					NRCGI,

timeUEStayedinCell		INTEGER (0..40950),

iE-Extensions			ProtocolExtensionContainer { { LTMUEHistoryItem-ExtIEs } }		OPTIONAL,

...

}

LTMUEHistoryItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

-- M

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

[snip]

maxnoofTimeWindowMea					INTEGER ::= 16

maxnoPreconfiguredSRS					INTEGER ::= 16

maxnoHopsMinusOne						INTEGER ::= 5

maxnoAggCombinations					INTEGER ::= 2

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxnoofCellsinLTMUEHistoryInfo			INTEGER ::= 16

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-LTMUEHistory										ProtocolIE-ID ::= 998 --to be assigned by MCC

[snip]

# Annex	TP for LTM for 38.413

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| LTM cell switch | O |  | ENUMERATED (“true”,...) |  | - |  |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

id-MaximumDataBurstVolume,

id-MBS-NGUFailureIndication,

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-LTMCellSwitch,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

[snip]

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional },

{ ID id-LTMCellSwitch			CRITICALITY ignore	EXTENSION ENUMERATED{true,...}	PRESENCE optional },

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

[snip]

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-LTMCellSwitch											ProtocolIE-ID ::= 998 --to be assigned by MCC

[snip]

# Annex	TP for CHO with candidate SCGs for 37.340

## 10.X	Self-optimisation for CHO with candidate SCGs

The procedure follows the steps described for Mobility Robustness Optimization in 38.300 [3] and Self-optimisation for PSCell addition/change in clause 10.18.

Handling multiple reports from a single CHO with candidate SCGs event

In case both the execution towards the target PCell and the execution towards the associated target PSCell succeed, MRO evaluation of sub-optimal conditional PCell execution and condition PSCell execution may be triggered simultaneously for the same mobility event. In this case, the corresponding SHR and SPR should be correlated.

# Annex	TP for S-CPAC for 37.340

## 13.3	SCG UE history information

The MN stores and correlates the UE History Information from MN and SN(s) as long as the UE stays in MR-DC, forwards UE History Information and optional UE History Information from the UE to its connected SNs. The resulting information is then used by SN for dual-connectivity operation. The SN is in charge of collecting SCG UE history information and providing the collected information to the MN.

If the UE stays in a PSCell for a duration exceeding the maximum value of the Time Stay parameter, the SN may store the PSCell information with consecutive entries using the same PSCell identity. The total stay time in this PSCell is the sum of stay time for all consecutive PSCell with the same identity.

The SN shall provide the collected SCG UE history information, if available, to the MN in the following procedures:

-	the SN Release, and SN initiated SN Change procedures

-	the MN initiated SN Modification procedure if requested by the MN in this procedure

-	the SN initiated SN modification procedure upon PSCell change if subscribed in the SN Addition procedure

When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via Handover Request message for CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source PSCell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving Handover Request message from the source NG-RAN node to receiving RRC Reconfiguration Complete message from the UE.

When the target SN receives the SCG UHI from the MN via SN Addition Request message for CPC, the target SN updates the time UE stayed in the cell of the latest PSCell entry (i.e. the source PSCell) when the UE successfully accesses to a candidate cell of the target SN. The updated value of the time UE stayed in the latest PSCell is equal to the value received from the MN via the SN Addition Request message plus the time from receiving SN Addition Request message from the MN to receiving SN Reconfiguration Complete from the MN.

For S-CPAC the MN may send an updated UE History Information to the initiating SN (if applicable) and may also send it to canadidate SN(s) using the MN initiated SN Modification procedure.

# Annex	TP for S-CPAC for 38.423

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

[snip]

If the PSCell History Information Retrieve IE set to "query" is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, use this information as specified in TS 37.340 [8].

If the UE History Information from the UE IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the UE History Information IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, store this information.

If the SCG UE History Information IE is included in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| ... |  |  |  |  |  |  |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

[snip]

-- **************************************************************

--

#### -- S-NODE MODIFICATION REQUEST

--

-- **************************************************************

SNodeModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeModificationRequest-IEs}},

...

}

SNodeModificationRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause												PRESENCE mandatory}|

{ ID id-PDCPChangeIndication					CRITICALITY ignore		TYPE PDCPChangeIndication							PRESENCE optional }|

{ ID id-selectedPLMN							CRITICALITY ignore		TYPE PLMN-Identity										PRESENCE optional }|

{ ID id-MobilityRestrictionList					CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-SCGConfigurationQuery					CRITICALITY ignore		TYPE SCGConfigurationQuery							PRESENCE optional }|

{ ID id-UEContextInfo-SNModRequest				CRITICALITY reject		TYPE UEContextInfo-SNModRequest					PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING										PRESENCE optional }|

{ ID id-requestedSplitSRB						CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-requestedSplitSRBrelease				CRITICALITY ignore		TYPE SplitSRBsTypes										PRESENCE optional }|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore		TYPE DesiredActNotificationLevel					PRESENCE optional }|

{ ID id-AdditionalDRBIDs						CRITICALITY reject		TYPE DRB-List											PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL			CRITICALITY reject		TYPE BitRate											PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL			CRITICALITY reject		TYPE BitRate											PRESENCE optional }|

{ ID id-LocationInformationSNReporting			CRITICALITY ignore		TYPE LocationInformationSNReporting				PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo			CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo				PRESENCE optional }|

{ ID id-PCellID									CRITICALITY reject		TYPE GlobalNG-RANCell-ID								PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern						CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3				PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3Release	CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3Release	PRESENCE optional }|

{ ID id-SNTriggered								CRITICALITY ignore		TYPE SNTriggered										PRESENCE optional }|

{ ID id-TargetNodeID							CRITICALITY ignore		TYPE GlobalNG-RANNode-ID								PRESENCE optional }|

{ ID id-PSCellHistoryInformationRetrieve		CRITICALITY ignore		TYPE PSCellHistoryInformationRetrieve			PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE			CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-CHOinformation-ModReq					CRITICALITY ignore		TYPE CHOinformation-ModReq							PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationModReq					CRITICALITY ignore		TYPE CPAInformationModReq							PRESENCE optional }|

{ ID id-CPCInformationUpdate					CRITICALITY ignore		TYPE CPCInformationUpdate							PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY ignore		TYPE UESliceMaximumBitRateList						PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList	CRITICALITY	ignore		TYPE MDTPLMNModificationList						PRESENCE optional }|

{ ID id-SelectedNID								CRITICALITY	ignore		TYPE NID												PRESENCE optional }|

{ ID id-QMCCoordinationRequest					CRITICALITY ignore		TYPE QMCCoordinationRequest							PRESENCE optional }|

{ ID id-Src-SN-to-Tgt-SNQMCInfoInquiry			CRITICALITY ignore		TYPE Src-SN-to-Tgt-SNQMCInfoInquiry				PRESENCE optional }|

{ ID id-IABAuthorizationStatus					CRITICALITY ignore	TYPE IABAuthorizationStatus							PRESENCE optional }|

{ ID id-UEHistoryInformation					CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional },

...

}

# Annex	LS to RAN2 on MRO for CHO with candidate SCGs

Title:	[DRAFT] LS on MRO for CHO with candidate SCGs

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN3 agreed to include information on whether the execution condition of PCell and PSCell is fulfilled at failure or fulfilled before failure (but not fulfilled at failure). RAN3 discussed whether the UE need to report the multiple PCell-PSCell associations, or a simple cell list containing up to 8 PCell-PSCell pairs.

RAN3 further discussed whether to have multiple times or a single time between first fulfilled execution condition and the failure, and agreed to include only one time value representing the maximum value of all associations. Furthermore, RAN3 believes that there is no need for UE to report the fulfilled events of each execution condition.

For SHR report, RAN3 discuss the correlation of SHR and SPR, which are generated due to the same CHO with candidate SCGs mobility event, and agreed that the time info between the execution of the mobility event and the reporting added to the SHR/SPR is beneficial to correlate the SHR and SPR.

RAN3 thinks the the time gap between the first met execution condition and second met execution condition should also added to SHR/SPR. The time gap could also be used as an implicit indication for the new HO type.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

RAN3#127	2025-02-17  – 2024-02-21	Athens, GR