3GPP TSG-RAN WG3 Meeting #126	R3-247160

Orlando, US, 18 - 22 Nov, 2024

Title:	SON and MDT for Network Slicing

Agenda Item:	10.3.2

Source:	Huawei

Document for:	other

# Introduction

This document discuss SON and MDT for Network Slicing.

# 2	Discussion

## 2.1	Enhancement for the MDT area scope

We did not spot any need for enhancement for this part.

## 2.2	Enhancement for slice-based cell reselection for logged MDT

The UE in RRC_IDLE mode or in RRC_INACTIVE mode may perform slice-based cell reselection for the purpose of camping on a more suitable cell. During the procedure, the UE takes both the mapping relationship between the S-NSSAI and NSAG received by NAS and the FreqPriorityListSlicing received in SIB16 or RRCRelease into account for frequency ranking and the cell selection evaluation.

Two different scenarios options where identified in last meeting

- The first one is about the optimizing mapping between the slice groups and frequency priority using logged MDT

- The second one is informing the network that UE performs slice based re-selection instead of general reselection

One important clarification to the above is whether the second one is to know the if UE performs slice based re-selection for the whole measurement period, or if incormation is provided per cell reselection event.

We believe that it is important to only log information that is only available in the UE and that information available to the network does not need to be reported.

In order for slice based reslection to be enabled for a UE, the following is required:

- UE should support this capability

- RAN node need to signal the NSAG+freq priority (SIB16 or dedicated) to the UE

- AMF must send confguration to the UE through NAS

Based on the above, we outline a set of possible solutions:

- Include a flag whether the UE is able to do slice-based cell reselection during the measurement period 
This means we can differentiate behaviour for different UE, i.e. differentiate UEs not supporting this functionality at all

- Include a flag for each cell reselection, to indicate the cell reslection was made using slice-based cell reselection
Similar to above, and in addition to thism we can analyse the coverage of the configured NSAG. On the other hand, we cannot know which NSAG was configured to the UE

- Include the NSAG indicating the NSAG used for each slice-based cell reslection (empty if if no configured NSAG was found)
This means we can analyse which NSAG the UE is able to follow, but we cannot know which NSAG was not possible to follow

- Include the NSAG configuration currently used by the UE
This information together with information on the configuration of the network will give information about both which NSAG that was possible to use and also which NSAG was not possible to use (e.g. in case a higher priority NSAG was not possible to be used)

The contents of this LS should probably focus on the potential benefit we can agree in RAN3 and not touch the detailed solution since this is in RAN2 domain.

Send an LS to RAN2 to propose the collection of NSAG information ino the logged MDT report.

## 2.3	MRO for failure cases

SON functionality is meant to exchange information between NG-RAN nodes to enable optimization in RAN. As we mentioned in [2], there could be a little impact on MRO enhancement. The mobility decisions may be made based on the slice support or the slice availability at the target NG-RAN node. For instance, a UE may be handed over to a target cell supporting the slice associated with the ongoing PDU session but without good radio conditions, it could be beneficial if this aspect could be analysed in the existing MRO mechanism. Hence, the NW may disregard mobility failures triggered by slice reasons.

Another possibility would be to let MRO perform different optimization depending on the impact on slice availability on mobility. This is similar to the previous mentioned solution (disregard some failures) but where it is left to implementation whether to disregard or split the reports into different sets where the different sets are used to optimize different mobility parameters used in different conditions.

Since we already have the possibility to retrieve stored context, any vendor wanting to support this feature would need to store information about mobility decisions until the report arrives from the UE. We do not think that any additions are needed to stage3. This may not even have impact on stage2 since the solution would rely on existing framework-

The Slice aware MRO can be supported without any standards impact

## 2.4	MRO for successful cases

One use case discussed in RAN3#124 was the successful case. The proposed scenario is to try to avoid that a UE passes through a cell not supporting the slice of interest for a UE corresponding to the red path depicted in the figure below, but instead tries to maintain the slice support (blue path) and immediately handover to Cell 3.

Cell1

slice a,b

Cell3

slice a,b

Cell2

slice a

Figure 1. Successful handover use case

For this scenario, our assumption is that a node optimizing the parameters with MRO will already know the boundaries for the mobility parameters, e.g. . what CIO is possible to assign without creating a too large risk for failed mobility. The nodes would also already know what slices are supported in their neighbours and would always strive to perform the mobility from cell1 to cell 3 when possible. Therefore, node handling cell1 would already try its best to handover the UE to cells supporting the desired slices. Adding the possibility for other nodes to feedback information about this (the red line) would not help the node handling cell1.

No need to enhance the successful case for MRO

# 3	Conclusion

This document proposes:

Proposal 1:	Send an LS to RAN2 to propose the collection of NSAG information ino the logged MDT report.

Proposal 2:	The Slice aware MRO can be supported without any standards impact

Proposal 3:	No need to enhance the successful case for MRO

# Annex – LS to RAN2

Title:	[DRAFT] LS on NSAG in Logged MDT

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:	SA5

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed enhancements for SON and MDT in the scope of the new slicing functionality added in Rel-18.

RAN3 agreed that it is beneficial to capture the impact of Slice based cell reselection in the logged MDT report, e.g. to analyse whether the UE is able to perform cell reselection to cells within the configured NSAG. The detailed solution is left to RAN2, but RAN3 considers that it is most important to log information only available in the UE, e.g.the NSAG configured to the UE.

RAN3 would kindly like to ask RAN2 to consider adding support of of slice based cell reselection to Logged MDT, if feasible.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US