3GPP TSG-RAN WG3 Meeting #126	R3-247201

Orlando, US, 18th – 22nd November 2024

Agenda Item:		11.2

Source:		NEC, ZTE, Qualcomm, Samsung, CMCC

Title:	(TP for BLCR for TS38.300) Support of enhancements on AI/ML for NG-RAN

Document for:		Discussion and decision

# Introduction

This contribution provides the TP to AIML BLCR TS38.300 on support of enhancements on AI/ML for NG-RAN, including the following changes in order to inherit all agreements made during SI phase:

- To add the two new Rel-19 use cases, i.e. Slicing and CCO.

As captured in the WID RP-242358, AI/ML-based Slicing and AI/ML based CCO are agreed to be specified in Rel-19 AI/ML for NG-RAN.

- To add the new information that can be configured to be reported by an NG-RAN node for slicing and CCO.

For the two new Rel-19 use cases, compared with what was specified during Rel-18, the following information are listed to be exchanged over Xn interface.

AI/ML based Network Slicing

- Predicted radio resource status per slice

- Predicted slice available capacity

AI/ML based CCO

- Future CCO State together with an associated coverage modification cause

- Timing Information

# TP to BLCR TS38.300

<Start of change>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, Network Slicing, Coverage and Capacity Optimization.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64].

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including radio resource status per slice;

-	UE performance feedback;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity;

The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

The Future CCO State together with an associated coverage modification cause and timing information can also be configured to be reported by an NG-RAN node via NG-RAN NODE CONFIGURATION UPDATE procedure.

<End of change>