3GPP TSG-RAN WG3 Meeting #126	R3-247202

Orlando, US, 18th – 22nd November 2024

Agenda Item:				11.2

Source:						NEC

Title:	(TP for BLCR for TS38.401) Support of enhancements on AI/ML for NG-RAN

Document for:				Discussion and decision

- Introduction

This contribution provides the TP to AIML BLCR TS38.401 on support of enhancements on AI/ML for NG-RAN, including the following changes in order to inherit all agreements made during SI phase and the new agreement made in last RAN3 meeting:

- To update the deployment scenarios to support CP-UP split architecture.

- To add Data Collection Reporting Initiation and Data Collection Reporting procedures to send the measured Energy Cost and the measured UE performance from gNB-DU to gNB-CU.

# TP to BLCR TS38.401

<Start of 1st change>

## 7.11	Support of AI/ML in NG-RAN

The support of AI/ML in NG-RAN is specified in TS 38.300 [2].

In case of split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP).

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

< Start of 2st change >

## 8.x	Data Collection Reporting Initiation

This procedure is used by an gNB-CU to request from gNB-DU the reporting of information to support, e.g., AI/ML in NG-RAN.

The signalling flow for Data Collection Reporting Initiation in Figure 8.x-1.

Figure 8.x-1: Data Collection Reporting Initiation procedure

gNB-CU initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-DU to start information reporting or to stop information reporting of the measured Energy Cost and the measured UE performance.

## 8.y	Data Collection Reporting

This procedure is initiated by an gNB-DU to report information accepted by the gNB-CU following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The signalling flow for Data Collection Reporting in Figure 8.y-1.

Figure 8.4.14.2-1: Data Collection Reporting procudure

gNB-DU shall report the measured Energy Cost and the measured UE performance in DATA COLLECTION UPDATE message.

<End of change>