3GPP TSG RAN WG3 #126					                 R3-247224

Orlando, USA, November 18th – 22nd, 2024

Source:	NEC

Title:	Discussion on CLI Handling Support

Agenda item:	19.2

Document for:   Discussion and decision

1 Introduction

A WI on the evolution of NR duplex operation was approved in the RAN #102 meeting. The objectives of the WI related to RAN3 work are listed below [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request

During the RAN3 #125bis meeting, the following agreement was made regarding NR duplex operations. [2]

RAN3 agrees to Exchange the following information over Xn interface and F1 interface: - Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP) - Measurement resource configuration - Strongest DL beam information (FFS on the details) - CLI-mitigation request (FFS on the details)

This contribution provides NEC views on information exchange among gNBs for CLI mitigation.

2 Discussion

As per the agreement given by RAN1, the information below may be exchanged among gNBs for gNB-gNB CLI handling through Xn and between gNB-CU and gNB-DU through F1:

- Semi-static cell-specific SBFD time and frequency location configuration

- Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Strongest DL beam information

- CLI-mitigation request

2.1 gNB-to-gNB over Xn

The procedure and workflow for gNB -gNB CLI mitigation and handling can be described as follows:

- Step 1: gNB1 sends semi-static cell-specific SBFD time and frequency location configuration information and measurement resource configuration information, i.e., SSB and/or periodic NZP CSI-RS, to gNB2.

- Step 2: Upon reception of NZP CSI-RS /SSB, gNB2 completes gNB1-gNB2 CLI measurement and feedback on the measurement result, such as strongest DL beam information, or sends CLI-mitigation request through backhaul (i.e., Xn) to gNB1. This may also happen further down via F1 from gNB-CU to gNB-DU under gNB-DU1 in split architecture. Alternatively, gNB2 applies a CLI handling scheme according to the CLI measurement result, e.g., power boosting, changing the transmission resource/direction, or transmitted traffic type/signals.

- Step 3(optional): gNB1 applies an interference mitigation scheme, such as power backoff, scheduling adjustment after receiving the strongest DL beam information, or CLI-mitigation request.

Figure 1: Signalling of CLI mitigation over Xn.

Proposal 1:

The signaling flow for gNB-gNB CLI handling over Xn/F1 in Figure 1 can be specified.

2.2 Periodicity of measurement and reporting

When dealing with the measurement and reporting of the strongest interfering beam, the periodicity of the measurement needs to be specified. This is important as the strongest beam can be determined in several ways. If the gNB-gNB CLI is handled by averaging the strongest interfering beam, then a periodic measurement scheme can be applied to this procedure. On the other hand, if the measurement of the strongest interfering beam needs to be triggered by a specific condition, event-based measurements will need to be applied. Whatever measurement periodicity (if any) may be required is up to RAN3 discussion, but discussions are required in Rel-19.

Likewise, with measurement for various gNB-gNB CLI aspects, reporting said information can also be sent via a wide range of periodicities. Of note is that measurements and reporting can have separate periodicities independent of each other. These aspects should be specified in RAN3 to allow for a clear specification.

Proposal 2:

Specify periodicities for measurement and reporting of gNB-gNB CLI quantities

2.3 Information Exchange for CLI Handling

As mentioned in Proposal 1 above, the information exchanges for CLI handling are further discussed, and enhancements are proposed.

2.2.1 Semi-static cell-specific SBFD time and frequency location configuration

This IE contains the SBFD time and frequency location configuration information used by gNB for CLI mitigation, and it is necessary for a gNB to do CLI measurements. The SBFD time and frequency location configuration IE includes the configuration of SBFD symbols position and frequency locations of the SBFD UL subband and DL subband. For the configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are included

-	A starting slot index

-	A starting symbol index within the starting slot

-	An ending slot index

-	An ending symbol index within the ending slot

For the configuration of frequency locations of the SBFD UL subband and DL subband, starting RB and bandwidth for each subband can be configured. One or two SBFD DL subbands can be configured for a cell. This information can be the same as what is configured for UE by RRC.

If the neighboring gNB configuration has the same SBFD time and frequency location as UE, the gNB-to-gNB CLI can be avoided as the uplink and downlink transmission scheduling of the two gNBs are the same.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SBFD time location configuration | M |  | 9.3.3.x1 | This IE indicates the SBFD subband time position, including a starting slot index, starting symbol index within the starting slot, an ending slot index and an ending symbol index within the ending slot. |
| SBFD frequency location configuration | M |  | 9.3.3.x2 | This IE indicates the SBFD subband frequency position, including starting RB and bandwidth for each subband. |

2.2.2 Measurement resource configuration

For gNB-gNB CLI mitigation and handling support, CLI measurement resource configuration IE can be defined, and it is used by the gNB to complete inter-gNB CLI measurement. The candidate values can be a set of one or more periodic NZP CSI-RS resources (relevant IE in TS38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) or SSB resources (relevant IE in TS38.331 is SSB-Configuration). Obviously, such IE should be per-cell basis.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI Measurement resource configuration | O |  | 9.3.3.x | The RS resource for gNB-gNB CLI measurement including a set of one or more periodic NZP CSI-RS resources set or SSB index, such as NZP-CSI-RS-Resource or NZP-CSI-RS-ResourceSet and SSB-Configuration as defined in TS 38.331 |

2.2.3 Strongest DL beam information

The strongest DL beam information IE can be defined, and the gNB uses it to handle inter-gNB CLI. The candidate value can be a CRI or an SSB index. NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. The SSB index is an absolute index value among the SSB(s) for which the configuration is provided to a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Strongest DL beam information | O |  | 9.3.3.x | Indication the strongest DL beam information a CSI-RS Resource index or an SSB index) |

2.2.4 CLI-mitigation request

CLI-mitigation request IE can be defined. The victim gNB uses it to send a CLI-mitigation request to the aggressor gNB to do CLI-mitigation. The aggressor gNB may choose to respond to said request accordingly. And new procedure to provide CLI-mitigation request can be introduced.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CLI-mitigation request | O |  | 9.3.3.x | request CLI-mitigation to the aggressor gNB to do CLI-mitigation |

Proposal 3:

RAN 3 should consider including the additional listed IEs for gNB-gNB CLI handling or mitigation.

3 Conclusion

Based on the discussion in the previous sections, this contribution is concluded with the following proposals:

Proposal 1:

The signaling flow for gNB-gNB CLI handling over Xn/F1 in Figure 1 can be specified.

Proposal 2:

Specify periodicities for measurement and reporting of gNB-gNB CLI quantities

Proposal 3:

RAN 3 should consider including the listed additional IEs for gNB-gNB CLI handling or mitigation.

4 References

[1] RP-241614, ‘New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)’ CMCC.

[2] RAN3_125bis_agenda_20241018_1430_EOM