3GPP TSG-RAN3 Meeting #126	R3-247231

Orlando, US, Nov 18th – Nov 22nd 2024

Agenda Item:	18.2

Source: 	Qualcomm Incorporated

Title: 	Support LP-WUS subgrouping in RRC_IDLE/RRC_INACTIVE

Document for:	Discussion/Decision

- Background

WID of Low-power wake-up signal and receiver for NR (LP-WUS/WUR) (RP-240801) has the below objective for RAN3:

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In this contribution, we discuss the details on supporting LP-WUS subgrouping to indicate paging via LP-WUS for a UE in RRC_IDLE/RRC_INACTIVE states.

# 2	Discussion

## 2.1 	F1 changes for LP-WUS subgrouping capability indication

Below is one agreements from RAN3#125bis on LP-WUS subgrouping [2]:

[Agreement]

The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

We can assume that for LP-WUS CN-assigned subgrouping, similar to PEI, UE indicates its CN-assigned subgrouping capability to CN via NAS signaling and CN assigns the LP-WUS subgroup ID. CN indicates the per UE CN-assigned subgroup ID in the NGAP paging messages only for the UEs which support CN-assigned subgrouping. Thus, it is not necessary for RAN to further check the CN-assigned subgrouping capability of the UE. Instead, the RAN is expected to support LP-WUS using CN-assigned subgroup ID for these UEs in the cells where LP-WUS subgrouping is supported.

Proposal 1: RAN3 assumes that UE indicates CN-assigned subgrouping capability to CN directly via NAS signaling. CN indicates the per UE CN-assigned subgroup ID in the NGAP paging messages only for the UEs which support CN-assigned subgrouping.

Proposal 2: No need to include per UE CN-assigned subgrouping capability information in the F1 paging message.

RAN3 made an agreement in RAN3#125bis to introduce LP-WUS UE_ID based subgrouping capability indication into F1 paging message with FFS on per UE level or per cell level.

[Agreement]

Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

For PEI, per UE based UE_ID based subgrouping capability indication was removed from the description in Paging section in Rel-18, V18.3.0 of TS 38.473 and it was never present in the UE Paging Capability IE. Therefore, it should not be supported for LP-WUS either.

Observation 1: For PEI subgrouping, per UE PEI UEID Subgrouping capability Indication, is not included (anymore) in the UE Paging Capability IE in F1 PAGING message sent to the DU.

Proposal 3: Reverse RAN3#125bis agreement “Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping.” as it was not present for PEI.

Assuming, for LP-WUS as well, UE indicates via UECapabilityInformation the Bands for which it supports radio capability for LP-WUS subgrouping, it is possible that there are only some cells under a DU where UE can support receiving LP-SS/LP-WUS. Also, there may be only certain cells where CU wants to support LP-WUS subgrouping. Thus, it is necessary for the CU to indicate to the DU in the F1 PAGING message information on a per cell level whether to support LP-WUS subgrouping or not for a UE. This is a feature level support indication and not UE_ID based subgrouping capability indication.

Proposal 4: In analogy to PEI subgrouping support indication, support including LP-WUS Subgrouping Support Indication IE in the Paging Cell Item IEs in F1AP PAGING message to indicate if the cell should support LP-WUS subgrouping indication.

## 2.2 	LP-WUS CN assigned subgroup ID in Xn paging

In RAN3#125bis, RAN3 discussed whether to include CN assigned LP-WUS subgroup ID into the Xn paging message and left it as FFS pending on RAN2 decision as per below statement.

The anchor gNB provides the new IE (FFS) for CN assigned subgroup ID into the Xn paging message for RAN paging. This agreement pending on RAN2 decision that UE can use the LP-WUS outside the last serving cell.

For PEI, PEI support in last-used-cell-only (i.e., the cell in which the UE most recently received RRCRelease without indicating that the last used cell for PEI shall not be updated) was introduced to support power savings for stationary UEs (like IOT UEs). Since this is a system information parameter, it need not be applied, e.g., in case the cells supports other UEs than IOT UEs.  Similar to PEI, LP-WUS in last-used-cell-only may be supported and the support may be indicated via system information. However, there is no obvious reason to mandate LP-WUS to be applied to last-used-cell-only for all cells. RAN3 should agree on this understanding. So, unless RAN2 agrees that last-used-cell-only is mandated for all cells,  be a configurable, Xn changes to include CN assigned LP-WUS subgroup ID in Xn Paging message are needed.

Observation 2: For PEI subgrouping, last-used-cell-only can be optionally applied to IOT cells, but it is not mandated in all cells.

Proposal 5: It is RAN3’s understanding that last-used-cell-only, if supported for LP WUS, will not be mandated to all cells.

Proposal 6: Xn to include CN-assigned LP-WUS subgroup ID in Xn Paging message unless RAN2 mandates last-used-cell-only to be applied to LP-WUS in all cells.

## 2.3 	LP-WUS configuration

In RAN3#125bis, RAN3 did not conclude on whether the entry/exit condition is generated at the gNB-DU or the gNB-CU. Below are some notes from RAN2#126 [1] and RAN2#127 [3] on LP-SS/LP-WUS configuration parameters

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Do not support dedicated LP-WUS configuration

Noted

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Observation 3: RAN2 assumes to use serving cell quality measured by MR/LR, i.e. (LP)RSRP or (LP)RSRQ, for entry/exit conditions determination of LR monitoring.

In RAN1/2 study report [4], it is clarified that there are scenarios where LP-SS link performance is not better than legacy physical channels (e.g., PDCCH or PUSCH), and there could be partial LP-SS coverage in the cell. For the partial coverage case, UE may need to keep track of whether it moves in and out of LP-SS coverage in the cell, so that UE is able to fall back to monitoring paging using the main receiver when the coverage of LP-WUS is not sufficient.

RAN2 agreed that the UE that supports LP-WUS, monitors both LP-SS and SSBs of the serving cells to determine whether the LP entry/exit conditions are met. The UE should use MR-based thresholds like RSRP/RSRQ and LR-based thresholds like LP-RSRQ/LP-RSRP to determine whether it is in LP-SS/LP-WUS coverage or not. The UE keeps the MR in ultra-deep sleep power state if the UE is in the LP-SS coverage (i.e. entry condition is met) in the serving cell, and it receives page indications via LP-WUS. When the UE moves out of LP-SS coverage, i.e. exit condition is met, the UE will wake up the MR to monitor the serving cell PEI and/or paging messages using MR. The LP-SS measurements are only used by the UE for switching between MR and LR in the serving cell and not for cell selection/reselection. Therefore, MR/LR entry/exit criteria are not affected by inter-DU relations, and consequently, these criteria can be determined by the gNB-DU rather than the gNB-CU.

Proposal 7: For RRC idle/inactive UEs, the entry/exit condition is generated by the gNB-DU since RAN2/RAN1 agreed to use these conditions only for MR/LR switch within the serving cell but not for inter-cell reselection.

## 2.4 	LP-WUS coexistence with eDRX

Observation 4: According to the RAN1/RAN2 TR [4] introduction section, LP-WUS is introduced to provide better power savings than long eDRX. Thus, UE will wake up MR only when paged and monitor per the long eDRX.

As per performance analysis in this TR, there seems to be no special handling for LP-WUS when eDRX is enabled. Further, enabling both, LP-WUS and eDRX, may provide better power savings. Thus, we believe that RAN3 should agree to support simultaneous operation of LP-WUS and (long)eDRX,and inform RAN2/RAN2 about this decision.

Proposal 8: From RAN3 perspective, simultaneous operation of eDRX and LP-WUS should be supported to provide better power savings for IOT devices.

## 2.5 	LP-WUS Extended UE Identity Index Value IE in F1 Paging

In RAN3#125bis, RAN3 discussed whether to introduce a new IE to include extra bits for UE_ID based LP-WUS

subgrouping, i.e. the Extended UE Identity Index Value IE into F1 paging. However, no agreement has been made.

In RAN2#127bis, RAN2 made the below agreement

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

For PEI, UE_ID based subgrouping ID formula in section 7.3.2 of [5] has a parameter X, where X is 32768, if eDRX is applied; otherwise, X is 8192 which is used to extend the UE_ID to support the maximum number of UE_ID based PEI subgroups of 8. Thus, Extended UE Identity Index Value IE was introduced into F1 paging for PEI to indicate whether to use value 1 (32768) or value 2 (8192) for UE_ID computation to determine the UE_ID based subgroup ID. Based on RAN2 agreements from RAN2#127bis, a similar formula as PEI subgrouping can be applied for LP-WUS. However, it is not yet agreed whether same values of X are reused for LP-WUS or not.

If same values of X are reused for LP-WUS, then there is no need to introduce new Extended UE Identity Index Value IE for LP-WUS. Otherwise (i.e. different values of X are defined for LP-WUS),   an IE similar to Extended UE Identity Index Value IE has to be introduced in F1 Paging message for LP-WUS.

Observation 5: For PEI, Extended UE Identity Index Value IE was introduced into F1 paging to indicate whether to use value 1 (32768) or value 2 (8192) for the UE_ID computation to determine the UE_ID based subgroup ID .

Proposal 9: RAN3 wait for RAN2/RAN1 progress to know which values of X are used for UE_ID computation in LP-WUS UE_ID based subgroup ID calculation.

# 3	Summary

Below are the observations and proposals discussed in this document.

On F1 changes for LP-WUS subgrouping capability indication

Proposal 1: RAN3 assumes that UE indicates CN-assigned subgrouping capability to CN directly via NAS signaling. CN indicates the per UE CN-assigned subgroup ID in the NGAP paging messages only for the UEs which support CN-assigned subgrouping.

Proposal 2: No need to include per UE CN-assigned subgrouping capability information in the F1 paging message.

Observation 1: For PEI subgrouping, per UE PEI UEID Subgrouping capability Indication, is not included (anymore) in the UE Paging Capability IE in F1 PAGING message sent to the DU.

Proposal 3: Reverse RAN3#125bis agreement “Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping.” as it was not present for PEI.

Proposal 4: In analogy to PEI subgrouping support indication, support including LP-WUS Subgrouping Support Indication IE in the Paging Cell Item IEs in F1AP PAGING message to indicate if the cell should support LP-WUS subgrouping indication.

On LP-WUS CN assigned subgroup ID in Xn paging

Observation 2: For PEI subgrouping, last-used-cell-only can be optionally applied to IOT cells, but it is not mandated in all cells.

Proposal 5: It is RAN3’s understanding that last-used-cell-only, if supported for LP WUS, will not be mandated to all cells.

Proposal 6: Xn to include CN-assigned LP-WUS subgroup ID in Xn Paging message unless RAN2 mandates last-used-cell-only to be applied to LP-WUS in all cells.

On LP-WUS configuration

Observation 3: RAN2 assumes to use serving cell quality measured by MR/LR, i.e. (LP)RSRP or (LP)RSRQ, for entry/exit conditions determination of LR monitoring.

Proposal 7: For RRC idle/inactive UEs, the entry/exit condition is generated by the gNB-DU since RAN2/RAN1 agreed to use these conditions only for MR/LR switch within the serving cell but not for inter-cell reselection.

On LP-WUS coexistence with eDRX

Observation 4: According to the RAN1/RAN2 TR [4] introduction section, LP-WUS is introduced to provide better power savings than long eDRX. Thus, UE will wake up MR only when paged and monitor per the long eDRX.

Proposal 8: From RAN3 perspective, simultaneous operation of eDRX and LP-WUS should be supported to provide better power savings for IOT devices.

On LP-WUS Extended UE Identity Index Value IE in F1 Paging

Observation 5: For PEI, Extended UE Identity Index Value IE was introduced into F1 paging to indicate whether to use value 1 (32768) or value 2 (8192) for the UE_ID computation to determine the UE_ID based subgroup ID .

Proposal 9: RAN3 wait for RAN2/RAN1 progress to know which values of X are used for UE_ID computation in LP-WUS UE_ID based subgroup ID calculation.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] (draft) RAN3 #125-bis Meeting Report, Hefei, China

[3] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

[4] TR 38.869 - Study on low-power wake-up signal and receiver for NR

[5] TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 18)