3GPP TSG-RAN WG3 Meeting #126	             R3-247246

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:	8.1

Source:	CMCC

Title:	Discussion on RAN3 support of multi-modality awareness

Document for:	Discussions & Approval

# Introduction

RAN2 replies the SA2 LS on multi-modality awareness as following [1]:

RAN2 thanks SA2 for the feedback on multi-modality awareness. For the usage of multi-modality information, RAN2 has discussed and reached the following agreements during the previous RAN2 meetings. First, RAN2 agreed that the multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping. RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation). Furthermore, RAN2 also agreed that traffic synchronization may be achieved by packet level synchronization information, if it can be provided from CN to RAN, which is up to SA2/4 to discuss. For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.. # To RAN3, SA4: # ACTION: According to the request from SA2, RAN2 would kindly asks RAN3 and SA4 to review the RAN2 feedback above.

# Discussions

RAN awareness of Multi-modal Service ID (MMSID) is beneficial, enabling RAN to coordinate scheduling and synchronize multiple QoS flows associated with the same MMSID.

To ensure the synchronized/coordinated transmission of the DRBs with multi-modal association, admission control enhancement can be considered, for example the QoS flows associated with the same MMSID should be admitted simultaneously or rejected simultaneously.

In split-bearer dual connectivity scenarios, where different QoS flows belonging to the same MMSID could be transmitted across both bearers, careful coordination is essential to ensure proper flow synchronization. The behaviour or transmission conditions of a QoS flow in one bearer can impact how related QoS flows should be managed in the other bearer. This coordination is particularly crucial for ensuring that the synchronization between flows is maintained, as any mismatch could degrade the user experience for time-sensitive services like XR or real-time communications. Additionally, this affects congestion management, as handling congestion in one bearer could influence how traffic is prioritized or adjusted in the other bearer to avoid delays or disruptions in the overall service. In order to achieve optimal performance for multi-modal services, many problems in split-bearer dual connectivity scenarios need to be revisited in RAN3.

Multi-model services require enhancement to the admission control in DRBs, for example QoS flows associated with the same MMSID need to be admitted or rejected simultaneously.

Multi-modal services pose new challenges in split-bearer dual connectivity scenarios, particularly in maintaining synchronization and coordination of multiple QoS flows under the same MMSID. RAN3 needs to revisit the problems in split-bearer dual connectivity scenarios.

# Conclusions

In this submission, we discussed the multi-modality awareness at RAN. The following proposals are made:

- Multi-model services require enhancement to the admission control in DRBs, for example QoS flows associated with the same MMSID need to be admitted or rejected simultaneously.

- Multi-modal services pose new challenges in split-bearer dual connectivity scenarios, particularly in maintaining synchronization and coordination of multiple QoS flows under the same MMSID. RAN3 needs to revisit the problems in split-bearer dual connectivity scenarios.

# References

R3-247009, Reply to SA2 LS on multi-modality awareness