3GPP TSG-RAN WG3 Meeting #126		R3-247257

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo, NTT DOCOMO INC

Title:	Discussion on LP-WUS for NR

Agenda Item:	18.2

Document for:	Discussion

- Introduction

In RAN#105 meeting, the updated WID of Rel-19 low-power Wake-up Signal and Receiver (LP-WUS/WUR) for NR in [1] was approved with the below objective involved RAN3:

For IDLE/INACTIVE modes • Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

RAN3 has the following agreements in RAN3#125 meeting [2] and in RAN3#125bis meeting [3]:

[Agreement] New IEs should be introduced to support LP-WUS on NG, Xn and F1. FFS For RRC idle/inactive UEs, RAN3 to discuss the LP-SS and LP-WUS configuration is generated at the gNB-DU or the gNB-CU is FFS. For RRC idle/inactive UEs, RAN3 to discuss whether Entry/exit condition is generated at the gNB-DU or the gNB-CU. The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups. The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups. The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups. Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping. The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

In this contribution, we discuss the corresponding RAN3 impacts on LP-WUS subgrouping, including CN assigned subgrouping and UE_ID based subgrouping.

- Discussion

  - LS from RAN2

RAN2 sent an LS [4] to RAN3 in RAN2#127bis meeting to inform the RAN2 progress on subgrouping, which is as follows:

- The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. - RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. - RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. - For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. - For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

It could be observed that for CN assigned subgrouping, RAN2 has agreed it is up to the CN to assign the LP-WUS subgroup ID to the UE. Accordingly, same as the signalling design in PEI the LP-WUS assistance information IE should be the LP-WUS subgroup ID to indicate that this UE supports the CN assigned subgrouping and its corresponding subgroup ID to the gNB, and the gNB could send LP-WUS with the indicated subgroup ID if appliable. And RAN3 has the agreement that the LP-WUS assistance information IE includes the CN assigned subgroup ID for LP-WUS which is aligned with the PEI design and RAN2 agreement. Thus, the LP-WUS assistance information IE is the CN assigned LP-WUS subgroup ID.

Besides, since RAN2 assumes the maximum number of subgroups for LP-WUS is no less than 8, while the exact maximum number of subgroups is being discussed in RAN1. Thus, RAN3 should wait for the decision on the exact maximum number of subgroups, after that, it could be used directly for the length of LP-WUS assistance information IE.

Observation 1: The LP-WUS assistance information IE only includes the CN assigned LP-WUS subgroup ID.

  - CN assigned subgrouping impact on XnAP interface

In last RAN3#125bis meeting, we agreed the CN assigned subgrouping information, i.e. the LP-WUS assistance information IE, will be included in the NG paging message, NGAP core networking assistance information IE and the F1 paging message. However, it is FFS whether the LP-WUS assistance information IE should be included in RAN paging message since it is impacted by the RAN2 design which is similar to the PEI. Specifically, it depends on whether the LP-WUS is only used in the last serving cell, as below. During the discussion in RAN3, some companies think the anchor gNB provides the new IE (FFS) for CN assigned subgroup ID into the Xn paging message for RAN paging. This agreement pending on RAN2 decision that UE can use the LP-WUS outside the last serving cell.

If the LP-WUS could be used in other camped cells in addition to the last serving cell which is same as the PEI, the LP-WUS assistance information IE have to be included in the Xn paging message, otherwise, if the LP-WUS is only used in last serving cell, there is no need to include this information in Xn paging message.

In our understanding, it could bring more power saving gain if not restrict the use of LP-WUS in the last serving cell. However, the issue should be determined by RAN2 as the same issue for CIOT WUS or PEI are both discussed and determined by RAN2. RAN3 could discuss the Xn enhancement after RAN2 has further progress on the issue for last serving cell.

On the other hand, even not related to the issue on whether LP-WUS could be used outside the last serving cell, the discussion baseline is that LP-WUS could be used in any camped cell supporting LP-WUS. In this way, for RAN paging, the LP-WUS assistance information IE should also be included in the Xn paging message.

Anyway, this discussion could wait for more progress in RAN2 on the issue for last serving cell.

Proposal 1: Whether anchor gNB provides the LP-WUS assistance information IE, which includes the CN assigned subgroup ID into the Xn paging message for RAN paging should wait for further RAN2 progress.

  - UE_ID based subgrouping impact on F1 interface

In Rel-17, RAN2 agreed to introduce UE-RadioPagingInfo IE in the RRC UECapabilityInformation message in NR on whether the UE supports PEI, and it is the same UE capability for supporting PEI and UE_ID based PEI subgrouping. Since the UE Radio Capability for Paging IE is sent within the NG Paging for RRC_IDLE UEs, and within NGAP core Networking assistance information IE and Xn paging for RRC_INACTIVE UEs, the NG-RAN node could acquire the information on whether the UE supports the UE_ID based PEI subgrouping information.

Observation 2: UE capability on UE_ID based PEI subgrouping is included in the UE Radio Capability for Paging IE in PEI.

For LP-WUS, since RAN2 has not discussed the UE capability on LP-WUS subgrouping information, the detailed signalling design on UE_ID based subgrouping for LP-WUS on RAN3 impacts should wait for more progress in RAN2. But anyway, similar as PEI design, the NG-RAN node could acquire the capability information for UE_ID based subgrouping for LP-WUS for CN paging and RAN paging.

Observation 3: NG-RAN node could acquire the UE capability information on UE_ID based LP-WUS subgrouping, no RAN3 impacts for the detailed signalling design.

Based on the above background, RAN3 has the following agreement on F1 interface enhancement to ensure the DU know whether to send the LP-WUS when paging a UE.

Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

According to the above agreement, LP-WUS subgrouping capability related information is introduced into F1 paging message for UE_ID based subgrouping. While in general, the detailed UE capability design should be decided in RAN2. During RAN2 discussion, it is common understanding that separate capability for CN assigned subgrouping and UE_ID based subgrouping would be introduced for LP-WUS in Rel-19. In this way, RAN3 should wait for RAN2 decision on the detailed UE capability design on CN assigned subgrouping and UE_ID based subgrouping for LP-WUS. After that, the left thing is to capture the corresponding capability in the specification, which requires little effort.

Similarly, according to the design in legacy PEI, RAN2 would also discuss the network capability, i.e. a gNB/cell would support or not support the UE_ID based subgrouping and/or CN assigned subgrouping, which will be indicated to UE via the LP-WUS configuration, and the detail configuration or indication is being discussed in RAN2, there should be no impact in RAN3.

Proposal 2: For LP-WUS subgrouping capability, wait for RAN2 decision on the UE capability for CN assigned subgrouping and/or UE_ID based subgrouping for LP-WUS, as well as the network capability on whether supporting the corresponding subgrouping method(s).

Besides, for UE_ID used in UE_ID based PEI subgrouping mechanism, it is 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 considering the maximum number of UE_ID based PEI subgrouping is 8, i.e., the UE_ID is extended 8 times compared to no PEI is used.

Similarly, for UE_ID based LP-WUS subgrouping, the UE_ID for the calculation would also be extended. According to RAN2 discussion in RAN2#127 meeting:

In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately (if UE is configured to use both PEI and LP-WUS and if UE support both) in order to reduce the false paging rate. FFS on detailed configurations.

It is possible that the PEI and LP-WUS is used together and to achieve the purpose that LP-WUS subgroup ID is separate from the PEI subgroup ID, additional bits will be needed for UE_ID when using LP-WUS. In this case, similarly, the “Extended UE Identity Index Value” similar IE should be included in NG paging for CN paging and RRC_IDLE UEs, and it should be included in NGAP core Networking assistance information IE and Xn paging for RAN paging and for RRC_INACTIVE UEs. Besides, the F1 paging message should also include the IE to inform the DU. But the detailed UE_ID for LP-WUS should be determined in RAN2, anyway, introducing new bits for UE_ID is necessary according to our above analysis.

Proposal 3: Introducing new IE to include extra bits derived from 5G-S-TMSI for UE_ID based LP-WUS subgrouping into F1 paging message. The extra bits are used for DU to determine the LP-WUS subgrouping ID of the paged UE which applies UE_ID based LP-WUS subgrouping.

  - LP-WUS configuration generation

In RAN3#125bis meeting, we have the following agreement and FFS:

The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

Regarding the Entry/exit condition for LP-WUS monitoring, RAN2 has the following agreements in RAN2#126 meeting and RAN2#127 meeting:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

It could be observed that the entry/exit conditions only include the measurement related threshold, which is related to the physical layer or MAC layer, thus the DU should determine the entry/exit condition.

Besides, this issue is pure RAN3 related. RAN3 could make the decision and there is no need to inform RAN2 and RAN1.

Proposal 4: It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring, etc.

  - LP-WUS subgrouping number

According to RAN2 agreement, the maximum number for LP-WUS subgrouping has been agreed to be no less than 8, and the detail number is being discussed in RAN1, and would be determined further. In PEI design, the CN assigned subgrouping for LP-WUS is determined by CN, and then the maximum number of UE_ID based subgrouping is determined by the total subgroup number minus the CN assigned subgroup numbers.

During the discussion for PEI in Rel-17, some companies proposed the NG-RAN node should communicate with AMF to acquire the CN assigned subgrouping number and assign suitable UE_ID based subgrouping. However, most companies opposed this suggestion since the NG-RAN node could acquire the information via OAM and no need to introduce the enhancement between NG-RAN and AMF. We think the similar principle for LP-WUS could be re-used.

Proposal 5: NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

  - Emergency PDU session

In Rel-17 PEI, RAN2 discussed whether UE monitors PEI when it has an emergence PDU session [5]. In details, for UE supports CN assigned subgrouping, if there is emergency PDU session, the UE won’t inform the CN that it has the corresponding UE capability on CU assigned subgrouping.

When the UE has an active emergency PDU Session: - The UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

In this way, the CN won’t assign the subgroup ID for this UE [6]. Hence there won’t be paging latency for the CN assigned subgrouping. We think the similar method could be used for LP-WUS to avoid the paging latency during the emergency PDU session. However, it should be up to the SA2/CT1 decision.

Proposal 6: For CN assigned subgrouping, RAN3 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

For UE_ID based subgrouping, in the case of CN triggered paging for a UE with an emergency PDU session, the RAN doesn’t know if there is an emergency PDU session, thus the UE_ID based subgrouping for PEI is not able to be restricted which will introduce the paging latency. After the discussion within a few of meetings in RAN2, RAN2 concluded that disabling UE_ID based PEI is not supported. It means that UE still monitors the PEI with UE_ID based subgrouping when it has an emergency PDU session. Considering the time offset between PEI and its corresponding PO is small, e.g., 50ms, RAN2 thinks the latency introduced by UE_ID based PEI could be accepted. Besides, SA2 confirmed that the service impact is limited even without any enhancement on UE_ID based subgrouping since it only has impacts on the emergency call back scenario. Besides, other features in addition to the PEI may also cause the MT call set up delay, thus it is not a big deal.

Observation 4: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

However, since the time offset between LP-WUS and its corresponding PO may be much larger than the PEI scenario, the paging latency may not be acceptable when UE has an emergency PDU session. We need to discuss and collect the companies’ views on whether to resolve this paging latency issue during an emergency PDU session.

If the latency issue on the use of UE_ID based subgrouping for LP-WUS during the PDU session needs be addressed, we think the following options could be considered:

- Option 1: Introducing an indication in NG paging, RAN paging (if needed) and F1 paging to disable the LP-WUS function.

- Option 2: Leaving it to UE implementation, i.e., if a UE supporting UE_ID based subgrouping indicates its capability to gNB, when there is an active emergency PDU session, the UE should update its UE capability via indicating the gNB that it doesn’t support the UE_ID based subgrouping to gNB by UE implementation. After the emergency PDU session is released, the UE could update its UE capability again and indicate it supports the UE_ID based subgrouping to gNB by UE implementation. Similar to the legacy, when UE reports the capability, the RAN will indicate UE RADIO CAPABILITY INFO INDICATION to the CN, and CN will indicate this capability in CN paging for RRC_IDLE UEs, or in NGAP core Networking assistance information for RRC_INACTIVE UEs.

Proposal 7: RAN3 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation.

- Conclusions and Proposals

In this contribution, we discussed the RAN3 impacts to support paging subgrouping for LP-WUS including the CN assigned subgrouping and UE_ID based subgrouping in NG-RAN, besides other impacts are also discussed. Based on the discussion, we have the following observations and proposals.

Observations:

observation 1: The LP-WUS assistance information IE only includes the CN assigned LP-WUS subgroup ID.

Observation 2: UE capability on UE_ID based PEI subgrouping is included in the UE Radio Capability for Paging IE in PEI.

Observation 3: NG-RAN node could acquire the UE capability information on UE_ID based LP-WUS subgrouping, no RAN3 impacts for the detailed signalling design.

Observation 4: No enhancement is introduced for the use of PEI with UE_ID based subgrouping during an emergency PDU session.

Proposals:

Proposal 1: Whether anchor gNB provides the new IE for CN assigned subgroup ID into the Xn paging message for RAN paging should wait for further RAN2 progress.

Proposal 2: For LP-WUS subgrouping capability, wait for RAN2 decision on the UE capability for CN assigned subgrouping and/or UE_ID based subgrouping for LP-WUS, as well as the network capability on whether supporting the corresponding subgrouping method(s).

Proposal 3: Introducing new IE to include extra bits derived from 5G-S-TMSI for UE_ID based LP-WUS subgrouping into F1 paging message. The extra bits are used for DU to determine the LP-WUS subgrouping ID of the paged UE which applies UE_ID based LP-WUS subgrouping.

Proposal 4: It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring, etc.

Proposal 5: NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

Proposal 6: For CN assigned subgrouping, RAN3 assumes the mechanism for PEI when there is an active emergency PDU Session could be re-used for LP-WUS as below, and send an LS to SA2/CT1 on the decision.

- When the UE has an active emergency PDU Session, the UE shall not signal Paging Subgrouping Support Indication in the Registration Request message.

Proposal 7: RAN3 to discuss whether and how to address the latency issue for LP-WUS with UE_ID based subgrouping during an emergency PDU session. And the following solutions could be considered if it needs to be addressed:

- Option 1: Introducing an indication to disable the LP-WUS function in NG Paging, RAN paging and F1 Paging during an emergency PDU session.

- Option 2: Leaving it to UE implementation.

- References

RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO

RAN3 #125 Meeting Report

RAN3 #125-bis Meeting Report

R3-247007/R2-2409226, LS on LP-WUS subgrouping

R2-2209018 Report of [AT119-e][004] [ePowSav] Subgrouping/PEI (MediaTek)

3GPP TS 23.501: " System architecture for the 5G System (5GS); stage 2".