3GPP TSG-RAN WG3 Meeting #126	R3-247261

Orlando, US, 18 - 22 Nov, 2024

Title:	(TP for LTM BLCR for TS38.473): Further discussion on inter-CU LTM procedure

Agenda Item:	13.2

Source:	Huawei

Document for:	Other

1	Introduction

In previous RAN3 meetings, RAN3 discussed Rel-19 inter-CU LTM and made some agreements.

RAN3 #123bis: Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution RAN3 #124: Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. RAN3 #125: Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure RAN3 #125bis: Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered. The LTM Configuration IDs are allocated by the source CU. WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

In this contribution, we continue to discuss the basic procedures of inter-CU LTM, including the standalone case and the NR-DC case.

2	Discussion

2.1	LS in

2.1.1 Semi-persistent CSI-RS measurement

RAN3 received an LS [1] from RAN1, which requests RAN3 to analyse the feasibility of supporting semi-persistent (SP) CSI-RS measurement in inter-DU LTM and inter-CU LTM.

R1-2409283, LS on the support of semi-persistent CSI-RS resource for LTM candidate cells Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective The following working assumption was made in RAN1#118bis for candidate cell L1-RSRP measurement based on CSI-RS: RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case, and hence RAN3’s view is needed to confirm the working assumption. Therefore, RAN1 would like to ask RAN3 for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

The semi-persistent CSI-RS is designed to only send the CSI-RS when needed to reduce the transmission overhead over air interface. In order to support semi-persistent CSI-RS measurement in LTM, the UE needs to know the CSI-RS resource to be measured and the starting time to measure, which is also the point of time that the candidate gNB-DU starts to transmit the corresponding CSI-RS resource.

To align the measuring time and transmission time between the UE and the candidate gNB-DU, the source gNB-DU needs to coordinate with both the UE and the candidate gNB-DU which may bring some network signalling requirements for intra-CU inter-DU LTM and inter-CU LTM.

There are different options which are shown in the figure below. Intra-CU inter-DU LTM is taken as an example.

Figure 1. Procedures to support SP CSI-RS measurement in intra-CU inter-DU case

As illustrated in Figure 1, after receiving SSB measurement report from the UE, the source gNB-DU determines to start semi-persistent CSI-RS measurement for the candidate cell. Before sending the SP CSI-RS measurement indication to the UE, the source gNB-DU has three options to trigger the candidate gNB-DU to transmit the CSI-RS correspondingly.

- Option 1: The source gNB-DU determines the CSI-RS resources that to be transmitted, and sends the CSI-RS resource (set) IDs to the candidate gNB-DU via the gNB-CU to request the candidate gNB-DU to transmit the CSI-RS. In this option, the source gNB-DU needs to know the SSB and CSI-RS association of the candidate cell, to determine the suitable CSI-RS resources.

- Option 2: The source gNB-DU forwards the SSB measurement report to the candidate gNB-DU and the candidate gNB-DU determines the CSI-RS resources. The candidate gNB-DU sends that information back to the source gNB-DU.

- Option 3: The gNB-CU decides to trigger semi-persistent CSI-RS measurement and sends the selected CSI-RS resource (set) IDs to the candidate gNB-DU and the source gNB-DU based on L3 measurement results. The candidate gNB-DU starts to transmit CSI-RS following the indication of the gNB-CU, and the source gNB-DU triggers the UE measurement.

To answer the question in the LS [1] from RAN1, we think that it is feasible to support semi-persistent CSI-RS measurement for candidate cells in inter-DU and inter-CU LTM. As for choosing which option, RAN3 may continue to discuss and decide.

Proposal 1: RAN3 to reply RAN1 that it is feasible to support the semi-persistent CSI-RS measurement for candidate cells in inter-DU and inter-CU LTM.

Proposal 2: RAN3 to further discusse which option is supported for the semi-persistent CSI-RS measurement.

2.1.2 Security update in subsequent inter-CU LTM

How to update the security key is one key issue to be considered in subsequent inter-CU LTM, In RAN2 #125bis meeting, RAN2 provided four options and sent an LS [2] to SA3 to request SA3 to analyse those options. SA3 replied the LS [3] to RAN2 and also copied to RAN3.

S3-244316, LS on security handling for inter-CU LTM in non-DC cases, SA3 SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

According to the reply LS [3], most companies prefer the RRC solution: “After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.” However, SA3 is not sure about whether RRC signalling is allowed in subsequent cell switch procedure, so RAN2 needs to discuss and decide whether RRC signalling can still be transmitted. In SA3’s decision, if RRC signalling is not allowed, they will further analyse the MAC CE solution: the NCC value is delivered via MAC CE, which may have security threat.

From RAN3 perspective, we need to discuss how to deliver the updated key from the source gNB-CU to the candidate/target gNB-CUs. This discussion is needed no matter the RRC solution or the MAC CE solution is adopted.

At the network side, when the UE switches from the source gNB to the target gNB for the first time cell switch, upon path switch completion, the target gNB (gNB2 in Figure 2) receives the new (NH, NCC) pair from the AMF.  There are two options for the gNB2 to deliver the new keys to the candidate/target gNBs (gNB3 in Figure 2) for next LTM cell switch:

  - Option 1: The new serving gNB (gNB2) sends the new keys to all candidate gNBs before next target cell is determined.

  - Option 2: The new serving gNB (gNB2) sends the new keys to the next target gNB (gNB3) once after next LTM cell switch is executed.

The two options are illustrated in the Figure 2 below.

Figure 2. Procedures to deliver new keys from the source gNB to the target gNB

Proposal 3: RAN3 to discuss and decide how to update the new security keys to the candidate gNBs based on above two options in subsequent LTM.

2.2	Inter-CU LTM in standalone case

2.2.1 LTM Preparation

General

In RAN3 #125bis meetings, RAN3 made a working assumption:

RAN3 #125bis WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

One main obstacle to introduce a list of candidate cell in Xn handover request is the huge impact on the current specification. For example, in order to support different admission control results for different candidate cells, the Handover Request Acknowledgment message needs to be enhanced to carry a list of PDU Session Resources Admitted List. This issue was already acknowledged during the discussion for intra-CU LTM in Rel-18.

Therefore, we propose to turn the working assumption to an agreement,

Proposal 4: RAN3 to confirm that the handover preparation procedure is per candidate cell for inter gNB LTM.

Stage 3 design

a) L1 measurement and report configuration

RAN3 had the following agreement in RAN3 #125bis meeting:

RAN3 #125bis Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

The SSB configuration exchanged in F1/Xn Setup does not include the SSB transmit power information, which is needed in the UE LTM configuration. Therefore, in Rel-18, the SSB Time/Frequency Configuration is included in the UE Context Setup Response message from the candidate DU to the CU.

In Rel-19, similarly, it’s better to let the candidate gNB-CU to respond the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 5: The candidate gNB-CU responds the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

b) Early RACH configuration

In order to support PDCCH ordered early RACH, candidate DU first needs to allocate early RACH resources in the preparation step. To avoid RACH resource conflict, the early RACH resources of one candidate cell should be different for different source gNB-DUs. This is the reason why RAN3 introduced the source gNB-DUs List in Early UL Sync Configuration in Rel-18. The gNB-DUs List ensures that the candidate gNB-DUs can allocate non-overlapping RACH resources for different source gNB-DUs.

In Rel-19 inter-CU LTM, two gNB-DUs of difference gNB-CUs may have the same gNB-DU ID. Therefore, to avoid early RACH resource conflict, the pair of (gNB ID, gNB-DU ID) needs to be transferred to the candidate gNB-CU, and then the candidate gNB-DUs during handover preparation.

Similar with L1 measurement and report configuration, the early RACH configuration can be allocated by Handover Request and Handover Request ACK procedure.

Proposal 6: The source gNB-CU sends the pair of (gNB ID, gNB-DU ID) to the candidate gNB-CU via the Handover Request message.

Proposal 7: The candidate gNB sends the early RACH configuration to the source gNB-CU in the Handover Request Acknowledge message.

c) Reference configuration

RAN2 agreed to support one single reference configuration in Rel-19 inter-CU LTM, as shown below:

RAN2 #127 1. Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

In RAN2 #127bis meeting, such agreement is retained. Basically, it means that the source gNB-CU can generate a common reference configuration and sends the reference configuration to all candidate gNBs in Handover Request message.

Proposal 8: The source gNB-CU generate a common reference configuration and sends it to all candidate gNBs in Handover Request message.

2.2.2 Early Synchronization

In Rel-19 inter-CU LTM, in PDCCH ordered early RACH procedure, in order to transfer the TA information between the candidate gNB and the source gNB, RAN3 agreed to introduce a new procedure on Xn.

RAN3 #124 Introduce a new procedure on Xn to transfer the TA information.

In RAN3 #125 meeting, there was no consensus on whether the TA information transfer message is UE associated message or non-UE associated message.

According to the RAN2 progress on PDCCH ordered early RACH, there is no intention in RAN2 to further enhance the Rel-18 mechanism. In this case, when a candidate cell receives a preamble for early TA acquisition, it does not know the UE identity because the early RACH resources are shared by multiple UEs in the source cell. Therefore, the TA information transfer procedure should be a new non-UE associated class 2 procedure also on Xn interface.

Proposal 9: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

After the candidate gNB-DU receives a preamble for early TA acquisition, it transfers the TA value to the source gNB-DU. In Rel-18, on F1 interface, messages (e.g. DU-CU/CU-DU TA Information Transfer) were introduced to transfer the TA information from candidate gNB-DU to source gNB-DU. They can be re-used between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 10: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, to let the candidate gNB-CU know where to forward the TA information, a source gNB ID is need in the corresponding DU-CU TA Information Transfer message.

Proposal 11: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

2.2.3 LTM configuration sync up among candidate gNBs

In addition, to support subsequent inter-CU LTM, some LTM configurations, e.g., early sync configuration, configuration ID, and data forwarding addresses, etc may need to sync up among candidate gNBs.

In general, there are three options to sync up those LTM configurations among candidate gNB-CUs. Option 1 is to sync up just after the initial handover preparation. Option 2 is that the source gNB sends the configuration to the target gNB once the LTM is initiated. Option 3 is when receiving the successful access notification from the target gNB. The three options are illustrated in Figure 1.

Figure 3: LTM Configuration Update for subsequent inter-CU LTM

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

Basically, the three options above can be mapped to the three cases of NOTE 2.

Proposal 12: To use the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

2.2.4 Candidate gNB-initiated LTM Cancel

In RAN3 #125bis meeting, RAN3 had the following agreement to support candidate gNB-initiated LTM cancellation:

RAN3 #125bis Introduce a new procedure for candidate gNB-initiated LTM cancellation.

We prefer to name the new procedure as LTM Cancel.

Proposal 13: RAN3 to introduce a new class 2 procedure, namely, LTM Cancel for candidate gNB initiated LTM cancellation.

Because of the serving gNB change during subsequent LTM, it needs to discuss and decide to which node, the candidate gNB initiate the LTM cancel message .

One option is that the candidate gNB always contacts the initial source gNB, then the initial source gNB forwards the LTM cancel message to the current serving gNB of the UE. This makes the initial source gNB behave like an anchor gNB of the UE and requires the initial source gNB should be always informed by the new serving gNB after each LTM cell switch.

Another option could be that after each LTM cell switch, the target gNB (the new serving gNB) notifies all candidate gNBs about the serving gNB change. A new procedure is needed for this purpose, for example, SERVING GNB CHANGE NOTIIFICATION/ACK.

Since the first option may be not workable if the source gNB is not a candidate gNB in subsequent case, we propose the candidate gNB to contact the new serving gNB directly.

Proposal 14: To support candidate gNB initiated LTM cancel, the target gNB (new serving gNB) notifies all candidate gNBs about the serving gNB change by a new class 1 procedure after each LTM cell switch (e.g., Serving gNB Change Notification/Ack).

Proposal 15: The candidate gNB initiates the LTM Cancel message towards the current  UE serving gNB to cancel the prepared LTM candidate cells in it.

2.2.5 UE association setup between the new serving gNB and candidate gNBs

The following agreement and FFS were made at #125bis meeting.

RAN3 #125bis Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

Three options are discussed:

  - Option 1: During the LTM preparation phase, the candidate gNB allocates the UE XnAP ID(s) and send to other candidate gNBs.

  - Option 2: After each cell switch, the new serving gNB signals the UE XnAP ID (s) to other candidate gNBs, and then the candidate gNB reply with the candidate node UE XnAP ID.

  - Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB.

Option 1 and option 3 seems contradicting to the following stage 2 principle in section 6.4 in TS 38.401.

A UE-associated logical connection is established during the first NGAP/XnAP/F1AP message exchange between the NG/Xn/F1 peer nodes.

Therefore, option 2 is preferred.

If proposal 14 is agreed, option 2 may reuse the same procedure to setup the UE associated Xn signalling connection between candidate gNBs.

Proposal 16: Reusing the serving gNB change notification procedure to setup the UE associated Xn signalling connection between the new serving gNB and the candidate gNBs after each cell switch.

2.3	Inter-CU LTM in NR-DC

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

RAN3 #123bis: Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. RAN2 #126 An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

In order to make sure that the inter-CU LTM is not configured at the same time in MN and SN, inter-node interaction is needed. When the MN decides to configure inter-MN LTM candidates, the MN should send a notification to the SN to disable inter-SN LTM. When the SN decides to configure inter-SN LTM candidates, the SN should send a request to the MN. The MN may either agree or reject the request from the SN.

Proposal 17: When the MN decides to initiate inter-CU MCG LTM for a UE, it sends a notification to the SN.

Proposal 18: When the SN decides to initiate inter-CU SCG LTM for a UE, it sends a request to the MN, which may be accepted or rejected.

SN initiated inter-CU SCG LTM with MN unchanged

A LS from RAN2 in [4] is received containing the following RAN2 agreements related to inter-CU LTM.

R2-2409378: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. RAN2 #127bis 3. The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9. Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10. Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

Since only SN-initiated inter-CU SCG LTM is supported, and the procedure is similar with SN-initiated subsequent CPAC, except that the cell switch triggering in LTM is by MAC CE, instead of by event evaluation. We suggest to take the SN-initiated subsequent CPAC in TS 37.340 section 10.20 as baseline to design the inter-CU SCG LTM procedure.

Proposal 19: RAN3 to design the SN-initiated inter-CU SCG LTM by taking into account the above RAN2 agreements and take the SN-initiated subsequent CPAC procedure in TS 37.340 section 10.20 as baseline.

3	Conclusion

This document proposes:

LS in:

Proposal 1: RAN3 to reply RAN1 that it is feasible to support the semi-persistent CSI-RS measurement for candidate cells in inter-DU and inter-CU LTM.

Proposal 2: RAN3 to further discuss which option is supported for the semi-persistent CSI-RS measurement.

Proposal 3: RAN3 to discuss and decide how to update the new security keys to the candidate gNBs based on above two options in subsequent LTM.

LTM Preparation:

Proposal 4: RAN3 to confirm that the handover preparation procedure is per candidate cell for inter gNB LTM.

Proposal 5: The candidate gNB-CU responds the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 6: The source gNB-CU sends the pair of (gNB ID, gNB-DU ID) to the candidate gNB-CU via the Handover Request message.

Proposal 7: The candidate gNB sends the early RACH configuration to the source gNB-CU in the Handover Request Acknowledge message.

Proposal 8: The source gNB-CU generate a common reference configuration and sends it to all candidate gNBs in Handover Request message.

Early Synchronization:

Proposal 9: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 10: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 11: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

LTM configuration sync up:

Proposal 12: To reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Candidate gNB-initiated LTM Cancel:

Proposal 13: RAN3 to introduce a new class 2 procedure, namely, LTM Cancel for candidate gNB initiated LTM cancellation.

Proposal 14: To support candidate gNB initiated LTM cancel, the target gNB (new serving gNB) notifies all candidate gNBs about the serving gNB change by a new class1 procedure after each LTM cell switch (e.g., Serving gNB Change Notification/Ack).

Proposal 15: The candidate gNB initiates the LTM Cancel message towards the current  UE serving gNB to cancel the prepared LTM candidate cells in it.

UE association setup:

Proposal 16: Reusing the serving gNB change notification procedure to setup the UE associated Xn signalling connection between the new serving gNB and the candidate gNBs after each cell switch.

Inter-CU LTM in NR-DC:

Proposal 17: When the MN decides to initiate inter-CU MCG LTM for a UE, it sends a notification to the SN.

Proposal 18: When the SN decides to initiate inter-CU SCG LTM for a UE, it sends a request to the MN, which may be accepted or rejected.

Proposal 19: RAN3 to design the SN-initiated inter-CU SCG LTM by taking into account the above RAN2 agreements and take the SN-initiated subsequent CPAC procedure in TS 37.340 section 10.20 as baseline.

The reply LS to RAN1 is provided in [5]. The corresponding TPs are provided in the annex and [6].

4	Reference

[1] R1-2409283	LS on the support of semi-persistent CSI-RS resource for LTM candidate cells, RAN1

[2] R2-2404037	LS on security handling for inter-CU LTM in non-DC cases, RAN2

[3] S3-244316 	LS on security handling for inter-CU LTM in non-DC cases, SA3

[4] R2-2409378 	LS on RAN2 agreements for inter-CU LTM

[5] R3-247263	[DRAFT] Reply LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

[6] R3-247262	(TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure

Annex – Text Proposal for LTM BLCR for TS 38.473

/*********************Start of changes***********************/

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | M |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

/*********************Next change***********************/

#### 9.2.1.24	DU-CU TA INFORMATION TRANSFER

This message is sent by the gNB-DU to inform the gNB-CU about TA information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DU to CU TA Information List |  | 0..1 |  |  | YES | ignore |
| >DU to CU TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Source gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Source gNB ID | M |  | Global gNB ID 9.3.1.305 |  | - |  |

/*********************End of changes***********************/