3GPP TSG-RAN WG3 Meeting #126                                            R3-247272

Orlando, US, 18th – 22th, Nov, 2024

Source:	CATT

Title:	On support on-demand SSB SCell operation

Agenda Item:	17.2

Document for:	Discussion and Decision

Introduction

For NES on-demand SSB SCell operation topic, the WID[1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In last RAN1 and RAN2 meetings[4][2], some agreements are achieved on various aspects related to on-demand SSB SCell, the agreements are as follows:

Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement - For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement - For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain RAN2 Agreements: 1. No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. 2. Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. 3. NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we will continue to discuss the on-demand SSB SCell operation topic, our views will be given as well.

Discussion

2.1 On-demand SSB initiation and configuration

RAN1 has discussed different scenarios for Case#1 (no always-on SSB) and Case#2 (with always-on SSB periodically transmitted) for on-demand SSB. If the cell applies on-demand SSB, the SSB transmission opportunities are reduced and energy consumed by SSB transmission can be saved.

Considering on-demand SSB is designed for low traffic load, it’s not suitable to pre-configure on-demand SSB SCells, as the traffic load is variable. Thus, the cells to apply on-demand SSB should not depend on OAM configuration, it would be gNB-DU or gNB-CU to decide. Since SSB transmission is controlled and performed by gNB-DU, the energy used for SSB transmission is consumed by gNB-DU. Thus, the gNB-DU (instead of the gNB-CU) can be aware of energy consumed for SSB transmission. In addition, most of the energy used for base band processing are consumed by gNB-DU, so the energy efficiency of a cell can be obtained by gNB-DU. If the energy efficiency of the NES cell is low due to common signal transmission, e.g., when the user data rate is low or few UEs are connected to the cell, and the cell supports on-demand SSB operations, gNB-DU can apply on-demand SSB to the NES cell. When on-demand SSB is applied, SSB transmissions are stopped or decreased. The gNB-DU can also decide to not apply on-demand SSB due to increase of cell load. So, NES gNB-DU can initiate applying on-demand SSB.

In split architecture, gNB-DU is responsible for SCell configuration including configuration of SSB of the SCell to UE, so we assume the on-demand SSB SCell configuration for UE, including the configuration for on-demand SSB, should also be carried out by gNB-DU.

Besides the on-demand SSB SCell configuration, the cell applying on-demand SSB should support the on-demand SSB operations which are realized by MAC-CE or DCI signaling. The MAC CE or DCI is transmitted by gNB-DU. gNB-CU has no knowledge about whether a cell supports on-demand SSB SCell operations or not and thus cannot know whether to configure the on-demand SSB. So, on-demand SSB SCell should be configured by gNB-DU. There is no F1AP impact.

Observation 1: NES gNB-DU initiates on-demand SSB and takes the role of configuring on-demand SSB SCell.

2.2 Interaction over F1 interface

Whether NES gNB-CU needs to know the information of on-demand SSB

In the previous RAN1 meeting, RAN1 have the following agreement about whether on-demand SSB operation applies to CD-SSB or NCD-SSB:

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported. - On-demand SSB on CD-SSB or NCD-SSB the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB. FFS: Additional support of OD-SSB for CD-SSB located on sync-raster.

RAN1 has already supported that on-demand SSB on the cell is NCD-SSB, but the additional support of on-demand SSB for CD-SSB located on sync-raster is FFS.

As CD-SSB is required for configuring PCell/PSCell and on-demand SSB is not supported by PCell/PSCell operation, if CD-SSB applies on-demand SSB on the cell, the cell is not suitable to be used as PCell/PSCell. The gNB-CU should know that information and prevent to configure such kind of cells as SpCell to UEs. Furthermore, UE will not be able to camp on the cell, and therefore the network coverage is changed due to the NES cell applying on-demand SSB to CD-SSB. So gNB-CU should know that a cell is applying on-demand SSB if on-demand SSB is used for CD-SSB, to decide the coverage of the cell.

On the other hand, if on-demand SSB on a cell is used by NCD-SSB only, coverage of the cell is not impacted for on-demand SSB, and the cell is still possible to be configured as UEs’ PCell/PSCell. NES gNB-CU does not need to know whether the cell is applying on-demand SSB.

Observation 2: If on-demand SSB is only used to NCD-SSB, NES gNB-CU would not need to know whether on-demand SSB is applied.

RAN1 has the conclusion that on-demand SSB can be used for L1 and/or L3 measurement. However, according to the WID[1], the on-demand SSB operations only target to SCell. Thus on-demand SSB based L1/L3 measurement should only be applied to SCell. That means on-demand SSB is not used for the SpCell or neighbor cell measurements. In other words, gNB-CU should only configure on-demand SSB as the objective of SCell measurement.

Observation 3: NES gNB-CU should only configure on-demand SSB as the objective of SCell measurement, but not configure on-demand SSB as the objectives for SpCell or neighbor cell measurements.

In last RAN2 meeting, RAN2 has discussed how to support L3 measurement based on on-demand SSB, and the candidate solutions are as follows:

3. L3 RRM Framework - Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted Need more time to think of.

For option 1, measure OD-SSB based on different measurement configuration i.e. traditional SMTC measurement configuration. If RAN2 agrees to choose option 1, it is necessary for gNB-DU to indicate gNB-CU the SMTC configuration of on-demand SSB and the SMTC configuration is for on-demand SSB. Then gNB-CU could configure on-demand SSB as the objective of SCell measurement, and avoid configuring on-demand SSB as the objectives for SpCell or neighbor cell measurements. Note that the inter-node RRC message MeasurementTimingConfiguration defined in TS 38.331[3] is used for gNB-DU to convey the configurations of SSB/CSI-RS based measurement to gNB-CU over F1 interface. The legacy MeasurementTimingConfiguration message can be used for gNB-DU indicating whether a configuration of SSB based measurement is for on-demand SSB.

If RAN2 agrees to choose option 2, the on-demand pattern is informed to the UE and UE measures on-demand SSB based on on-demand SSB pattern. Since the legacy SMTC cannot be reused, new solution would be needed for gNB-DU indicating the measurement configuration of OD-SSB.

Observation 4: If RAN2 agrees to choose option 1, there would be impact on inter-node RRC message MeasurementTimingConfiguration, while if option 2 is selected, there would be impact to F1AP.

Proposal 1: To make sure the gNB-CU not to configure on-demand SSB as the objectives for neighbor cell measurements, the gNB-DU needs to indicate the gNB-CU whether a SSB based measurement is for on-demand SSB.

Whether gNB-CU needs to assist on decision of on-demand SSB activation

As discussed above, if CD-SSB applies on-demand SSB on the cell, the cell is not suitable to be used as PCell/PSCell and the cell coverage is impacted. The gNB-CU should know that information and needs to assist gNB-DU for deciding the cells to apply on-demand SSB. For example, gNB-CU can send the allowed cells list to gNB-DU for applying on-demand SSB to CD-SSB.

However, if on-demand SSB on a cell is NCD-SSB only, coverage of the cell is not impacted for on-demand SSB and the cell is still possible to be configured as UEs’ PCell/PSCell. In that case, gNB-DU can decide whether to apply on-demand SSB by itself, i.e. gNB-CU does not need to assist the gNB-DU to decide which cell to apply on-demand SSB. Nevertheless, as RAN1 has not conclude whether CD-SSB supports on-demand SSB, RAN3 should wait for RAN1’s conclusion on the issue of gNB-CU assisting gNB-DU on decision of on-demand SSB activation.

Observation 5: On the issue gNB-CU assisting gNB-DU to decide on-demand SSB activation, RAN3 wait for conclusion of RAN1 on whether CD-SSB supports on-demand SSB.

2.3 Interaction over Xn interface

Since the SSB applying on-demand SSB mode cannot be configured as objective of neighbor cell measurement, the neighbor nodes should not configure the on-demand SSB as a measurement objective.

Observation 6: The neighbor nodes of NES gNB should not configure an on-demand SSB based measurement.

To prevent the neighbor nodes configuring the on-demand SSB of NES cell as a measurement objective, one option is that the NES gNB does not convey the measurement configuration based on the on-demand SSB to the neighbor nodes over Xn, another option is that the NES gNB indicates to the neighbor nodes whether a SSB based measurement configuration is for on-demand SSB or not. RAN2 has discussed the two options to support L3 measurement as above.

If RAN2 agrees to choose option 1, gNB-DU indicates gNB-CU the SMTC configuration of on-demand SSB through inter-node RRC message MeasurementTimingConfiguration over F1AP interface and the NES gNB can transmit the SMTC of on-demand SSB transparently to the neighbour NG-RAN nodes over XnAP interface. If RAN2 adopts 2, the gNB-CU is not be able to send the OD-SSB based SMTC to neighbour nodes. It can be observed, no matter what option RAN2 selects, there is no impact to XnAP.

Observation 7: No impact to XnAP is observed.

Conclusion

According to the analysis in section 2, we have following observations and proposals:

Observation 1: NES gNB-DU initiates on-demand SSB and takes the role of configuring on-demand SSB SCell.

Observation 2: If on-demand SSB is only used to NCD-SSB, NES gNB-CU would not need to know whether on-demand SSB is applied.

Observation 3: NES gNB-CU should only configure on-demand SSB as the objective of SCell measurement, but not configure on-demand SSB as the objectives for SpCell or neighbor cell measurements.

Observation 4: If RAN2 agrees to choose option 1, there would be impact on inter-node RRC message MeasurementTimingConfiguration, while if option 2 is selected, there would be impact to F1AP.

Observation 5: On the issue gNB-CU assisting gNB-DU to decide on-demand SSB activation, RAN3 wait for conclusion of RAN1 on whether CD-SSB supports on-demand SSB.

Observation 6: The neighbor nodes of NES gNB should not configure an on-demand SSB based measurement.

Observation 7: No impact to XnAP is observed.

Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- Chair notes RAN1#118bis v20

- 3GPP TS 38.331 V18.3.0

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM