3GPP TSG-RAN WG3 #126                                             	    R3-247307

Orlando, USA, Nov 18th –22nd, 2024

Agenda item:	   8.1

Source: 		Qualcomm Incorporated

Title: 	NR UE capability storing limitation discussion and draft reply LS

Document for:	Discussion

# 1	Introduction

RAN3 received following LS R2-2409313 from RAN2:

RAN2 standard currently supports UL RRC message segmentation for transferring UECapabilityInformation message. Each segment can be as large as the maximum PDCP SDU size and the RRC signalling supports up to 16 segments.

RAN2 however came to the understanding that the existing network implementations do not always support the maximum number of RRC segments. RAN2 therefore agreed to introduce a new RRC signalling solution where the gNB indicates the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message. The UE then is required to generate UECapabilityInformation ensuring the size of the message does not exceed the maximum allowed size according to the gNB indication.

It was pointed out in the RAN2 discussion that the network limitation mentioned above can reside in RAN or CN (given that the reported UE capability needs to be stored in RAN and CN). In the case that it is CN’s limitation, RAN2 believes that signalling from CN to gNB is necessary so that gNB can indicate the correct value in UECapabilityEnquiry message.

RAN2 therefore would like to ask RAN3 to define a signalling solution where CN informs RAN about the maximum UE capability size that CN can accommodate.

In this paper, we express our view about different options to solve UE capability storing issue between RAN and CN.

# 2	Discussion

During RAN2 discussions, when UE is sending multiple UL RRC segments for UL Capability Information, if network is unable to store maximum allowed 16 UL RRC segments, gNB sends RRC Release message and thus causing UE to initiate RRC connection again and report UE capability with less number of UL RRC segments. UE is not aware of how many UL RRC UE capability segments can be stored by network and this procedure has to be repeated blindly and causing large delay for UE to establish RRC connection and successfully report UE capabilities with less number of segments.

In RAN2 discussion, operators have commented that UE capability storing limitation is either at gNB side or AMF side. In order to address this issue over the air-interface, RAN2 agreed that based on gNB indicated max. number of UL RRC segments storing limitation then UE will restrict max. number of UL RRC segments for UE Capability reporting.

When UE transition from RRC_CONNECTED state to RRC_IDLE state, gNB will send UE capability information to AMF for storing. In some AMF implementations, due to limited memory, it fails to store UE capability Information received from gNB. gNB is not aware of how many max UL RRC UE capability segments can be stored by AMF. This issue can be addressed if AMF provides what is max. memory size it can store for UE capabilities and gNB can determine what is max. number of RRC segments AMF can store without dropping.

Based on both AMF limitation and gNB limitation of max. number of UL RRC UE capability segments that can be supported, gNB can determine what value to be signalled to UEs.

AMF limitation can be provided to RAN through NGAP signalling (either via NG Interface management procedures or via UE context management procedures) or via OAM based approach.

Thus, we propose

Proposal 1 : RAN3 to discuss one of the following options to indicate gNB about AMF limitation of max. number of UE capability segments that can stored in AMF.

Option 1 : OAM based approach

Option 2 : NGAP signalling approach (either via NG interface management or UE context management signalling)

Proposal 2 : RAN3 to send reply LS to RAN2 indicating RAN3 agreed solution.

# Conclusion

This contribution discussed about how RAN will be informed about AMF limitation of max. size of UE capabilities that can be stored by AMF. Following are proposals.

Proposal 1 : RAN3 to discuss one of the following options to indicate gNB about AMF limitation of max. number of UE capability segments that can stored in AMF.

Option 1 : OAM based approach

Option 2 : NGAP signalling approach (either via NG interface management or UE context management signalling)

Proposal 2 : RAN3 to send reply LS to RAN2 indicating RAN3 agreed solution.

3GPP TSG RAN WG3 Meeting #126		R3-24xxxx

Orlando , USA, Nov, 18th – 22nd , 2024

Title:	Reply LS on UL RRC message segmentation for UECapabilityInformation

Response to:	R3-247011/R2-2409313 LS on UL RRC message segmentation for UECapabilityInformation

Release:	Rel-17

Work Item:	TEI 17

Source:	Qualcomm Inc [to be RAN3]

To:	RAN WG2

Contact person:	Prasad Kadiri

pkadiri@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 would like to thank RAN2 for the LS on UL RRC message segmentation for UECapabilityInformation.

RAN3 discussed following options about how to indicate to gNB about AMF limitation of max. number of UL RRC UE capability segments that can be stored in AMF.

Option 1 : RAN OAM based

Option 2 : NGAP Signalling based

RAN3 agreed to adopt RAN OAM approach or NGAP signalling approach.

# 2	Actions

To RAN2

ACTION: RAN3 respectfully asks RAN2 to take the RAN3 reply into account.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#127	17th – 21st February    		Athens, Greece

RAN3#127bis	07th – 11th April    		China