3GPP TSG-RAN WG3 Meeting #126	R3-247317

Orlando, US, 18th – 22nd November 2024

Agenda item:	14.3

Source:	Xiaomi

Title:	(TP to BL CR TS 38.300) Support of Regenerative Payload

Document for:	Other

# 1	Introduction

In this contribution, we’d like to discuss the LS from SA2 [1], NG connection handling related issue and mapped cell ID issue.

# 2	Discussion

### LS from SA2 R3-247016

SA2 send an LS to RAN3 to ask RAN3 to apply the NG impacts in NR IoT to S1 in LTE NTN, details are in below.

1. Overall Description: SA2 thanks for the LS on Support of Regenerative-based Satellite Access. SA2 identified that the LS from RAN3 referred only to “5GS” while in the last paragraph also referred to “AMF/MME”. SA2 also noticed that the RAN WI “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” (RP-242397) did not contain objectives for Regenerative-based Satellite Access for E-UTRAN/EPC. SA2 would like to inform RAN3 that the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS and therefore the related RAN3 impacts can affect both S1 and NG procedures. On the contrary, conclusions for FS_5GSAT_Ph3 for “KI#2: Support of Store and Forward Satellite operation” in clause 8.2 of TR 23.700-29 apply only to E-UTRAN/EPS. 2. Actions: To RAN3, RAN: ACTION: RAN3 and RAN is requested to take the above information into account.

According to RAN3’s WI in RP-241667 and RP-242397, LTE NTN is not in the NR NTN enhancement WI scope, on the other hand, S1 via feeder link is not in the IoT NTN WI scope. Whether to support S1 via feeder link in LTE NTN should be discussed in RAN plenary.

Observation 1, whether support S1 in LTE NTN should be discussed in RAN plenary.

Proposal 1, RAN3 noted the LS and no need to reply.

### NG connectivity handling

In regenerative payload NTN, especially for the non-GEO satellite, due to the satellite movement, the TNL connectivity between the satellite and the ground segment is dynamic changing. However, this does not mean the NG connectivity between the onboard gNB and on-ground AMF has short window.

We believe that the following two cases shows that the NG connection can remain active even when there is a feeder link switch from one gateway to another:

- In case 1, as shown in figure 1, even if gNB1 loses its feeder link connection with NTN GW, if there are intersatellite links between satellites, the NG connection between gNB1 and AMF can still active.

- In case 2, as shown in figure 2, gNB1 may still connect with AMF via NTN GW2.

Figure 1

Figure 2

In theory, if there’re enough GW deployed and inter-satellite links are available, the NG connectivity can also be always on, however, when the satellite moves to one side of the earth, while the AMF is located in the other side, considering the transmission delay, it may not be necessary to keep the NG connection.

In our view, the NG connection may be always-on, it can also be removed due to unavailable the feeder link or unavailable inter-satellite link or low transmission efficiency. The disconnection time does not depend on a specific connection between on satellite and one gateway, it depends on the on-ground gateway deployment and availability of the inter-satellite link, which means the disconnection time may not last too long.

With the above understanding, we think the following should be discussed before NG removal is performed.

Node level context handling

Considering the disconnection may not last too long and the satellite may re-connect the AMF frequently. We think it would be worth to keep some node level context between gNB onboard and AMF on ground. Our proposal is that, when NG removal is triggered, the gNB on board can decide whether to keep some node level contexts between the gNB and the AMF. NG suspend/resume can be introduced as an optional feature for operators when dealing with frequently removed or re-setup of the NG connections. To minimize specification impacts, it can be implemented as an optional indicator or flag in NG removal requests. For example, if this indicator is included in the NG removal request, the AMF can store node configuration information.

UE level context handling

When a gNB onboard decides to trigger NG removal, before it sends the NG removal request, it can trigger handovers for the RRC_CONNECETED UEs, and release the contexts for the RRC_INACTIVE UEs. One may say it’s possible to keep UE context in the gNB, which means the UE contexts are retained between gNB and AMF, this topic may relate and depend on the discussion on how to support INACTIVE UE in the context of NTN.

Observation 2, In theory, the NG connection can be always-on by the well deployed on-ground gateways and inter-satellite links, but it’s not necessary to keep the connection all the time as the AMF may be far away from the satellite gNB.

Observation 3, the disconnection time between onboard gNB and on-ground gateway depends on the on-ground gateway deployment, availability of the inter-satellite link, and transmission latency trade-off, which means the disconnection time may not last too long.

Observation 4, it would be worth to keep the node level context if the gNB onboard will frequently re-connect to the same AMF.

Proposal 2: RAN3 agree to an indicator in the NG removal procedure to retain some node-level configurations in both gNB and AMF.

Proposal 3, before sending the NG removal request, the RRC_CONNECTED UEs can be moves to other AMF/gNB by performing handover. The context of the RRC_INACTIVE UEs can be either released or relocated to other gNBs. RAN3 agree the corresponding TP in the Annex.

### Mapped cell ID

In Non-GEO Satellites, due to their orbital movement, their satellite beams cover different portions of that area. In R17, for transparent payload, mapped cell ID is introduced to solve the issue. And RAN3 agrees this mapped cell ID can be reused in regenerative payload.

We have concerns on whether the mapped cell ID have relation to the gNB ID of the moving satellite-gNB in NTN regenerative payload.

If the mapped cell ID has no relation to the gNB ID of the moving satellite-gNB, we think there’s no issue, the mapped cell ID can be planed and setup by OAM.

If the mapped cell ID includes the gNB ID of the moving satellite-gNB, meanwhile it should represent the fixed area on-ground, the cell ID will be changed as the satellite moves, we have concerns on whether the cell ID can be enough to represent all the areas that the satellite covers. According to the NCGI content as below, the minimum cell number can be assigned to a gNB is range 0-15 in case 4 bits is allocated to cell ID.

No matter how the mapped cell ID designs, we want to make it clear, so we propose RAN3 to discuss this issue.

Observation 5: It is not clear whether the mapped cell ID follows the same planning as the legacy CGI, which includes the gNB ID of a gNB that can be moving if it’s deployed on the satellite.

Observation 6, if the mapped cell ID include the gNB ID, the cell ID may not be enough to represent all the fixed areas on ground.

Proposal 4, RAN3 discuss whether the mapped cell ID has relation to the gNB ID.

# 3	Conclusion

Based on the discussion, we have the following observations and proposals.

### LS from SA2 R3-247016

Observation 1, whether support S1 in LTE NTN should be discussed in RAN plenary.

Proposal 1, RAN3 noted the LS and no need to reply.

### NG connectivity handling

Observation 2, In theory, the NG connection can be always-on by the well deployed on-ground gateways and inter-satellite links, but it’s not necessary to keep the connection all the time as the AMF may be far away from the satellite gNB.

Observation 3, the disconnection time between onboard gNB and on-ground gateway depends on the on-ground gateway deployment, availability of the inter-satellite link, and transmission latency trade-off, which means the disconnection time may not last too long.

Observation 4, it would be worth to keep the node level context if the gNB onboard will frequently re-connect to the same AMF.

Proposal 2: RAN3 agree to an indicator in the NG removal procedure to retain some node-level configurations in both gNB and AMF.

Proposal 3, before sending the NG removal request, the RRC_CONNECTED UEs can be moves to other AMF/gNB by performing handover. The context of the RRC_INACTIVE UEs can be either released or relocated to other gNBs. RAN3 agree the corresponding TP in the Annex.

### Mapped cell ID

Observation 5: It is not clear whether the mapped cell ID follows the same planning as the legacy CGI, which includes the gNB ID of a gNB that can be moving if it’s deployed on the satellite.

Observation 6, if the mapped cell ID include the gNB ID, the cell ID may not be enough to represent all the fixed areas on ground.

Proposal 4, RAN3 discuss whether the mapped cell ID has relation to the gNB ID.

# 4	Reference

[1] R3-247016, Reply LS on Support of Regenerative-based Satellite Access, SA2

# 5	Annex (TP to BL CR 38.300)

<<<<<<<<<<<<<<<<<<<< Change starts>>>>>>>>>>>>>>>>>>>>

### 16.14.x	NG Removal

For the case of regenerative payload, when disconnecting from an AMF is required due to e.g. the satellite movement in a LEO constellation, the gNB on board the satellite may request the removal of the NG interface by initiating the NG Removal procedure toward the AMF. Before initiating the NG removal request, the onboard gNB should initiate handovers to move the RRC_CONNECTED UE(s) associated with the NG connection to other AMF/gNB(s). Concurrently, it should either release or relocate the contexts of RRC_INACTIVE UE(s).

Editor’s Note: NG Removal procedure may also be used for WAB; how to capture this for WAB may need further discussion (pending WAB discussion).

<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>