3GPP TSG-RAN WG3 Meeting #126	R3-247320

Orlando, US, 18th – 22nd November 2024

Agenda item:	20.2

Source:	Xiaomi

Title:	(TP to BL CR 38.455) Support of LMF-side model (case 3b)

Document for:	other

# 1	Introduction

RAN3#125bis:

Consider aggregated gNB case first, then split architecture.

The existing LCS framework is re-used for AI/ML positioning.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we’d like to further discuss the leftover issues for case 3b

# 2	Discussion

Figure 1 illustrates a potential deployment scenario for case 3b, focus on the interaction between gNB and LMF, based on the functional framework detailed in section 4.4 of TR 38.843[1].

Figure 1 deployment scenario for case 3b

According to RAN1 agreements for training data and inference data, the training data and inference data both include the channel measurement, quality indicator and time stamp,  the measurement procedure can be re-used.

Agreement (RAN1#116) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

Agreement (RAN1#118bis) From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement.

The data collection for case 3b can be summarized as in below table:

|  | gNB to LMF | UE to LMF |
| --- | --- | --- |
| Training data | • channel measurement • quality indicator of channel measurement • time stamp of channel measurement | • ground truth label (or its approximation) • quality indicator of label • time stamp of label |
| Inference data | • Channel measurement; • Quality of the channel measurement; • Time stamp of the channel measurement. |  |

In last RAN3 meeting, it’s agreed to re-use existing signalling to provide input data for training and inference to LMF, but the details are pending to RAN1.

Regarding channel measurement, there’re two types of measurements are studied in RAN1: sample-based and path-based.

If path-based measurements are used, the existing measurement signalling can be reused.

Agreement (RAN1#116) Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

For sample-based measurements, based on the following RAN1 agreements, it is one kind of new measurement type compare to existing path-based measurement. New measurement type and information needs to be introduced in measurement procedures, but FFS on the value of the IE. The impacts to NRPPa can be found in the Annex.

Agreement (RAN1#117) Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement (RAN1#118) For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Observation 1, existing measurement types can be reused for path-based measurements.

Observation 2, new measurement type and parameters need to be introduced for sample-based measurements.

Proposal 1, RAN3 agrees to introduce a new measurement type “sample-based” in measurement type IE and sample-based measurement information IE in Measurement Request message, and introduce sample-based measurement results in the TRP Measurement Result IE, to support sample-based measurements for case 3b.

For the quality indictor and time stamp of the measurement, they are already included in TRP Measurement Result IE as below, so that the existing IEs can be re-used.

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>; -. >CHOICE Measured Results Value; M; -. >>UL Angle of Arrival; 9.2.38. >>UL SRS-RSRP; INTEGER (0..126). >>UL RTOA Measurement; 9.2.39. >>gNB Rx-Tx Time Difference; 9.2.40. >>Z-AoA; 9.2.67; YES; reject. >>Multiple UL-AoA; 9.2.71; YES; reject. >>UL SRS-RSRPP; 9.2.72; YES; reject. >>UL RSCP; 9.2.92; YES; reject. >>Sample-based Measurement Results; 9.2.y; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -.

Proposal 2, RAN3 agree to re-use the time stamp and measurement quality in TRP Measurement Result IE

Model monitoring

For monitoring data, it is unclear whether any assistance information is needed from the gNB side or if the LMF can handle it independently. We can wait for RAN1’s clarification.

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

Observation 3, According to RAN1 agreements, it is unclear whether and what assistance information and/or measurement reports may be sent from gNB to LMF to assist with performance monitoring.

Proposal 3, RAN3 wait for RAN1’s progress on the monitoring data from gNB for case 3b.

If RAN split architecture is supported, the F1 impacts can be the same as NRPPa impacts.

Observation 4, if RAN split architecture is supported, the F1 impacts can be the same as NRPPa impacts.

# 3	Conclusion

In this contribution, we discussed the AI-based positioning from RAN3 perspective and have the following proposals.

Observation 1, existing measurement types can be reused for path-based measurements.

Observation 2, new measurement type and parameters need to be introduced for sample-based measurements.

Proposal 1, RAN3 agrees to introduce a new measurement type “sample-based” in measurement type IE and sample-based measurement information IE in Measurement Request message, and introduce sample-based measurement results in the TRP Measurement Result IE, to support sample-based measurements for case 3b.

Proposal 2, RAN3 agree to re-use the time stamp and measurement quality in TRP Measurement Result IE

Observation 3, According to RAN1 agreements, it is unclear whether and what assistance information and/or measurement reports may be sent from gNB to LMF to assist with performance monitoring.

Proposal 3, RAN3 wait for RAN1’s progress on the monitoring data from gNB for case 3b.

Observation 4, if RAN split architecture is supported, the F1 impacts can be the same as NRPPa impacts.

# 4	Reference

[1] 3GPP TR 38.843 V18.0.0 (2023-12), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

# 5	Annex TP to 38.455 (support of case 3b)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, Sample-based) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |
| Sample-based Measurement Information | ifMeasSampleBased |  | 9.2.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |
| ifMeasSampleBased | This IE shall be present if the TRP Measurement Type IE is set to “Sample-based” |

Editor note: the details of sample-based measurement information is pending to RAN1.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Sample-based Measurement Results |  |  | FFS |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

Editor note: the details of Sample-based Measurement Results is pending to RAN1.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>