3GPP TSG-RAN WG3 Meeting #126	R3-247321

Orlando, US, 18 – 22 November, 2024

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.473 and for TS 37.340) MRO Enhancements for LTM, CHO with Candidate SCG(s) and S-CPAC

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM, CHO with candidate SCG(s) and S-CPAC in Rel-19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #125bis [1]:

- For TP to BL CR to TS 38.300:

  - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

  - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

  - Change“successful handover" definition, as proposed in R3-245230.

- For TP to BL CR to TS 38.401:

  - Capture wrong LTM prepared cell selection.

- Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

- CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

The agreements achieved in RAN3 #125 [2]:

- Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

FFS to enhance failure scenario.

- Take the existing detection mechanism descriptions as baseline.

- For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

- CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed.

The agreements achieved in RAN2 #127bis [3]:

- The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure.

- If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD.

- Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

- Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

- Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell.

- UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

The agreements achieved in RAN2 #127 [4]:

- Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

- -introduce a new field in RLF report to indicate the LTM recovery cell id.

- RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch.

- We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

- RA-based access will not be a new triggering condition for the SHR report.

- HO interruption time will not be a new triggering condition for the SHR report

- Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

- We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

- We will not log interruption time in SON reports (e.g. SHR) for LTM.

- We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not.

- Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

### 2.1.2 Near Failure Monitoring for MCG LTM

Main benefits of the LTM shall be seen in avoiding the complete reset of L3, avoiding RACH procedure which shall result into reduction of the interruption time. The interruption time is significantly impacted with RACH procedure. In case the TA of the candidate target LTM cell is not included into Cell Switch Command (MAC CE) the UE will initiate the legacy RACH procedure to access the target candidate cell. Despite the procedure can be monitored by means of T304 timer, but the quality in terms of having it in a RACH-less manner is not considered. However, it is essential to get information whether the cell change occurred RACH-less which is key for LTM. It resulted into proposal to define new SHR triggers specific to LTM near failure cases related to RACH-based LTM by some of companies. However, during the RAN2 # 127 meeting it was agreed not to use this as a new trigger event for SHR for LTM. As we think this conclusion in RAN2 was made quickly and not considering all the aspects we propose to discuss it again in RAN3.

Proposal 1-1: RAN3 discuss new SHR trigger specific to LTM near failure cases related to RACH-based LTM.

On the other hand, in case the RACH-less cell change is triggered, i.e. MAC-CE msg contains TA, the first UL transmission is done in RLC AM. So, it may require several RLC retransmissions to get the transmission through.

Proposal 1-2: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM as new SHR trigger.

### 2.1.4 Connection Failure Monitoring for MCG LTM

Based on the agreements the gNB-CU is responsible for optimizing LTM candidate cells. Two options are possible to optimize the LTM candidate cell list:

- UE based solution

- Network based solution

LTM compared to CHO supports subsequent LTM and therefore the UE stores LTM configuration also after successful LTM execution. UE can therefore report LTM candidate cell list also in scenario when RLF occurred shortly after successful LTM execution.  On the other hand, as Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

Observation 1-1: To optimize LTM candidate cell list both UE and network-based solutions may be used for Rel-18 LTM cell switch.

As the info on LTM candidate cells is available in the network (gNB-CU) it is proposed not to ask UE to monitor this information.

Proposal 1-3: RAN3 agrees on network-based solution for optimizing the LTM candidate cell list.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report together with the failure type to the DU responsible for the failure as agreed in RAN3 # 125bis meeting:

- Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

Based on the above agreement the TP R3-245832 was agreed to update the TS 38.473 and add in addition also C-RNTI in case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure. However the reasoning why the C-RTNI is added was not agreed to be part of TS 38.401 in the related TP R3-245831. It is therefore proposed to update the Semantics description for the C-RNTI in TS 38.473 as proposed in the attached TP.

Proposal 1-4: RAN3 agrees the attached TP for TS 38.473 to update the Semantics description for the C-RNTI.

There may be a plurality of problems causing the RLF. Besides the wrong cell selection at cell switch and wrong cell switch timing the problem may be related to wrong or missing TA in the Cell switch command.  In general, it may be related to an issue in Early TA Acquisition procedure which oversees source DU. Despite the TA was included in Cell Switch Command it may become outdated because the UE significantly moved between the time of the Early TA Acquisition and the time of the LTM execution. The UE then fails to attempt the target LTM cell and had to switch RACH based access via LTM recovery/re-establishment procedure when new TA value is provided from network in RAR message. We think that reporting of the information about the difference between the TA reported in the Cell switch command for which the RA less LTM failed, and TA received consequently in RAR for recovery/re-establishment cell may help to optimize the wrong cell switch timing.

- Proposal 1-5: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed and TA received in RAR for recovery/re-establishment cell.

## 2.2 MRO for CHO with Candidate SCG(s)

### 2.2.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #125bis [1]:

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

The agreements achieved in RAN3 #125 [2]:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 2c, 3c, 7c, 8c are FFS.

The RLF report includes:

  - The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions before the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the RLF is encountered

  - FFS whether to optimize the association of the PCell and PSCell.

The agreements achieved in RAN2 #127bis [3]:

UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

The agreements achieved in RAN2 #127 [4]:

UE includes following information in RLF report:

Time information regarding condition fulfilment for CHO with candidate SCGs. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

Measurement results of PCells and PSCells.

### 2.2.3 Near Failure Monitoring for CHO with Candidate SCG(s)

During the RAN3 # 125 meeting all the companies agreed the following:

Study solutions to reduce the time between fulfilment of CHO execution condition and CPAC execution condition.

In case of CHO with candidate SCG(s), it may happen that if one of the execution conditions is not configured properly, the entire execution will be delayed as the configuration cannot be executed partially. Even the CHO with candidate SCG(s) is at the end successfully completed, the delayed execution may have negative impact on end user when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier. If execution completion fails to either both or one of them the end user will perceive it as a complete drop or as degradation of provided QoS in case of SCG execution completion failure. Even there is a general agreement between the companies on the need to monitor the time between fulfilment the associated conditions the details of such monitoring and optimization steps to reduce this time have not been discussed yet. In the remaining part of this chapter, we focus on near failure monitoring as it is more probable having both conditions fulfilled in this scenario than in connection failure scenario.

Example 1

Let us consider a UE is connected to PCell 1 and PSCell 1. The UE is configured with CHO with candidate SCG(s) configuration PCell 2 and PSCell 2.  Let’s further consider the UE starts evaluating the execution conditions of candidate PCell 2 and PSCell 2 simultaneously and after some time the CPC execution conditions of the candidate PSCell 2 are fulfilled first and then with some delay also the CHO execution conditions for candidate PCell 2 are fulfilled. Then for near failure monitoring from end user perspective when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier we see the following possible scenarios. The scenarios primarily focus on CHO with CPC. It is mentioned where it is applicable also for CHO with CPA.

1a) during the wating for fulfilment of the CHO execution conditions, the T310 on SCG of PSCell 1 is started. The T310 time value is above the configured T310 threshold to trigger the SPR. This case is applicable only for CPC not for CPA.

1b) the T310 on SCG of PSCell 1 is started before the CPC execution conditions are fulfilled. The T310 time value is above the configured T310 threshold to trigger the SPR. This case is applicable only for CPC not for CPA.

1c) during the wating for fulfilment of the CHO execution conditions the T310 on SCG of PSCell 1 is started and also T310 on MCG for PCell 1 is started. The T310 time value both for MCG and SCG is above the configured T310 threshold to trigger the SHR and SPR, respectively. This case is applicable only for CPC not for CPA.

1d) during the wating for fulfilment of the CHO execution conditions the T310 on MCG for PCell 1 is started. The T310 is above the configured T310 threshold to trigger the SHR. This case is applicable also for CPA.

1e) during the wating for fulfilment of the CHO execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable.

Observation 2-1: The scenario 1a when during the wating for fulfilment of the second CHO execution conditions the T310 on SCG of PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR may require the optimization of the CHO execution conditions.

Observation 2-2: The scenario 1b when the T310 on SCG of PSCell 1 is started before the CPC execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SPR requires the legacy optimization of the CPAC execution conditions.

Observation 2-3: The scenario 1c when during the wating for fulfilment of the CHO execution conditions the T310 on SCG of PSCell 1 is started and T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR and SHR may require the optimization of the CHO execution conditions.

Observation 2-4: The scenario 1d when during the wating for fulfilment of the CHO execution conditions T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR requires the optimization of the CHO execution conditions.

Observation 2-5: The scenario 1e when during the wating for fulfilment of the CHO execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Example 2

Let us consider a UE is connected to PCell 1 and PSCell 1. The UE is configured with CHO with candidate SCG(s) configuration PCell 2 and PSCell 2.  Let’s further consider the UE starts evaluating the execution conditions of candidate PCell 2 and PSCell 2 simultaneously and after some time the CHO execution conditions of the candidate PCell 2 are fulfilled first and then with some delay also the CPC execution conditions for candidate PSCell 2 are fulfilled. Then for near failure monitoring from end user perspective when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier we see the following possible scenarios. The scenarios primarily focus on CHO with CPC. It is mentioned where it is applicable also for CHO with CPA.

2a) during the wating for fulfilment of the CPAC execution conditions the T310 on MCG of PCell 1 is started. The T310 time value is above the configured T310 threshold to trigger the SHR. This case is applicable also for CPA.

2b) The T310 on MCG of PCell 1 is started before the CHO execution conditions are fulfilled. The T310 time value is above the configured T310 threshold to trigger the SHR. This case is applicable also for CPA.

2c) during the wating for fulfilment of the CPC execution conditions the T310 on MCG of PCell 1 is started and also T310 on SCG for PSCell 1 is started. The T310 time value both for MCG and SCG is above the configured T310 threshold to trigger the SPR and SHR, respectively. This case is applicable only for CPC not for CPA.

2d) during the wating for fulfilment of the CPC execution conditions T310 on SCG for PSCell 1 is started. The T310 time value is above the configured T310 threshold to trigger the SPR. This case is applicable only for CPC not for CPA.

2e) during the wating for fulfilment of the CPAC execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable.

Observation 2-6: The scenario 1a when during the wating for fulfilment of the second CPAC execution conditions the T310 on MCG of PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR may require the optimization of the CPAC execution conditions.

Observation 2-7: The scenario 2b when the T310 on MCG of PCell 1 is started before the CHO execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SHR requires the legacy optimization of the CHO execution conditions.

Observation 2-8: The scenario 2c when during the wating for fulfilment of the CPC execution conditions the T310 on MCG of PCell 1 is started and T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR and SPR may require the optimization of the CPAC execution conditions.

Observation 2-9: The scenario 2d when during the wating for fulfilment of the CPC execution conditions T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR requires the optimization of the CPAC execution conditions.

Observation 2-10: The scenario 2e when during the wating for fulfilment of the CPAC execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Note: In some of the above scenarios for simplicity only T310 is considered but it may be extended also to T312.

Proposal 2-1: RAN3 agrees to consider the above scenarios 1a,b,c,d,e and 2a,b,c,d,e as a baseline to study improvement of the UE reports for near failure monitoring for UEs with CHO with Candidate SCG(s) only configured.

### 2.2.4 Connection Failure Monitoring for CHO with Candidate SCG(s)

During RAN2#127bis, it was decided to log the following information in RLF report in case of failed CHO w/ candidate SCG:

UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

Apart from this information, we believe it may be beneficial for more information to be collected by the UE to aid the network in better understanding the root cause of the failure and decide on appropriate corrective actions. If only one execution condition was met before the RLF on MCG was detected, the network may benefit from knowing how close the second execution condition was to fulfilment at that time, for example if TTT was running or not for the second condition.

Proposal 2-2: In case RLF on MCG is detected before execution of CHO w/ candidate SCG, the RLF report should be enhanced with additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not.

SCG failure may also be detected before, during or shortly after execution of CHO w/ candidate SCG, as subcase b) in the RAN3 agreed scenarios. In this case, only SCGFailureInformation will be generated, not RLF report. The enhancement of SCGFailureInformation is still FFS in RAN2. The current content of SCGFailureInformation only includes PSCell measurements but no PCell measurements. Also, MN may not have up to date PCell measurements reported by the UE. This makes it hard for the network to know or deduce which execution condition (CHO or CPAC or both) is to blame for stalling or the failure in the execution of CHO w/ candidate SCG.

Observation 2-11: In case SCG failure is detected before, during or shortly after the execution of CHO w/ candidate SCGs, the only report generated will be the SCGFailureInformation which only contains PSCell measurements.

In order to aid the network in determining which execution condition/event needs reconfiguration to avoid failures in the future, the SCGFailureInformation message could also be enhanced with the same information as the RLF report in this case.

Proposal 2-3: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled.

## 2.3 Optimisation of the CPAC and S-CPAC use (MRO for S-CPAC)

The consequences of the use of S-CPAC introduced in Rel-18 (NR_Mob_enh2) [5] have been discussed in [6], here we remind them briefly:

Normal Rel-17 CPAC-configured UE has to release the CPAC configurations when completing random access towards the target PSCell. The advantage of S-CPAC, where multiple cells are prepared, is that an immediate UE autonomous execution for fast subsequent cell changes is possible, since conditional configuration for other candidates are not released after PSCell change. The justification of the feature in [2] was that in the case of frequent SCG changes when operating FR2 delays and signalling overhead can be avoided.

However, if such an immediate cell change is not needed, the resource blockage resulting from the prepared and unused candidate cells can be seen as disadvantage, as well as the increased UE memory needed because of the numerous condConfig options for each potential future hop. Obviously, there are disputable pros and cons for the two CPAC methods (classical vs. subsequent) which depend on dedicated cell layout situations. In case of fast subsequent cell changes the “Subsequent CPAC” is the preferred method.

Observation 3-1: Depending on the cell deployment and UE mobility, usage of either CPAC or S-CPAC may result in more efficient resource utilisation in involved cells.

The optimality of S-CPAC usage depends mainly on the time between needed PSCell changes. The shorter time between two consecutive CPCs, the more preferable is S-CPAC. If time is such short that UE fails with RLF before candidate cell preparation and UE configuration is completed, it is obvious that S-CPAC might be beneficial. Long residence time, on the other hand, hints usage of a the classic CPAC, so that resources are not blocked in prepared cells, while there is enough time for proper CPAC preparation.

Observation 3-2: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes.

The key aspect is monitoring the time between PSCell changes: it shall not be too short in case of CPAC, while not too long in case of S-CPAC. This must be monitored per-cell pair, since cell change criteria are set per source cell towards each target cell.

Proposal 3-1: RAN3 agrees on enabling of the optimal usage of S-CPAC as part of SON based optimization

In principle, the residence time is logged in the UE history information (UHI). The problem with the UHI is that it does not provide information about the SCG mobility type (i.e. whether the UE may be configured at the same time with CPAC or S-CPAC). And reporting can be requested anytime unsynced with issue the resources where blocked unnecessary for long time..

In theory, the network may monitor the inter-change time as well. The problem with this approach is that the MN, which coordinates S-CPAC (also in case of SN-initiated S-CPAC the MN is the node providing communication with other SNs) may not be aware of intra-SN PSCell changes. Also, XnAP signalling between the MN and involved SNs affects time monitoring. Thus, network-based solutions are not feasible here.

In order to enable optimal usage of S-CPAC, it might be better that the UE is configured to report that suboptimal cases where time passes a threshold which depends on the configured method. In case of CPC configuration, it has to be checked whether the time is shorter than a defined “threshold_min_consecutive_CPC”, and if it is the case, it has to be reported to MRO instance responsible for this cell border where, for instance, a counter is incremented, which expresses that the two consecutive CPC-based cell changes are “tight in succession” indicating that S-CPAC might be preferable.

If UE is however already configured with S-CPAC, the opposite is monitored, namely whether the time is longer than a defined “threshold_max_consecutive_S-CPAC”, and this logged information is to be reported to the MRO instance responsible for this cell border, where a counter is incremented, which expresses that the two consecutive S-CPAC based cell changes are “too widely spaced” and would indicate that S-CPAC might waste network resources and normal CPC would be preferable, since time for preparation is not an issue.

Proposal 3-2: RAN3 shall request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC.

# 3	Conclusions

We have the following observations and proposals to enhance MRO for LTM:

Proposal 1-1: RAN3 discuss new SHR trigger specific to LTM near failure cases related to RACH-based LTM.

Proposal 1-2: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM as new SHR trigger.

Observation 1-1: To optimize LTM candidate cell list both UE and network-based solutions may be used for Rel-18 LTM cell switch.

Proposal 1-3: RAN3 agrees on network-based solution for optimizing the LTM candidate cell list.

Proposal 1-4: RAN3 agrees the attached TP for TS 38.473 to update the Semantics description for the C-RNTI.

- Proposal 1-5: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed and TA received in RAR for recovery/re-establishment cell.

To support MRO for CHO with candidate SCG(s), we propose what follows:

Observation 2-1: The scenario 1a when during the wating for fulfilment of the second CHO execution conditions the T310 on SCG of PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR may require the optimization of the CHO execution conditions.

Observation 2-2: The scenario 1b when the T310 on SCG of PSCell 1 is started before the CPC execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SPR requires the legacy optimization of the CPAC execution conditions.

Observation 2-3: The scenario 1c when during the wating for fulfilment of the CHO execution conditions the T310 on SCG of PSCell 1 is started and T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR and SHR may require the optimization of the CHO execution conditions.

Observation 2-4: The scenario 1d when during the wating for fulfilment of the CHO execution conditions T310 on MCG for PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR requires the optimization of the CHO execution conditions.

Observation 2-5: The scenario 1e when during the wating for fulfilment of the CHO execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Observation 2-6: The scenario 1a when during the wating for fulfilment of the second CPAC execution conditions the T310 on MCG of PCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR may require the optimization of the CPAC execution conditions.

Observation 2-7: The scenario 2b when the T310 on MCG of PCell 1 is started before the CHO execution conditions are fulfilled. with the T310 time value above the configured T310 threshold to trigger the SHR requires the legacy optimization of the CHO execution conditions.

Observation 2-8: The scenario 2c when during the wating for fulfilment of the CPC execution conditions the T310 on MCG of PCell 1 is started and T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SHR and SPR may require the optimization of the CPAC execution conditions.

Observation 2-9: The scenario 2d when during the wating for fulfilment of the CPC execution conditions T310 on SCG for PSCell 1 is started with the T310 time value above the configured T310 threshold to trigger the SPR requires the optimization of the CPAC execution conditions.

Observation 2-10: The scenario 2e when during the wating for fulfilment of the CPAC execution conditions no T310 is started which means that no one of the carriers (MCG or SCG) is temporarily unavailable and thus may not require any optimization.

Proposal 2-1: RAN3 agrees to consider the above scenarios 1a,b,c,d,e and 2a,b,c,d,e as a baseline to study improvement of the UE reports for near failure monitoring for UEs with CHO with Candidate SCG(s) only configured.

Proposal 2-2: In case RLF on MCG is detected before execution of CHO w/ candidate SCG, the RLF report should be enhanced with additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not.

Observation 2-11: In case SCG failure is detected before, during or shortly after the execution of CHO w/ candidate SCGs, the only report generated will be the SCGFailureInformation which only contains PSCell measurements.

Proposal 2-3: In case RLF is detected on the SCG before, during or shortly after successful completion of CHO w/ candidate SCG, the SCGFailureInformation message should be enhanced with the following information:

- An indication of the first fulfilled event/execution condition (CHO or CPAC)

- Additional information on the state of the execution event that was not triggered, e.g. was TTT running for this event or not

- Time duration between the two execution conditions being fulfilled when both have been fulfilled.

In this paper, we also discuss the problem of selecting the most appropriate mobility scheme for a UE: CPAC or S-CPAC. We make following observations:

Observation 2-1: Depending on the cell deployment and UE mobility, usage of either CPAC or S-CPAC may result in more efficient resource utilisation in involved cells.

Observation 2-2: The main criteria to decide between CPAC and S-CPAC configuration are the time between PSCell changes.

Based on those, we propose:

Proposal 2-1: RAN3 agrees on enabling of the optimal usage of S-CPAC as part of SON based optimization

Proposal 2-2: RAN3 shall request RAN2 to enable configuring a time threshold at which the UE is to report to the network either that between two cells time was too short for a classic CPAC or too long for S-CPAC.

Related TP for stage-2 and a draft LS to RAN2 are proposed in the annexes 2 and 3 below.

# References

- Report of 3GPP TSG RAN3 meeting #125bis

- Report of 3GPP TSG RAN3 meeting #125

- Report of 3GPP TSG RAN2 meeting #127bis

- Report of 3GPP TSG RAN2 meeting #127

- RP-223520, RAN #98-e

- R3-241562, RAN3 #123-bis

# Annex 1: Text proposal to TS 38.473 (MRO for LTM)

First change, ommited text not changed

# 9	Elements for F1AP Communication

[snip]

## 9.2	Message Functional Definition and Content

[snip]

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU added in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Remaining text not changed

# Annex 2: Text proposal to TS 37.340 (MRO for S-CPAC)

The below text is proposed to be added to the BL CR to TS 37.340.

First change, ommited text not changed

10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration/S-CPAC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC/S-CPAC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC/S-CPAC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Detection mechanism

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated or SN initiated.

The detailed detection mechanisms for Too Late CPC Execution, Too Early CPC/CPA Execution and CPC/CPA Execution to wrong PSCell are carried out in the NG-RAN node which initiates the CPC/CPA or S-CPAC procedure:

-	Too Late CPC Execution: if CPC/S-CPAC is configured and there is no recent CPC execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt) , and there is a suitable PSCell different from the PSCell where the UE is located at the time of the failure occurrence.

-	Too Early CPC/CPA Execution: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and in case of CPC, the source PSCell is a suitable PSCell, in case of CPA, no suitable PSCell is found.

-	CPC/CPA Execution to wrong PSCell: there is a recent CPC/CPA execution for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the suitable PSCell is not the source PSCell or the target PSCell.

The "UE reported timer" above indicates the time elapsed since the CPC/CPA execution until connection failure.

For CPA or MN initiated CPC or initial S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.

For SN initiated CPC or a following S-CPC initiated by MN, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

Optimisation of S-CPAC

Another function of self-optimisation for S-CPAC is to coordinate the usage of CPAC and S-CPAC. For that purpose, the UE may be configured to provide the needed information:

-	If the UE is configured with the CPC, it may be requested to report when the time between two consecutive CPC executions is lower than 'threshold_min_consecutive_CPC';

-	If the UE is configured with the S-CPAC, it may be requested to report when the time between two consecutive CPC executions is higher than 'threshold_max_consecutive_S-CPAC'.

Next change, ommited text not changed

# Annex 3: Draft LS to RAN2 (MRO for S-CPAC)

3GPP TSG-RAN WG3 #126		R3-24xxxx

Orlando, USA, 18-22 Nov 2024

Title:	[DRAFT] LS on MRO for S-CPAC

Response to:

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:

Contact Person:	Krzysztof Kordybach (krzysztof.kordybach@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	R3-24xxxx

1. Overall Description:

RAN3 would like to inform RAN2 about decisions made in the context of MRO for S-CPAC.

- Regarding handling execution failures during S-CPAC operations, RAN3 assumed the existing framework defined for CPAC will be used.

- RAN3 has also decided to enable a mechanism to support selection of the configuration for the UE: CPAC or S-CPAC. For this purpose, RAN3 agreed a TP, as attached to the LS.

2. Actions:

To RAN2

RAN3 would like to ask RAN2 to enable the needed signalling, as described in the attached TP.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127	17-21 February 2025			Athens, Greece, EU

TSG-RAN3 Meeting #127-bis	7-11 April 2025					China