3GPP TSG-RAN3 Meeting #126	R3-247328

Orlando, US, Nov 18-22, 2024

Agenda Item:	14.3

Source: 	Qualcomm Incorporated, Xiaomi, LGE, NTT Docomo

Title: 	(TP for TS 38.300) NR NTN Regenerative Payload Architecture

Document for:	Discussion/Decision

# Discussion

In RAN3#125-bis meeting, Thales had submitted a good paper R3-245404 on different scenarios to carry NGAP over ISL and XnAP over Feeder link. Similar proposals were also mentioned in other papers – R3-245126 (SS), R3-245085 (Xiaomi)

ISL and Feeder link are NTN transport link and can carry any RAN3 interface protocol without specification impact. Hence specification should not be restrictive to mention that XnAP is carried over ISL and NGAP is carried over Feeder link only.

Inter Satellite link and Feeder link are NTN transport links and can route any RAN3 interface protocol without specification impact.

XnAP can be carried over Feeder link if ISL doesn’t exist between 2 gNBs with regenerative payload and NGAP can be carried over ISL via another gNB to AMF, if gNB doesn’t have Feeder link connection to the NTN GW.

XnAP can be transported via Feeder Link to another gNB with regenerative payload if ISL doesn’t exist between 2 gNBs with regenerative payload and NGAP can be transported via ISL to AMF if gNB doesn’t have Feeder link connection to the NTN GW.

However the baseline architecture in BL CR R3-245761 agreed in RAN3#125bis only mentions Xn over ISL and NG over Feeder link. Also text below the figure in BL CR states the same as highlighted below.

From BL CR R3-245761 to TS 38.300 The NTN transparent payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface (via the service link) and the NG interface (via the feeder link). The following connectivity is supported by the NTN payload: Editor’s Note: The above section needs further checking. - An NTN gateway may serve multiple transparent or regenerative NTN payloads; - A transparent or regenerative NTN payload may be served by multiple NTN gateways; - A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads; - The feeder link is a transport link; it may transport the NG interface between the 5GC and the gNB hosted by the regenerative payload; - The inter-satellite link is a transport link; it may transport the Xn interface between the gNBs hosted by the two regenerative payloads.

From the BL CR R3-245761 to TS 38.300 XnAP is mentioned to be transported only over ISL and NGAP is transported only over Feeder link.

Hence we propose to update the text in BL CR R3-245761 to state that XnAP and NGAP can be carried over ISL or SRI depending on deployments without specification impact.

RAN3 agrees to mention that XnAP and NGAP can be transported over ISL or Feeder link without specification impact and agree to the TP to TS 38.300 below

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Inter Satellite link and Feeder link are NTN transport links and can route any RAN3 interface protocol without specification impact.

Observation 2.	XnAP can be transported via Feeder Link to another gNB with regenerative payload if ISL doesn’t exist between 2 gNBs with regenerative payload and NGAP can be transported via ISL to AMF if gNB doesn’t have Feeder link connection to the NTN GW.

Observation 3.	From the BL CR R3-245761 to TS 38.300 XnAP is mentioned to be transported only over ISL and NGAP is transported only over Feeder link.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 agrees to mention that XnAP and NGAP can be transported over ISL or Feeder link without specification impact and agree to the TP to TS 38.300 below

# TP To TS 38.300

START CHANGE

## 16.14	Non-Terrestrial Networks

### 16.14.1	Overview

Figures 16.14.1-1 and 16.14.1-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 16.14.1-1 the payload is transparent; in Figure 16.14.1-X the payload is regenerative and hosts a gNB.

Figure 16.14.1-1: Overall illustration of an NTN with transparent payload

Figure 16.14.1-X: Overall illustration of an NTN with regenerative payload hosting a gNB

NOTE 1:	Figures 16.14.1-1 and 16.14.1-X illustrate an NTN; RAN4 aspects are out of scope.

The NTN transparent payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface (via the service link) and the NG interface. The following connectivity is supported by the NTN payload:

Editor’s Note: The above section needs further checking.

-	An NTN gateway may serve multiple transparent or regenerative NTN payloads;

-	A transparent or regenerative NTN payload may be served by multiple NTN gateways;

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads;

-	The feeder link is a transport link; it may transport e.g the NG interface between the 5GC and the gNB hosted by the regenerative payload, Xn interface between gNBs hosted by regenerative payload;

-	The inter-satellite link is a transport link; it may transport e.g the Xn interface between the gNBs hosted by the two regenerative payloads, NG interface between 5GC and gNB hosted by regenerative payload.

NOTE 2:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

END CHANGE