3GPP TSG-RAN WG3 Meeting #126															R3-247339

Orlando, USA, 18th – 22nd November 2024

Agenda item:	8.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the LS related to XR

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a LS ([1]) from SA2 on PDU Set Information Marking Support without QoS parameters.

RAN3 also received a LS ([2]) from RAN2 on multi-modality awareness.

This contribution analyses these LS.

# 2	Discussion on SA2 LS PDU Set Information Marking Support without QoS parameters

In the SA2 LS ([1]) on PDU Set Information Marking Support without QoS parameters, the RAN3 related aspects are highlighted.

1. Overall Description: SA2 would like to update RAN 2 and RAN3 on the latest progress of Rel-19 FS_XRM_Ph2: At the CN side, the PDU Set information marking on the GTP-U header of the downlink packets can be supported, even when the PDU Set QoS parameters are absent. Related to this, SA2 has agreed the following: - If the PDU Set QoS parameters are absent, but the PDU Set Information marking is supported, the SMF sends NG-RAN a new PDU Set Information Marking Support Indicator to indicate that DL PDU Set Information Marking is supported. - Based on the received PDU Set Information Marking Support Indicator from the CN, the NG-RAN sends information to the SMF to activate the PDU Set Information Marking for the PDU Set based handling (e.g., PSI based discarding due to congestion). Based on the NG-RAN provided information, the SMF instructs the PSA UPF to perform PDU Set information marking in the GTP-U header. Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters. SA2 would like to understand what information is sent by the NG-RAN to the SMF. Some companies think the NG-RAN can respond with a PDU Set Based Handling Support Indication, to indicate that PDU Set based handling is supported and to activate the PDU Set information marking. Some other companies think the NG-RAN could dynamically request the SMF to activate or deactivate the PDU Set Information marking at any time with an NG-RAN initiated procedure. Q2:SA2 would like RAN3 feedback which approach to support in Rel-19. 2. Actions: RAN2/RAN3: ACTION: SA2 kindly asks RAN2 and RAN3 to reply to the above questions.

In some scenarios (e.g. AF doesn't pay and request PDU Set QoS for a data flow of XR service), the PDU Set QoS parameters are not provided to the NG-RAN node. Nevertheless, the PDU Set Information marking may be still beneficial to the NG-RAN, e.g. the NG-RAN node may perform PSI based discarding in case of congestion. The SA2 LS discusses about the scenario that SMF informs NG-RAN node that PDU Set Information Marking is supported, and the NG-RAN node may inform SMF to request the PDU Set Information marking in the UPF.

In Rel-18,

- From SMF to NG-RAN node, the PDU Set QoS Parameters IE is provided to the NG-RAN node when setup/modify a QoS Flow.

- From NG-RAN node to SMF, current PDU Set based Handling Indicator IE is included in PDU Session Resource Setup Response Transfer IE, PDU Session Resource Modify Response Transfer IE, Path Switch Request Transfer IE, and Handover Request Acknowledge Transfer IE. Based on the received PDU Set based Handling Indicator IE, the SMF can request the UPF to perform PDU Set Information Marking.

For Rel-19, the SA2 LS request following enhancements:

- From SMF to NG-RAN node, a new indication is needed to inform NG-RAN node. This new indicator can be added to NGAP by similar method as the PDU Set QoS Parameters IE, i.e. the new PDU Set Information Marking Support Indicator can be provided to the NG-RAN node when setup/modify a QoS Flow.

- From NG-RAN node to SMF, SA2 LS asked two options on providing the type of information provided from NG-RAN to SMF

- Option 1: reuse the existing PDU Set based Handling Indicator IE and existing procedures

- Option 2: introduce a new indicator in a n NG-RAN initiated procedure, to activate/deactivate the PDU Set Information marking at any time.

RAN3 first need to discuss the necessity of this activation/deactivation at any time. When establishing or modifying a QoS flow, the NG-RAN node knows whether it need to perform PSI-based discarding later. Once this information is provided to the SMF, it is questionable why the NG-RAN node would need to change it at any time. Even if the NG-RAN node changes it, the NG-RAN node may still receive some packets marked (or unmarked) during the transition period. In all cases, the NG-RAN can use or ignore those markings, This raises the question the usefulness of informing the SMF for the change. In addition, it is up to the implementation of the NG-RAN node on how to perform the PSI based discarding. Allowing the activating/deactivation at any time adds unnecessary coordination complexity to the NG-RAN node, SMF and other CN nodes. It also adds more signaling to the N2 and other interfaces (e.g. between AMF and SMF, etc). This is an overkill, without a clear benefit.

Observation 1: There is no clear benefit to allow the NG-RAN node to inform SMF, to activate/deactivate the PDU Set Information marking at any time.

Other than the Path Switch Request procedure, current PDU Set based Handling Indicator IE is included in the response message of Class-1 procedures (i.e. PDU Session Resource Setup procedure, PDU Session Resource Modify procedure, and Handover Resource Allocation procedure). This is enough to inform SMF. There is no need to introduce additional mechanism to inform SMF via an NG-RAN node initiated procedure at any time, as this brings no benefits to NG-RAN. Notice that this mechanism could result to conflicts, as it would introduce NG-RAN as an extra decision makers (on top of SMF) on when PDU Set Information marking is applied.

In addition, Option 2 uses a NG-RAN initiated procedure to inform CN. So Option 2 does not allow to use the response message of AMF initiated procedure, e.g. PDU Session Resource Setup procedure, PDU Session Resource Modify procedure, and Handover Resource Allocation procedure. In case the NG-RAN node needs to inform SMF during the AMF initiated procedure, e.g. PDU Session Resource Setup procedure, the NG-RAN node has to first send the response message to complete the AMF initiated procedure, then initiate a NG-RAN initiated procedure to inform CN. This is very inefficient.

Observation 2: It is enough to use the PDU Session Resource Setup procedure, PDU Session Resource Modify procedure, Handover Resource Allocation procedure, and Path Switch Request procedure to inform SMF about support of the PDU Set Information marking.

The NG-RAN to SMF indicator is to inform SMF that PDU Set Marking is needed. The purpose is same as current PDU Set based Handling Indicator IE. So it is straightforward to reuse current PDU Set based Handling Indicator IE. The Stage-3 TP needs to be updated to specify that the NG-RAN node should send the existing indicator upon reception of the new indicator from the SMF even without QoS Parameters, i.e., in addition to current condition that PDU Set QoS Parameters is received. The Stage-3 TP can be discussed later once TU is assigned.

Based on above analysis, we propose following enhancement to NGAP:

- From SMF to NG-RAN node, introduce a new PDU Set Information Marking Support Indicator when setup/modify a QoS Flow.

- From NG-RAN node to SMF, reuse the existing PDU Set based Handling Indicator IE

Proposal 1: For PDU Set Information Marking Support without QoS parameters, NGAP need following enhancements:

- From SMF to NG-RAN node, introduce a new PDU Set Information Marking Support Indicator when setup/modify a QoS Flow, to indicate that DL PDU Set Information Marking is supported.

- From NG-RAN node to SMF, reuse the existing PDU Set based Handling Indicator IE to activate/deactivate the PDU Set Information Marking

It is proposed to reply SA2 based on above proposal. The draft Reply LS can be found in [3].

# 3	Discussion on RAN2 LS on multi-modality awareness

In the RAN2 LS ([2]) on multi-modality awareness, the RAN3 related aspects are highlighted.

# 1. Overall Description: RAN2 thanks SA2 for the feedback on multi-modality awareness. For the usage of multi-modality information, RAN2 has discussed and reached the following agreements during the previous RAN2 meetings. First, RAN2 agreed that the multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping. - RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation). Furthermore, RAN2 also agreed that traffic synchronization may be achieved by packet level synchronization information, if it can be provided from CN to RAN, which is up to SA2/4 to discuss. - For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. - RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.. # 2. Actions: # To SA2, SA4: ACTION: RAN2 kindly asks SA2, and SA4 to take the above feedback on multi-modality awareness into consideration. # To RAN3, SA4: # ACTION: According to the request from SA2, RAN2 would kindly asks RAN3 and SA4 to review the RAN2 feedback above.

RAN3 discussed the SA2 LS in last meeting, and agreed the reply LS in ([4]). RAN3 agreed that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node and replied accordingly to SA2. RAN3 also agreed to discuss the Stage-3 details, e.g., the NG-RAN node behaviour in a future meeting after TU is assigned. The RAN2 LS clarifies the gNB’s behaviour and should be considered when RAN3 start Stage-3 TP to support multi-modality. At current stage, there is no immediate action required in RAN3.

Proposal 2: Note the RAN2 LS. The gNB behaviour described in RAN2 LS can be considered when RAN3 starts the Stage-3 TP design.

# 4	Conclusion

In this contribution, we have analyzed the LS from SA2. Our proposals are:

Observation 1: There is no clear benefit to allow the NG-RAN node to inform SMF, to activate/deactivate the PDU Set Information marking at any time.

Observation 2: It is enough to use the PDU Session Resource Setup procedure, PDU Session Resource Modify procedure, Handover Resource Allocation procedure, and Path Switch Request procedure to inform SMF, to activate/deactivate the PDU Set Information marking.

Proposal 1: For PDU Set Information Marking Support without QoS parameters, NGAP need following enhancements:

- From SMF to NG-RAN node, introduce a new PDU Set Information Marking Support Indicator when setup/modify a QoS Flow, to indicate that DL PDU Set Information Marking is supported.

- From NG-RAN node to SMF, reuse the existing PDU Set based Handling Indicator IE to activate/deactivate the PDU Set Information Marking

The draft Reply LS can be found in [3].

Proposal 2: Note the RAN2 LS. The gNB behaviour described in RAN2 LS can be considered when RAN3 starts the Stage-3 TP design.

# References

- R3-247113	LS for PDU Set Information Marking Support without QoS parameters	SA2(Vivo)

- R3-247009/R2-2409272, Reply to SA2 LS on multi-modality awareness

- R3-247340	[Draft] Reply LS on PDU Set Information Marking Support without QoS parameters

- R3-245682  Reply LS on multi-modality awareness at RAN