3GPP TSG-RAN WG3 Meeting #126	R3-247366

Orlando, USA, 18 – 22 November, 2024

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for TS 38.300) Access control for NR Femto

Document for:	Discussion

1	Introduction

The previous RAN #105 meeting agreed the new WID for topology enhancement, which includes the objectives of femto part [1]:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

.

In this contribution, we mainly focus on the Access control of NR Femto.

2	Discussion

During the SI phase of the Topology enhancement, it concludes that “For Access control, NR Femto reuses the existing CAG functionalities, no stage3 impact has been identified.” And, in the TR 38.799 [2], how to support the different access modes based on the existing CAG functionality has been clearly captured in clause 5.3.

----------Excerpt from TR 38.799----------

5.3	Access control

- With the existing CAG mechanism, the open, hybrid and closed access mode, can be supported as follows:

-	To support the open access mode: The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	To support the hybrid access mode: The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	To support the closed access mode: The NR Femto node activates an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

As captured in the TR 38.799, the existing CAG functionality can support open, hybrid and closed modes for NR Femto.

The access mode “open”, “hybrid” and “closed” were defined for 4G HeNB based on the CSG mechanism. As analysed during the SI, using 5G CAG functionality can equally support all the three modes as 4G without any stage 3 impact. In 4G HeNB, the “access mode” is defined and indicated in S1 interface, and the HeNB GW or MME (depends on which one the HeNB directly connects) needs to determine and verify the access mode for the HeNB, as excerpted below:

----------Excerpt from TS 36.300----------

- 4.6.2	Functional Split

......

The HeNB GW hosts the following functions:

-	Relaying UE-associated S1 application part messages between the MME serving the UE and the HeNB serving the UE, except the UE CONTEXT RELEASE REQUEST message received from the HeNB with an explicit GW Context Release Indication. In that case, the HeNB GW terminates the S1 UE Context Release Request procedure and releases the UE context if it determines that the UE identified by the received UE S1AP IDs is no longer served by an HeNB attached to it. Otherwise it ignores the message.

-	……

-	In case of S1 INITIAL UE MESSAGE message, S1 PATH SWITCH REQUEST and S1 HANDOVER REQUEST ACKNOWLEDGE message, verifying, as defined in TS 33.320 [53], for a closed HeNB, that the indicated cell access mode and CSG ID are valid for that HeNB.

-	Terminating non-UE associated S1 application part procedures towards the HeNB and towards the MME. In case of S1 SETUP REQUEST message, verifying, as defined in TS 33.320 [53], that the identity used by the HeNB is valid and determining whether the access mode of the HeNB is closed or not. In case of S1 PWS RESTART INDICATION message and PWS FAILURE INDICATION message, verifying, as defined in TS 33.320 [53], that the indicated cell identity is valid and replacing the HeNB ID by the HeNB GW ID before sending the PWS RESTART INDICATION message (respectively the PWS FAILURE INDICATION message) to the MME.

……

In addition to functions specified in clause 4.1, the MME hosts the following functions:

-	Access control for UEs that are members of Closed Subscriber Groups (CSG):

-	In case of handovers to CSG cells, access control is based on the target CSG ID of the selected target PLMN provided to the MME by the serving E-UTRAN (see TS 23.401 [17]).

-	Membership Verification for UEs handing over to hybrid cells:

-	In case of handovers to hybrid cells the MME performs Membership Verification based on UE's selected target PLMN, cell access mode related information and the CSG ID of the target cell provided by the source E-UTRAN in S1 handover, or provided by the target E-UTRAN in X2 handover (see TS 23.401 [17]).

-	Membership Verification for UEs for which the hybrid cell is served by an SeNB is described in clause 4.9.3.3.

-	CSG membership status signalling to the E-UTRAN in case of attachment/handover to hybrid cells and in case of the change of membership status when a UE is served by a CSG cell or a hybrid cell.

-	Supervising the E-UTRAN action after the change in the membership status of a UE.

-	In case of a HeNB directly connected:

-	verifying as defined in TS 33.320 [53], that the identity used by the HeNB is valid when receiving the S1 SETUP REQUEST message and determining whether the access mode of the HeNB is closed or not;

-	verifying as defined in TS 33.320 [53], for a closed HeNB, that the indicated cell access mode and CSG ID are valid when receiving the S1 INITIAL UE MESSAGE message, the S1 PATH SWITCH REQUEST and the S1 HANDOVER REQUEST ACKNOWLEDGE message;

In our understanding, the concept of access mode is introduced in 4G and indicated in S1AP messages, mainly because the MME needs to perform the UE access control based on the membership status and the access mode (if the mode is hybrid or closed) of the UE’s serving cell. For example, to determine whether a non-member UE can access the HeNB cell. If the access mode is “hybrid”, the non-member UE can access this cell. If the access mode is “closed”, the non-member UE cannot access this cell. However, for the CAG based mechanism in 5G, A non-CAG-capable UE (i.e., PLMN UE) will not be able to access an NPN-only cell because it will be barred by the cellReservedForOtherUse in SIB1. There is no need for the AMF to further check the membership and the “cell mode (whether the cell allows PLMN UE access or not)”. The AMF will just perform access control for CAG-capable UE if the UE access the 5GS via a CAG cell. So, we do not see any additional benefit if introducing the 3 analogue access modes for NR femto. The introducing of NR Femto GW does not influence the existing CAG access control on AMF.

The existing CAG functionality works well for NR Femto. Introducing 3 access modes for NR femto do not bring any additional benefit for access control.

Regarding the highlighted sentence in the TS 36.300 description as excerpted above for 4G HeNB, MME and/or the HeNB GW should determine/verify the access mode of the HeNB. Based on our understanding, such determination and verification were introduced from security point of view, which should be out of RAN3’s scope. If RAN3 agrees to not introduce access mode for NR femto, we may further coordinate with other WGs (e.g., SA2 and SA3) about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) can shared by both PLMN and CAG.

It is proposed:

- RAN3 to avoid defining access mode for the NR femto. No access mode indication is needed in UE associated NGAP messages for the access mode determination and verification.

- NR Femto GW is not responsible for CAG access control. The CAG access control is implemented on the AMF.

- RAN3 to further coordinate with SA2 and SA3 about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) can shared by both PLMN and CAG.

It is suggested RAN3 to agree the access control descriptions for TS 38.300, as in the Annex.

- RAN3 to agree the stage 2 TP to capture the access control part for NR Femto.

3	Conclusion

This paper provides analysis for the access control of NR femto, the following observation and proposal are obtained:

- Access control

- As captured in the TR 38.799, the existing CAG functionality can support open, hybrid and closed modes for NR Femto.

The existing CAG functionality works well for NR Femto. Introducing 3 access modes for NR femto do not bring any additional benefit for access control.

- RAN3 to avoid defining access mode for the NR femto. No access mode indication is needed in UE associated NGAP messages for the access mode determination and verification.

- NR Femto GW is not responsible for CAG access control. The CAG access control is implemented on the AMF.

- RAN3 to further coordinate with SA2 and SA3 about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) can shared by both PLMN and CAG.

- RAN3 to agree the stage 2 TP to capture the access control part for NR Femto.

4. Reference

- RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T.

- 3GPP TR 38.799 v 19.0.0, Study on additional topological enhancements for NR.

- TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18).

Annex——TP for TS 38.300

Start of Change

4.X.Y	Access control for NR Femto

- The access control for NR Femto follows the access control for PNI-NPN as introduced in clause 16.7.4. More specifically, there are three options for the setting of the cell served by the NR Femto node:

-	The NR Femto node activates a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto activates a cell shared by both PLMN and CAG, through broadcasting both a list of PLMN identity information and a list of NPN identity information in the SIB1, but without the indication for determination of NPN-only cell. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

-	The NR Femto node activates an NPN-only cell, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

End of Change