3GPP TSG-RAN WG3 Meeting #126	R3-247374

Orlando, USA, 18 - 22 Nov 2024

Title:	Discussion on AI/ML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In RAN3#125-bis meeting, the following agreements and assumption were made with several FFSs.

Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

The Coverage Modification Cause reflects the predicted CCO issue.

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

Information to validate the prediction is needed.

In this document, we continue discussing the AI/ML based Coverage and Capacity Optimization (CCO) and provide our proposals.

2. Discussion

In RAN3#125-bis meeting, the time for predicted CCO issue indicates the point in time when the CCO issue is predicted to happen. However, it does not specify how long the predicted CCO issue will last. In our view, the duration of the predicted CCO issue is also crucial for deciding whether to update CCO configurations. For instance, if the CCO issue duration is short, an update to the CCO configuration may not be necessary.

Therefore, we have the following proposal:

Proposal 1: Introduce the time duration for predicted CCO issues to specify how long the issues are going to last.

If the NG-RAN node is responsible for model training and inference, it may need to collect ground truth data to validate the predictions and optimize the performance of the AI/ML-based CCO model. As shown in Figure 1, after the CCO configuration is updated, NG-RAN node 1 receives feedback from NG-RAN node 2 to validate the predictions and optimize the performance of the AI/ML-based CCO model. Based on TR 38.743 [1], following feedback can be considered to be collected from NG-RAN node 2:

-	Measured radio resource status

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	SON Reports (e.g., RLF, CEF, RA)

Therefore, we have the following proposal:

Proposal 2: Information such as measured radio resource status, UE performance, and SON reports is required to validate predictions and optimize the performance of the AI/ML-based CCO model.

Figure 1. Model Training and Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based coverage and capacity optimization.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based coverage and capacity optimization based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based coverage and capacity optimization optionally, which can also generate predicted results.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based coverage and capacity optimization.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g., predicted CCO issues, future coverage status, etc).

Step 9: NG-RAN node 1 executes coverage and capacity optimization actions according to the model inference output.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have following proposals:

Proposal 1: Introduce the time duration for predicted CCO issues to specify how long the issues are going to last.

Proposal 2: Information such as measured radio resource status, UE performance, and SON reports is required to validate predictions and optimize the performance of the AI/ML-based CCO model.

4. Reference

- TR 38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19)