3GPP TSG-RAN WG3 Meeting #126	   				         R3-247381

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:		16.2

Source:			China Telecom

Title:		A-IoT architecture aspects

Document for:	 	Other

- Introduction

The last RAN3 meeting discussed the RAN architecture of A-IoT, including the definition of the device entities that provide A-IOT services and the system architecture diagram of topology 1&2, corresponding agreements are as follows:

In Topology 1 the signalling transport for XXAP termination at the AIoT RAN node is SCTP/IP. Capture in the TP other transport layer options had been discussed in this study is not pursued.

To only consider no split architecture, and capture this in the TR.

An AIoT enabled gNB could support both topologies, this is an implementation matter.

The RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers, for both T1 and T2.

In T1, an A-IoT RAN node may serve one or multiple readers.

For reader selection:

in T1, reader selection may need the coordination between AIoT RAN and AIoT CN;

in T2, all options are still open, different alternatives are possible as per call flows.

In this contribution, we will continue to discuss RAN architecture aspects from the perspective of RAN3.

- Discussion

Currently, SA2 has completed the interim conclusion on Key Issue #1. Based on the latest progress of SA2 and RAN2, we suggest updating the RAN architecture in TR.

There are many editor’s notes in RAN3 part of the TR:

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g., for topology 1 it may only represent a single interface instance, e.g., a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g., Inventory, Command) e.g., in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

For EN4&6, the functional division of A-IoT RAN and A-IoT CN has been covered in the current discussion and captured in TR, the ENs could be removed.

For EN5&8, not within the scope of discussion for RAN3, the ENs could be removed directly.

For EN7, SA2 agrees that AIOTF can communicates with a BS Reader via a direct interface Nx or via an AMF, and it is not expected a deployment will use both direct communication between a BS Reader and an AIoTF and indirect communication between a BS Reader and an AIoTF via an AMF. Details could be further updated in 6.4.1&6.4.2, if needed. The EN could be removed.

For EN9, the control of the A-IoT radio resources is pending to RAN2, Details could be further updated in 6.5. we prefer to remove this EN.

Proposal 1: Remove the Editor’s Notes 4-9 in section 6.4.

About the architecture for topology 1, we updated the following content in the pCR the last meeting:

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE:	The protocol stack in Figure	 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

Editor’s Note 3: The A-IoT CN may include AMF and A-IoT related functions which is up to SA2 decision.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

The XXAP supports A-IoT services including at least delivery of inventory/command messages.

SA2 agrees that AIOTF can communicates with a BS Reader via a direct interface Nx or via an AMF. If the XX interface is based on NG, NGAP between the A-IoT RAN and the AMF is enhanced to support A-IoT Services.

Proposal 2: The XXAP supports A-IoT services including delivery of inventory/command messages. If the XX interface is based on NG, NGAP will be enhanced to support A-IoT Services.

For Topology 2, there’re still some open issues:

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions, which is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

Editor’s Note 5:	In Topology 2, it is FFS on reader selection, may need coordination between A-IoT RAN

For EN1, the detail protocol stack between A-IoT enabled UE and A-IoT CN could be further updated in 6.4.2.1, the EN could be removed.

For EN2, which RRC based solution to use, i.e. whether A-IoT enabled gNB connect to AIoTF directly or via AMF is still FFS in SA2. Similar to the topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface. The EN could be removed.

For EN3&4, the details are pending to other WGs, nothing to do in RAN3.

For EN5, a new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF manages the device related information and triggers the A-IoT RAN / A-IoT-enabled UE to perform A-IoT service operations towards the A-IoT Devices. Similar to the topology 1, Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

Proposal 3: In Topology 2, Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

Proposal 4: Remove the Editor’s notes for topology 2 architecture.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Remove the Editor’s Notes 4-9 in section 6.4.

Proposal 2: The XXAP supports A-IoT services including delivery of inventory/command messages. If the XX interface is based on NG, NGAP will be enhanced to support A-IoT Services.

Proposal 3: In Topology 2, Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

Proposal 4: Remove the Editor’s notes for topology 2 architecture.

- References

[1] 	3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things, V1.1.0 (2024-10).

[2]	R3-245849, (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR.

- Text Proposal for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g., in TS 22.369 [10] and TR 38.848 [2].

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

A-IoT RAN node function:	A function that contains e.g., the control of the A-IoT radio resources used towards the A-IoT device.

6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE:	The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node. The XXAP supports A-IoT services including delivery of inventory/command messages.

NOTE:	If the XX interface is based on NG, NGAP between the A-IoT RAN and the AMF is enhanced to support A-IoT Services.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter. In this deployment scenario, it is not expected that different instance of XX interface will be supported simultaneously for the two topologies.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>