3GPP TSG RAN WG3#126	R3-247387

Orlando, USA, 18 – 22 Nov. 2024

Agenda item:	17.3

Source: 	China Telecom

Title: 	On support on-demand SIB1

Document for:	Discussion and decision

- Introduction

In RAN#105, the work item on “Enhancements of network energy savings for NR” was revised [1]. The three objectives in the work item were listed below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA;

- Specify support for on-demand SIB1 for UEs in idle/inactive mode;

- Specify adaptation of common signal/channel transmissions.

In RAN3#125bis, some agreements were made as follows:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast. Further check on first activation of UL WUS configuration broadcast in Cell A.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

In this paper, we discuss on the mechanism for UL WUS configuration broadcast and give our observation and proposals.

- Discussion on UL WUS configuration broadcast

2.1 NES cell cascade scenario

The following agreements in RAN2#126 shows that more than one Cell A providing configuration for the same NES cell has been supported.

RAN2#126

Agreements on on-demand SIB1:

Further clarification

Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Figure 1 illustrates three cells provide 5G coverage for a road. When there are no users on this road, the middle cell (Cell B) intends to go to OD-SIB1 mode and become a NES cell. To do so, it will send a UL WUS configuration request to two neighbor cells, namely Cell A#1 and Cell A#2. If either Cell A#1 or Cell A#2 responds with a failure, the NES Cell B should not enter the energy-saving state. Otherwise, it will result in a coverage hole, as UE would be unable to receive the UL WUS configuration from the cell that rejected the request of NES Cell. As a result, the UE would not be able to trigger NES Cell B to send OD-SIB1.

Based on above discussion, we can conclude that the NES Cell can enter the energy-saving state only if each of Cell A units has agreed to provide the UL WUS configuration for it.

Observation 1: The NES Cell can enter the energy-saving state only if each of Cell A units has agreed to provide the UL WUS configuration for it.

Figure 1 Multiple cells providing UL WUS Configuration

Considering this factor, we can review the three options (i.e. Option 1,2,3) of WUS configuration procedure discussed in the last meeting, shown as below:

Option 1: After the procedure of UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB, Cell A could confirm NES Cell gNB with Timer meaning when Cell A has broadcasted UL WUS in its SIB1.

Figure 2 Option 1 procedure of UL WUS Configuration

Option 2: After the procedure of UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of the explicit “UL config provided” message from Cell A gNB.

Figure 3 Option 2 procedure of UL WUS Configuration

Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs until the second message is sent from NES Cell gNB to Cell A gNB to indicate it is time to broadcast UL WUS configuration. Then Cell A broadcasts WUS configuration and NES cell goes to OD-SIB Mode.

Figure 4 Option 3 procedure of UL WUS Configuration

Among the three options above, Option 1 and Option 2 can both work well, the only difference is that in option 2, the NES Cell must wait to receive a message “UL config provided” from Cell A before entering OB-SIB mode. In Option 1, the NES Cell can enter OB-SIB mode independently, based on the confirmation response and Wait Timer received from Cell A.

Compared with Option 2, Option 1 gives the NES Cell control over the OD-SIB1 process which is particularly important in scenarios with multiple Cell A units.

For example, in Fig 1, NES Cell B sends two UL WUS Configuration Provision requests to two different Cell A units. If Option 1 is used, NES cell B receive two UL WUS Configuration Provision Confirm message from two Cell A units. The NES Cell can decide time when to enter OB-SIB mode. Even if two Cell A units send different Wait Timers values in their confirmation messages, the NES cell can select the maximum value of two Timers. After waiting for the maximum timer duration, the NES Cell will enter OB-SIB mode. In this procedure, the NES Cell retains control over when it enters the energy- saving state.

In Option 2, the NES Cell does not have control, even if it receives two UL WUS Configuration Provision Confirm messages from two Cell A unit, it must wait for a third message (i.e. UL config provided) from two Cell A. This mechanism seems inefficient.

For this reason, we prefer Option 1 over Option 2.

Proposal 1: Use Option 1 as the procedure for UL WUS Configuration.

In Option 3, Cell A must wait for an indication from the NES Cell which remove Cell A’s control over when to broadcast the WUL configuration broadcast. However, it is clear the Cell A is better equipped to determine the optional time for broadcasting the WUL configuration than the NES Cell. Therefore, the NES Cell should not dictate when Cell A broadcasts the WUL configuration.

If a NES Cell need to inform the Cell A of the time when it is ready to transition to OD-SIB1 mode, it can communicate this by sending a UL WUS Configuration Provision request message. This message indicates to Cell A that the NES Cell is ready to enter OD-SIB1 message as soon as it received the request response.

Based on this, we consider that Options 3 mechanism to be unnecessary due to its lack of efficiency.

Proposal 2: Option 3 mechanism for UL WUS Configuration is unnecessary due to inefficiency.

- Conclusion

Based on the above discussion regarding support for on-demand SIB1 operation, we have the following observations and proposals:

Observation 1: The NES Cell can enter the energy-saving state only if each of Cell A units has agreed to provide the UL WUS configuration for it.

Proposal 1: Use Option 1 as the procedure for UL WUS Configuration.

Proposal 2: Option 3 mechanism for UL WUS Configuration is unnecessary due to inefficiency.

- Reference

[1] RP-242354, New WID: Enhancements of network energy savings for NR, Ericsson.