3GPP TSG RAN WG3#126	R3-247388

Orlando, USA, 18 – 22 Nov. 2024

Agenda item:	12.3

Source: 	China Telecom

Title: 	On NR Femto architecture and functional split

Document for:	Discussion and decision

- Introduction

In RAN#105, the work on “Additional topological enhancements for NR” goes on the work item phase from the study item [1]. The Femto related objectives in the new work item were listed below:

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In RAN3#125bis, some agreements were made and captured in R3-245754, but there are still some issues need to be further discussed, such as:

- Protocol Stack for NG User Plane to be continued.

- Add a note for Figure 4.X.3.2-1, stating that it is FFS whether the figure is needed.

- Discussion on Figure 4.X.3.2-1 to be continued…

- WA: an NR Femto may have more than one cell.

In this paper, we discuss issues related to NR Femto architecture and present our proposals.

- Discussion on NR Femto architecture

2.1 Architecture of Femto GW for NG interface

In last meeting, the following issues on Figure 4.X.3.2-1 was left.

- Add a note for Figure 4.X.3.2-1, stating that it is FFS whether the figure is needed.

- Discussion on Figure 4.X.3.2-1 to be continued…

In R3-245754, Figure 4.X.3.2-1 presents the protocol stack for control plane for NG Interface from NR Femto to AMF without the NR Femto GW. There is no similar figure in TS 38.300 to illustrate this content. We can retain the figure, just it is done in TS 36.300.

Proposal 1: Figure 4.X.3.2-1 in R3-245754 should be kept in TS 38.300.

- Discussion on Protocol Stack for NG User Plane

The NG-U data plane is defined between the NR Femto, NR Femto GW, and the UPF. For the control plane protocol stack of the NG Interface from NR Femto to AMF, we can reuse Figure 4.6.3.1-1 and Figure 4.6.3.1-2 from TS 36.300 [5] as a similar reference. The two figures below illustrate the NG-U protocol stack with and without the NR Femto GW.

Figure 1: User plane for NG-U interface for NR Femto without NR Femto GW

Figure 2: User plane for NG-U interface for NR Femto with NR Femto GW

The NR Femto GW may optionally terminate the user plane both towards the NR Femto and towards the UPF, and relay user plane data between the NR Femto and the UPF.

Proposal 2: Figures 1 and 2 should be included in TS 38.300 to illustrate the user plane protocol stack for the NG Interface from NR Femto to AMF without the NR Femto GW.

2.2 Functional split of NR Femto GW

In the NR architecture with deployed NR Femto GW, while an NR Femto hosts the same functions as a gNB, it should also additional capability specifics in case of its connection to the NR Femto GW, such as the following:

- A NR Femto should discover a suitable serving NR Femto GW;

- A NR Femto shall connect to only one single NR Femto GW at one time;

- The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW.

For NR Femto GW, referring to the functionality of the HeNB GW, the functionality of it should be as follows at least:

- A NR Femto GW can relay UE-associated NG application part messages between the AMF serving the UE and the NR Femto serving the UE;

- Terminating non-UE associated NR application part procedures towards both the NR Femto and the AMF;

- Optionally terminating NR-U interface with the NR Femto and with the UPF;

- Supporting TAC and PLMN ID used by the NR Femto.

- Ensuring that Xn interfaces shall not be established between the NR Femto GW and other nodes.

Proposal 3: The basic functions outlined above for NR Femto and NR Femto GW should be included in TS 38.300.

- Conclusion

Based on the above discussion regarding architecture and functional split for NR Femtos, we provide the following proposals:

Proposal 1: Figure 4.X.3.2-1 in R3-245754 should be kept in TS 38.300.

Proposal 2: Figures 1 and 2 should be included in TS 38.300 to illustrate the user plane protocol stack for the NG Interface from NR Femto to AMF without the NR Femto GW.

Proposal 3: The basic functions outlined above for NR Femto and NR Femto GW should be included in TS 38.300.

- Reference

[1] RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

[2] RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.

[3] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2 Rel-18, 3GPP.

[4] TS 38.413, NG-RAN; NG Application Protocol (NGAP) Rel-18, 3GPP.

[5] TS 36.300, E-UTRAN; Overall description; Stage 2 Rel-18, 3GPP.

[6] TS 36.413, E-UTRAN; S1 Application Protocol (S1AP) Rel-18, 3GPP.