3GPP TSG-RAN WG3 Meeting #126	R3-247408

Orlando, US, 18 - 22 Nov, 2024

Title: 	UL RRC message segmentation for UECapabilityInformation

Source: 	Huawei

Agenda item:	8.1

Document Type:	 Discussion

# 1. Introduction

The LS on the UL RRC message segmentation for UECapabilityInformation was received from the previous RAN2-127bis meeting, with the following contents.

1. Overall Description: RAN2 standard currently supports UL RRC message segmentation for transferring UECapabilityInformation message. Each segment can be as large as the maximum PDCP SDU size and the RRC signalling supports up to 16 segments. RAN2 however came to the understanding that the existing network implementations do not always support the maximum number of RRC segments. RAN2 therefore agreed to introduce a new RRC signalling solution where the gNB indicates the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message. The UE then is required to generate UECapabilityInformation ensuring the size of the message does not exceed the maximum allowed size according to the gNB indication. It was pointed out in the RAN2 discussion that the network limitation mentioned above can reside in RAN or CN (given that the reported UE capability needs to be stored in RAN and CN). In the case that it is CN’s limitation, RAN2 believes that signalling from CN to gNB is necessary so that gNB can indicate the correct value in UECapabilityEnquiry message. RAN2 therefore would like to ask RAN3 to define a signalling solution where CN informs RAN about the maximum UE capability size that CN can accommodate. 2. Actions: To RAN3 group. ACTION: RAN2 asks RAN3 to take the above information into account and to define a signalling solution where CN informs RAN about the maximum UE capability size that CN can accommodate.

This contribution provides our views on the RAN3 action discussed in the incoming LS.

# 2. Discussion

## 2.1 Background of maximum number of UL segments

The whole discussion in RAN2 was mainly based on [2], and concluded with the following agreements.

Agreements 1) Agree on the signalling solution for or RRC message segmentation of UECapabilityInformation where: - The UE indicates its support for network-requested maximum number of UL segments. - The network indicates the maximum number of UL segments the UE is allowed to use. - The UE generates UECapabilityInformation ensuring the total size of the message does not exceed the maximum allowed size according to the maximum number of UL segments the UE is allowed to use. 2) The proposed new signalling solution is introduced from release-17. 3) The RRC processing delay requirement for UE capability enquiry / transfer procedure when the network indicates the maximum allowed number of UL segments is TBD ms. 4) We will send an LS to RAN3 saying that we plan to specify this feature and that we have a need for CN to indicate the support to the RAN.

From the incoming LS and [2] above, it can be observed that there are some network implementations which are not always allow the maximum number of RRC segments (i.e., up to 12 segments). It may happen that the network would not be able to acquire the full UE radio capability, and lead to failure cases.

The solution agreed in RAN2-127bis is that the gNB could indicate the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message, upon receiving the capability indicator (e.g., ul-RRC-MaxCapaSegments-r17) from the UE in the RRC Setup Complete message. Then the UE will segment the RRC UECapabilityInformation accordingly, and send to the gNB.

Observation 1: The gNB can indicate the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message, upon receiving the capability indicator in the RRC Setup Complete message.

## 2.2 Potential RAN3 impacts and solutions

- From the incoming LS in [1], RAN2 indicates that the network limitation may also happen in the CN side, in addition to the RAN. The rationale behind is that the CN would store the UE radio capability, and signal to the NG-RAN each time the UE initially accesses if the UE radio capability is available. And RAN2 suggests RAN3 to define a signalling solution where CN informs RAN about the maximum UE radio capability size that CN can accommodate. With this information, the NG-RAN can decide the proper maximum number of UL segments value to the UE under both the CN and gNB limitation.

- We consider there are several options for the RAN to be aware of the CN storage limitations as follows.

- Option 1: OAM configuration

- Option 2: Signaling based

  - Option 2a: AMF node level message

  - Option 2b: UE level message

- For option 1, it is simple and without any signaling impact, especially considering the static storage size limitation in CN. On the other hand, this option is lack of flexibility and is difficult for multiple vendors scenarios.

- For option 2a, the CN can indicate its allowed maximum radio capability size for a single UE in the NG setup response and the AMF configuration update acknowledge message. While for option 2b, it can be included in the Initial Context Setup Request (ICSR) message, when the UE radio Capability IE is not included.

- When comparing the above options, option 1 is always possible. On top of that, option 2a can provide more flexibility while option 2b is not necessary in a UE level message.  Hence option 2a can be considered as the signaling based solution. If RAN3 made agreements on option 2a, then the LS to SA2 is required, so that SA2 can check if any specification change is needed.

RAN3 to discuss the following options to indicate to the gNB the CN limitation of UE radio capability size:

- Option 1: OAM configuration

- Option 2: Signaling based

  - Option 2a: AMF node level message

  - Option 2b: UE level message

RAN3 to consider option 1 is the basis. In order to allow flexibility, option 2a (signalling based – AMF node level message) can also be considered, i.e., including the allowed UE radio capability size in the NG setup response and the AMF configuration update message.

RAN3 to send LS to SA2 so that SA2 can check if any specification change is needed.

- In addition to UE capability delivery from the UE to the gNB procedures, the gNB also includes UE radio capability in Handover Required message for NG based handover. It should be noted that the UE radio capability in Handover Required message should also consider the maximum radio capability size limitation indicated by CN. This implies that if gNB receives UE capabilities which is larger than size limitation (e.g., from the previous Xn based handover from a legacy gNB), the gNB can re-enquiry UE radio capability from UE based on the limitation indicated by the CN, before perform initiating NG based handover.

For NG based HO, the UE radio capability included in the Handover Required message should also consider the maximum radio capability size limitation indicated by CN.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Observation 1: The gNB can indicate the maximum number of UL segments the UE is allowed to use, in UECapabilityEnquiry message, upon receiving the capability indicator in the RRC Setup Complete message.

- RAN3 to discuss the following options to indicate to the gNB the CN limitation of UE radio capability size:

- Option 1: OAM configuration

- Option 2: Signaling based

  - Option 2a: AMF node level message

  - Option 2b: UE level message

RAN3 to consider option 1 is the basis. In order to allow flexibility, option 2a (signalling based – AMF node level message) can also be considered, i.e., including the allowed UE radio capability size in the NG setup response and the AMF configuration update message.

RAN3 to send LS to SA2 so that SA2 can check if any specification change is needed.

For NG based HO, the UE radio capability included in the Handover Required message should also consider the maximum radio capability size limitation indicated by CN.

- The reply LS and the corresponding CRs can be found in [3-5].

# 4. Reference

- R2-2409313 LS on UL RRC message segmentation for UECapabilityInformation, RAN2

- R2-2408371	Network signalling of maximum number of UL segments	Qualcomm Incorporated, AT&T

- R3-247409	Introduction of Allowed UE Radio Capability Size for UECapabilityInformation [UL_Capa_Segmentation]		Huawei

- R3-247410	Introduction of Allowed UE Radio Capability Size for UECapabilityInformation [UL_Capa_Segmentation]		Huawei

- R3-247411	[draft] Reply LS on UL RRC message segmentation for UECapabilityInformation	Huawei