3GPP TSG-RAN WG3 Meeting #126	R3-247412

Orlando, US, 18th – 22nd November, 2024

Title:	On Rel-18 UE performance feedback for AI/ML for NG-RAN

Agenda Item:	9.2

Source:	Huawei, Lenovo, Deutsche Telekom, Telecom Italia, BT, 	InterDigital, Nokia

Document for:	Discussion

# 1	Introduction

Since RAN3#124 meeting (May 2024), RAN3 is discussing how to properly derive UE-level UE performance metrics introduced as part of the Rel-18 WI “Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1] and a clear conclusion is still missing.

In this contribution we provide a way forward for the per-UE UE performance metrics definition.

# 2	Discussion

In the context of the Rel-18 “AI/ML for NG-RAN” RAN3 introduced in TS 38.423 [2] a set of metrics representing the UE performance that could be requested by the source gNB to be reported by the target gNB via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision, i.e., handover. Such UE performance could be used as feedback information to the source gNB for further optimization of the AI/ML model which recommended the previous UE’s mobility action from source gNB to target gNB.

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11. **** Skip non-relevant text ****

The above metrics, i.e., Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss DL, are intended to be collected by the target gNB within a short time window starting immediately after the handover and on a per-UE basis. However, there is no clear understanding on how a gNB could perform such per-UE measurements because the corresponding measurement definitions in relevant specifications, e.g., TS 28.558 and TS 37.320, are specified for a different granularity, i.e., not per-UE.

Observation 1: UE performance metrics in TS 38.423 are collected on a per-UE basis. However, there is no clear understanding in RAN3 on how a gNB could derive such per-UE measurements.

In RAN3#124 meeting (May 2024) the contribution in [3] made an attempt to address the issue of how the gNB could derive the UE-level UE performance metrics introduced by RAN3 and, among the proposed options, it was captured in RAN3#124 meeting minutes the common understanding that, for Rel-18, UE level performance is derived as the average value of per-DRB level measurements of the UE performance metrics [4].

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

In RAN3#125 meeting (August 2024) the CR to TS 38.423 in [5] was submitted to add a NOTE in TS 38.423 clause 9.2.3.179 clarifying that the value of each IE of the UE Performance results from averaging the corresponding DRB level measurements over all configured DRBs in the UE, which is in line with the common understanding in RAN3 as captured in [4]. However, the CR was not pursued due to the concerns of some companies that such a NOTE may create issues when dealing with the Rel-19 normative phase discussions related to the finer granularity of UE performance metrics for the AI/ML-based Network Slicing use case.

During the RAN3#125bis pre-meeting offline discussion among RAN3 companies involved in this UE performance granularity discussion, a revised version of the CR to TS 38.423 was proposed where it is clarified that the NOTE above only applies to UE-level UE performance metrics. However, it was argued that TS 28.558 [6], clause 6.3.1.4.1 (6.3.1.4.2), already refers to the Average DL UE Throughput in gNB (Average UL UE Throughput in gNB) as a per-UE measurement:

##### 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. b) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for option 3) and for each supported S-NSSAI. If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s] If ThpTimeDl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise ThpTimeDl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE.. The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer).. d) Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs. e) The measurement name has the form DRB.UEThpDl, or optionally DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI. **** Skip non-relevant text **** **** Skip non-relevant text ****

Therefore, based on the above, it seems that the NOTE reflecting the RAN3 common understanding on how to derive UE-level UE performance metrics applies to Average Packet Delay and Average Packet Loss DL only.

Observation 2: By following the definitions of Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2, it can be assumed that these measurements are already specified as per-UE. Therefore, the RAN3 common understanding on how to derive UE-level UE performance metrics (as captured in RAN3#124 meeting minutes) should apply to Average Packet Delay and Average Packet Loss DL only.

Even though it seems that SA5 specified the Average UE Throughput DL/UL in TS 28.558 as a per-UE measurement, the way such measurement is derived from corresponding per-DRB measurements is different from the RAN3 common understanding as captured in RAN3#124 meeting. The following numerical example should provide clarifications according to the above statement.

Let’s consider the case of two DRBs being configured for a certain UE:

- for DRB#1 the data volume is DV1=5Mbit and the time interval to deliver such data volume it ∆T1=500ms; this implies that the throughout for this DRB#1 is TputDRB#1=DV1∆T1=5MBit500ms=10Mbps;

- for DRB#2 the data volume is DV2=10Mbit and the time interval to deliver such data volume it ∆T2=200ms; this implies that the throughout for this DRB#2 is TputDRB#2=DV2∆T2=10MBit200ms=50Mbps.

By applying the formula as in clause 6.3.1.4.1 of TS 28.558 to calculate the Average DL UE Throughput in gNB the following results:

AvgTputDLUE-SA5=DV1+DV2∆T1+∆T2=5+10Mbit500+200ms≅21.43Mbps

Note that the above calculation assumes that there is neither a time overlap between the transmissions over the two DRBs configured to the UE nor a time gap between such transmissions. In our understanding, this means that the transmission over the second DRB is assumed to start just after the transmission over the first one ends.

But if we compute the per-UE Average UE Throughput DL as per the RAN3 common understanding, i.e., by averaging per-DRB average throughput, the Average UE Throughput DL will be:

AvgTputDLUE-RAN3=TputDRB#1+TputDRB#22=10+502Mbps=30Mbps

The above simple numerical example shows that even though we are referring to the same measurement, i.e., Average UE Throughput DL, the way such measurement is derived could lead to different values if there is no clear understanding of which approach needs to be followed.

Observation 3: Even though it seems that SA5 specified the Average UE Throughput DL/UL in TS 28.558 as a per-UE measurement, the way such measurement is derived from corresponding per-DRB parameters (UE data volume and delivering time interval) differs from the RAN3 common understanding of averaging per-DRB UE throughout DL.

From the excerpt of TS 28.558 clause 6.3.1.4.1 reported above, it can also be noticed that field e) – “Measurement Type” as described in Annex A of TS 28.558 – indicates DRB.UEThpDl (and optionally DRB.UEThpDl.QoS as well as DRB.UEThpDl.SNSSAI) as “the measurement name used to identify the measurement type in the result files”. In our understanding such name indicates a per-DRB measurement of the UE throughput measured in the downlink direction and, according to the analysis made so far, this seems to us a misalignment between the name of the measurement (i.e., DRB.UEThpDl, corresponding to a per-DRB UE throughput measurement) and the actual measurement result that is provided by applying the formula in clause 6.3.1.4.1 (i.e., the per-UE average UE throughput in DL).

Observation 4: There seems to be a misalignment between the name of the measurement in TS 28.558 clause 6.3.1.4.1 (i.e., DRB.UEThpDl, corresponding to a per-DRB UE throughput measurement) and the actual measurement result that is derived by applying the formula in clause 6.3.1.4.1 (i.e., the per-UE average UE throughput).

Besides the name misalignment as per Observation 4, we think that it would be better to refer to what SA5 specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL, respectively. However, it should be clarified by SA5 under which conditions the calculation of the Average UE Throughput DL and Average UE Throughput UL in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 is performed, such as absence of any time overlap between the transmission over the concerned DRBs configured to the UE or of any time gap between such transmissions. Moreover, from a pure technical perspective, it should be recalled that, in order to monitor the per-UE UE performance as a consequence of an AI/ML-driven handover, it could be needed to collect per-UE UE performance metrics before the handover (hence by the source node) which will then be compared to the same metrics but collected after the handover (hence by the target node): if per-UE metrics are collected as per the RAN3 common understanding, there could be some impairments in the measurement process due to the possible DRB re-mapping at the target node side.

Observation 5: Formulas specified by SA5 in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL could be used for deriving UE throughput metrics provided that SA5 clarifies under which conditions the calculation is performed.

Therefore, we propose the following:

Proposal 1: RAN3 to agree to ask SA5 to clarify the conditions under which the calculation of Average UE Throughput DL and Average UE Throughput UL as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2, respectively, is performed.

It should be discussed how to avoid any misunderstanding in the derivation of the remaining per-UE UE performance metrics, i.e., Average Packet Delay and Average Packet Loss DL. Currently such measurements are defined in TS 28.558 on a per-DRB granularity, meaning that if corresponding per-UE measurements are needed the only way to derive them is by following the RAN3 common understanding as captured in RAN3#124 meeting.

Observation 6: Average Packet Delay and Average Packet Loss DL are defined in TS 28.558 on a per-DRB granularity only. The corresponding per-UE measurements seem to lack a guidance on how to derive them as for the Average UE Throughput DL/UL.

The risk that we see in adopting the RAN3 common understanding for deriving per-UE metrics from corresponding per-DRB measurements – which should be applicable only for Average Packet Delay and Average Packet Loss DL – is that we would still refer in general to per-UE UE performance which however is computed by following two distinct “averaging” approaches while only one of these is standardized (in TS 28.558) and applicable only for the average UE throughput. In other words:

- for Average UE Throughput DL/UL: these metrics could be calculated as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2, and we could label these metrics as per-UE “average” metrics;

- for Average Packet Delay and Average Packet Loss DL: these metrics can only be calculated by averaging the corresponding per-DRB measurements (i.e., as per the RAN3 common understanding), and we will also label these metrics as per-UE “average” metrics even though the averaging method is different from the standardized one suggested for the UE throughput.

The above situation should be avoided so that the risk of misinterpreting the per-UE UE performance metrics results is minimized. In our view this can only be achieved if some guidance is made available also for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics, and we think that this should be in the scope of SA5 rather than RAN3.

Observation 7: In order to avoid misinterpretation of the per-UE UE performance metrics, some guidance needs to be made available from SA5 also for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics as already done for the per-UE Average UE Throughput DL/UL.

Based on the above observations we propose to liaise SA5 to ask for feedback on whether some guidance for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics could be provided by SA5 as already done for the per-UE Average UE Throughput. Clarifications on the contents of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 for deriving per-UE Average UE Throughput DL and Average UE Throughput UL could also be asked to SA5 as well. Hence, we propose:

Proposal 2: RAN3 to send a LS to SA5 to ask whether SA5 can provide guidance for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics as already done for the per-UE Average UE Throughput.

A draft LSout to SA5 can be found in [7].

# 3	Conclusion

This document proposes the following observation and proposals:

Observation 1: UE performance metrics in TS 38.423 are collected on a per-UE basis. However, there is no clear understanding in RAN3 on how a gNB could derive such per-UE measurements.

Observation 2: By following the definitions of Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2, it can be assumed that these measurements are already specified as per-UE. Therefore, the RAN3 common understanding on how to derive UE-level UE performance metrics (as captured in RAN3#124 meeting minutes) should apply to Average Packet Delay and Average Packet Loss DL only.

Observation 3: Even though it seems that SA5 specified the Average UE Throughput DL/UL in TS 28.558 as a per-UE measurement, the way such measurement is derived from corresponding per-DRB parameters (UE data volume and delivering time interval) differs from the RAN3 common understanding of averaging per-DRB UE throughout DL.

Observation 4: There seems to be a misalignment between the name of the measurement in TS 28.558 clause 6.3.1.4.1 (i.e., DRB.UEThpDl, corresponding to a per-DRB UE throughput measurement) and the actual measurement result that is derived by applying the formula in clause 6.3.1.4.1 (i.e., the per-UE average UE throughput).

Observation 5: Formulas specified by SA5 in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 for the Average UE Throughput DL and Average UE Throughput UL could be used for deriving UE throughput metrics provided that SA5 clarifies under which conditions the calculation is performed.

Observation 6: Average Packet Delay and Average Packet Loss DL are defined in TS 28.558 on a per-DRB granularity only. The corresponding per-UE measurements seem to lack a guidance on how to derive them as for the Average UE Throughput DL/UL.

Observation 7: In order to avoid misinterpretation of the per-UE UE performance metrics, some guidance needs to be made available from SA5 also for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics as already done for the per-UE Average UE Throughput DL/UL.

Proposal 1: RAN3 to agree to ask SA5 to clarify the conditions under which the calculation of Average UE Throughput DL and Average UE Throughput UL as specified in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2, respectively, is performed.

Proposal 2: RAN3 to send a LS to SA5 to ask whether SA5 can provide guidance for deriving per-UE Average Packet Delay and Average Packet Loss DL metrics as already done for the per-UE Average UE Throughput.

A draft LSout to SA5 can be found in [7].

# 4	References

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), Rel-18

- R3-243578, Discussion on definition of UE performance metrics, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- R3-244503, Clarification of UE performance, CR to TS 38.423, CATT, Huawei, Nokia, Qualcomm, Deutsche Telekom, NTT Docomo, Lenovo, NEC, China Telecom, China Unicom, Ericsson, ZTE, Samsung, InterDigital

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-18, v18.0.0

- R3-247413, [Draft] LS on per-UE UE performance metrics, Huawei, Lenovo, Deutsche Telekom, Telecom Italia, BT, InterDigital, Nokia