3GPP TSG-RAN WG3 Meeting #126	R3-247423

Orlando, USA, 18-22 Oct 2024

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Positioning (Case 3a&3b)

Document for:	Discussions & Approval

1	Introduction

The objectives of AI/ML for air interface WI [1] are listed as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

  - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Based on the current discussions in RAN1 and RAN2, the topic that requires normative work in RAN3 is AI/ML-based positioning. Further analysis indicates that Case 3a and Case 3b may have impacts on RAN3.

In the last meeting, following is initial RAN3 progress:

The existing LCS framework is re-used for AI/ML positioning.

Case 3a agreements:

- LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

- How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

- NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

- WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For case 3b:

- NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we provide our understanding on the case 3a & 3b about assisted AI/ML positioning.

2	Discussion

2.1 Case 3a

2.2.1 Intermediate positioning measurement

Following is the agreement in RAN1 regarding the intermediate positioning measurement from NG-RAN node to LMF:

Agreement

For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting.

- If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214.

- If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215.

- Note: details of the report are pending further discussion.

LOS/NLOS indicator and timing information, generated by AI/ML model was agreed to report to LMF from NG-RAN node. Checking the current TS 38.455 [3], the existing LOS/NLOS indicator and timing information (e.g., UL RTOA or gNB Rx-Tx time difference) is already supported.

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

- The intermediate positioning measurements reported from the NG-RAN node to the LMF, such as LoS/NLoS Information, UL RTOA, and gNB Rx-Tx Time Difference, are already supported in the current specification. There is no need to introduce additional new IEs for them.

Then, one question may be raised whether to support AI/ML report includes an indicator that indicates the measurement information is derived using AI/ML model or not.

- RAN3 needs to discuss whether to add an indicator that indicates that the measurement information is derived using AI/ML model or not.

This issue are still under discussion in RAN1. From our perspective, the indicator is not needed, the reasons are as follows.

- The mentioned indicator will not affect LMF’s calculation. For example, in a general TDOA/RTT positioning procedure, LMF will collect the RSTD/RTT measurement information of different TRP pairs, and calculate UE’s location information based on TRPs’ location information and the reported RSTD/RTT. LMF doesn’t care the RSTD/RTT measurement which is derived using AI/ML model or legacy measurement.

- If the LOS/NLOS indicator is associated with the AI/ML model output, i.e., virtual LOS path, the reported LOS/NLOS indicator should always be equal to 1. It can be inferred that the timing information reported with full LOS indications is likely to be inferenced by AI/ML model.

- For reporting model output of AI/ML assisted positioning case 3a, an indicator identifying whether the measurement is based on AI/ML model is not needed.

2.2.4 Model Training

If data collection and generation procedures are executed at the same entity, there is no additional specification impact concerning data collection. Otherwise, if data collection and generation procedures are executed at the different entities, the following procedure should be considered. The general procedure of data generation and collection (data collection/generation node can be different entities in different use cases) can be listed as follows (refer to Figure 1):

| - Step1: Data collection node indicates the requirement for data collection - Step2 (UE/PRU/gNB’s behavior): Data generation node generates data based on the configured requirement - Step3: Data generation node reports the collected data | Figure 1 Data generation and collection procedure |
| --- | --- |

It is feasible to let the data collection node configure the data collection requirement for AI/ML positioning model considering the positioning accuracy requirement. More specifically, AI/ML models with higher QoS requirements can collect data with higher quality requirements during the data collection procedure, where the requirements may include quality requirement of the channel measurement, report periodicity, etc. And the data collection/generation entity can be different among use cases.

For case 3a model training, LMF need to provide ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. Therefore it is proposed to introduce new data collection initiation and reporting procedure over NRPPa, as similar mechanisms as data collection procedure over XnAP.

- Introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP.

2.2.3 Model performance evaluation

In the last meeting, RAN3 has achieved a working assumption that RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

And following is the RAN1 agreement on this issue:

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

There are four different ways for model performance evaluation：

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

For model performance evaluation, RAN3 discussed this topic during the R17 and R18 stages and concluded that model performance is out of RAN3's scope. This wording to reflect this into stage2 specification is still under discussion regarding whether it needs further revision, but the principle is to avoid transmitting model performance metrics over NG-RAN interfaces (e.g., loss, accuracy, etc.).

- Option 1 (gNB performs model monitoring metric calculation and makes the monitoring decision), also referred to as Option A, is the appropriate approach for model performance evaluation in Case 3a. And revert the WA as an agreement.

RAN1 requests confirmation from RAN3 regarding the performance monitoring metric calculation.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

2.2.4 Association between part A and part B

- In RAN1#117 meeting, the association between part A and part B was discussed, and the following agreement was achieved:

Agreement in RAN1#117: For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed

- In this subsection, we provide our views on the association between part A and part B.

- From training data generation entities’ perspective, the reported data can be Part A + Part B, Part A only, and Part B only. And in previous several meetings, the entities for graining data generation were discussed, where part A and/or part B can be obtained from the same/different entities for different use cases. The specific model training procedure is up to training entities’ implementation, i.e., training entity determines whether the model training is performed using part A + part B or only part A. For example, in case 3b (LMF side model), part A can be provided by a TRP (e.g., using TRP measurement result in TS 37.355), whereas part B can be provided by a PRU (e.g., using ProvideLocationInformation in TS 37.355). After collecting the training data, LMF needs to pair or associate part A with part B. However, how to pair and how to train the model depends on LMF’s implementation. LMF may pair part A and part B based on the associated timestamp, or the ID of PRU as well as the TRP’s measurement report. Among them, the time stamp for TRP channel measurement information are agreed to reuse the current Time Stamp IE in TS 38.455. The time stamp for UE channel measurement report can be discussed later as UE’s measurement reports are related to 2nd priority use cases, i.e., case 2a and case 2b.

- It can be observed that most of the training data can be provided by the current structure except Part B for case 3a. In this case, LMF should provide assistance information including ground truth label as well as other association information to gNB. Based on the above analysis, we have the following proposals:

- The association between part A and part B can be determined by the model training entity based on the collected measurement information and label information.

- Part A can be provided by:

  - TRP measurement result in TS 38.455 (case 3a, case 3b).

- Part B can be provided by:

  - ProvideLocationInformation in TS 37.355 (case 3b).

  - New assistance information sent from LMF to gNB (case 3a).

- Part A and Part B can be associated using:

  - the timestamp provided in measurement and/or location information.

  - UE ID associated with the measurement and/or location information.

2.2 Case 3b

RAN3 needs to specify new measurements, which are not supported in the current specification. For the new measurement, following is the corresponding agreement from RAN1:

Agreement

- For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting:

- timing information;

- paired timing information and power information.

Agreement

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives:

      - Alternative (a).  Sample-based measurements, where the timing information is an integer multiple of sampling periods.

      - Alternative (b).  Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

Agreement

For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time.

- FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time

Agreement

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

It can be seen that time domain channel measurements are considered as input information reported from the NG-RAN node to the LMF. The existing Measurement Report message over NRPPa and F1AP can be reused.

However, RAN1 still needs more time to finalize whether sample-based or path-based measurements will be used. Currently, only a more general reporting method has been agreed upon: for timing, the UL discussion is to reuse the existing RTOA time, and for power, RSRPP is used as the starting point.

- In Case 3b, time domain channel measurements are considered as input information reported from the NG-RAN node to the LMF. The details of time domain channel measurements are still pending and subject to further discussion in RAN1.

- The existing Measurement Report message over NRPPa and F1AP can be reused for input information transferring.

3	Conclusion

Following is our observations and proposals:

Case 3a:

- The intermediate positioning measurements reported from the NG-RAN node to the LMF, such as LoS/NLoS Information, UL RTOA, and gNB Rx-Tx Time Difference, are already supported in the current specification. There is no need to introduce additional new IEs for them.

- For reporting model output of AI/ML assisted positioning case 3a, an indicator identifying whether the measurement is based on AI/ML model is not needed.

- Introduce new data collection initiation and reporting procedure over NRPPa, enabling NG-RAN request part B information from LMF, as similar mechanisms as data collection procedure over XnAP.

- RAN3 is kindly asked to discuss which option is the appropriate way for model performance evaluation for case 3a:

Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision.

- Option 1 (gNB performs model monitoring metric calculation and makes monitoring decision), namely option A is the appropriate way for model performance evaluation for case 3a.

- Send the LS to RAN1 to inform RAN3 preference on the option of model performance monitoring metrics calculation for case 3a.

- RAN3 is kindly asked to discuss which entity (CU or DU) is responsible for model inference to generate the intermediate positioning measurement.

- The association between part A and part B can be determined by the model training entity based on the collected measurement information and label information.

- Part A can be provided by:

  - TRP measurement result in TS 38.455 (case 3a, case 3b).

- Part B can be provided by:

  - ProvideLocationInformation in TS 37.355 (case 3b).

  - New assistance information sent from LMF to gNB (case 3a).

- Part A and Part B can be associated using:

  - the timestamp provided in measurement and/or location information.

  - UE ID associated with the measurement and/or location information.

Case 3b:

- In Case 3b, time domain channel measurements are considered as input information reported from the NG-RAN node to the LMF. The details of time domain channel measurements are still pending and subject to further discussion in RAN1.

- The existing Measurement Report message over NRPPa and F1AP can be reused for input information transferring.

3	Reference

[1] RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] TR 38.843

[3] TS 38.455

4	Draft LS to RAN1

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, USA, 18-22 Nov 2024

Title:	[Draft] LS on AI/ML assisted Positioning about model performance monitoring

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN1

Cc:	RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

- Overall description

RAN3 discussed the following options for model performance monitoring in the case of case 3a,

- Option 1: gNB performs model monitoring metric calculation and makes monitoring decision

- Option 2: LMF performs model monitoring metric calculation and makes monitoring decision

- Option 3: LMF performs model monitoring metric calculation and gNB makes monitoring decision

- Option 4: gNB performs model monitoring metric calculation and LMF makes monitoring decision

RAN3 prefers the option 1 that gNB performs model monitoring metric calculation and makes monitoring decision to avoid transmitting model performance metrics over NG-RAN interfaces.

- Actions

To RAN1:

ACTION: 	RAN3 kindly asks RAN1 to take the above information into consideration.

3	Dates of next TSG SA WG 5 meetings

RAN3#127	17 February 	- 21 February   2025		Athens, GR

RAN3#128	17 February 	- 21 February   2025		China