3GPP RAN WG3 Meeting #126	R3-247473

# Orlando, Florida, USA, 18th – 22nd November 2024

Source:	CATT

Title:	Potential RAN3 impacts of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and decision

- Introduction

This document is mainly a revised one based on R3-24xxxx. Some main differences include:

- Full update on the part of UE paging subgroup ID.

- Addition on the part of UE capability.

- Minor update on the reason that LP-WUS monitoring entry/exit condition may be more suitable to be determined by the gNB-CU.

- Discussion

  - UE paging subgroup ID

One LS [1] was received from RAN2, stating that:

1) The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. 2) RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. 3) RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. 4) For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. 5) For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

In addition, last meeting RAN1 assumed that:

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

For CN-based subgrouping, RAN2’s assumptions are consistent with RAN3’s current progress. Considering that RAN1 has decided the maximum number of total subgroups and for PEI the maximum number of total subgroups is identical to the maximum number of CN-assigned subgroups, we can now assume directly that the maximum number of CN-assigned subgroups is 32 and generates BL CRs for NGAP and F1AP capturing it accordingly. The case on XnAP was captured as pending RAN2 last meeting, thus we do not propose any BL CR for XnAP now.

Proposal 1: Introduce an “LP-WUS assistance information” IE into the NGAP PAGING message, the “Core Network Assistance Information for RRC INACTIVE” structure in NGAP, and the F1AP PAGING message. The “LP-WUS assistance information” IE includes a “CN Subgroup ID” IE with type “INTEGER (0..31, …)”.

For UE-ID-based subgrouping, the case is relatively complex. RAN2 agreed that the formula used in calculating UE-ID-based subgroup for LP-WUS is similar with the one for PEI, but this may mean the “Extended UE Identity Index Value” provided in the “Core Network Assistance Information for RRC INACTIVE” structure—which is 16-bit, used in RAN-initiated paging—may not be enough:

- Up to 10 bits in the UE ID are used in calculating POs (if no short eDRX), and up to 3bits in the UE ID are used in calculating the subgroup of PEI, which shall never overlaps with the prior 10 bits according to the formula. There may be only 3 bits left. If 3GPP agreed that the bits used in calculating the subgroup of LP-WUS shall neither overlap with the ones used in calculating the subgroup for PEI, there is no way for the gNB to determine the UE’s subgroup (requiring 5 bits) by using the remaining 3 bits.

- Up to 12 bits in the UE ID are used in calculating POs if the UE is configured with short eDRX, so there may be only 4 bits left. If 3GPP agreed that LP-WUS can be used together with short eDRX, there is no way for the gNB to determine the UE’s subgroup (requiring 5 bits) by using the remaining 4 bits.

For the most extreme case that all of short eDRX, PEI and LP-WUS exists, the total number of the required bits is 20.

Observation 1: The current 16-bit “Extended UE Identity Index Value” delivered in the “Core Network Assistance Information for RRC INACTIVE” structure may not be enough for determining the subgroup of LP-WUS.

The simplest solution for this problem is extending—for another time—the UE Identity Index Value provided by the core network. However, this value is acquired by every RAN node when serving the UE and thus can be used as tracking ID by an attacker if the IE is too unique. We believe RAN3 shall consult SA2/SA3 before deciding to extend.

Observation 2: The “Core Network Assistance Information for RRC INACTIVE” structure is available at every gNB which serves the UE. Further extending the UE Identity Index Value provided in this structure may cause concern on privacy (enabling attackers to track UEs) and thus requires allowance from SA2/SA3.

If SA2/SA3 rejects the proposal of further extending the UE Identity Index Value, RAN2 will probably have to find other solutions, which in turn will probably cost some time. Therefore, we believe SA2/SA3 should be consulted as early as possible.

Observation 3: If SA2/SA3 rejects the proposal of further extending the UE Identity Index Value, RAN2 will probably have to find other solutions, which in turn will probably cost some time.

Proposal 2: Send an LS to SA2/SA3 (as a reply LS to the RAN2’s) pointing out the problem stated in Observation 1 and asking SA2/SA3 whether it is acceptable to introduce a further extended UE ID—20-bits at least—into the “Core Network Assistance Information for RRC INACTIVE” structure.

  - UE capability

When paging a UE, the UE’s capability may impact whether to use LP-WUS and what PO to use, e.g.:

- If a UE does not support UE-ID-based LP-WUS subgrouping, the paging gNB should not use UE-ID-based LP-WUS subgrouping when paging the UE.

- If a UE does not support LP-WUS on a given band, the gNB should not use LP-WUS on the given band when paging the UE.

- The minimum offset between the LP-WUS and the PO may depend on the UE’s capability as well—it is useless to send any paging message on the POs which is too early for the UE to wake up.

These capability items will probably be introduced as a part of UE’s radio capability, so they are naturally included within RRC containers for NGAP and XnAP. On F1AP, however, they have to be delivered as F1AP IEs.

Considering the complexity, it seems hard for RAN3 to conclude soon on what IE to introduce and how to encode. We therefore prefer a simpler approach: fully following the ASN.1 design in RRC—for example, if there is a per-band UE capability on LP-WUS support in RRC, RAN3 either introduce the entire RRC band list as an RRC container, or introduce a similar band list (rather than a cell list) into F1AP.

Proposal 3: For UE capability items which are needed on F1AP, assume similar IEs are introduced as they are in RRC.

  - System Information

Configuration of LP-WUS (except for CN-based subgroup ID) is provided to the UE by the system information. It will not cost many bits and is likely included in SIB1. The content of it is assumed by RAN2 that:

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

-	LP-SS configuration

-	LP-WUS configuration

-	Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

The first two bullets indicate the configuration of the signal itself and should of course be decided by the gNB-DU. The last bullet, on the other side, may be more suitable to be determined together with many thresholds for cell reselection, including the potential threshold on LP-WUS RSRP that triggers neighbour cell measurement. Therefore, the entry/exit condition for LP-WUS may be more suitable to be decided by the gNB-CU.

Therefore we may assume now that the entry/exit condition for LP-WUS should be delivered from the gNB-CU (where it is determined) to the gNB-DU (where SIB1 is encoded).

Proposal 4: Assume that the entry/exit condition for LP-WUS monitoring should be delivered as separate F1AP IEs from the gNB-CU to the gNB-DU. This may be revisited upon further RAN2 discussion.

  - Emergency service

A UE using LP-WUS will experience significant extra paging delay compared with UEs not monitoring only conventional (Rel-15) POs. This may be suboptimal for emergency service. If there is an (already established) emergency session between the UE and the network, and the UE is currently not in RRC_CONNECTED state, there may be some benefit to prevent the UE from using LP-WUS.

The case is simple at the UE side: A UE knows whether there is an established emergency session (by checking e.g. its own NAS context), and if there is, the UE may simply act as if it does not support LP-WUS for shorter paging delay.

The case in CN is simple as well: The only impact on CN is assigning the CN-assigned subgroup ID. As long as the UE does not indicate its support of LP-WUS, the CN will not assign—the same method as in PEI.

The case in RAN, however, is not so easy, since RAN is not provided with necessary information whether there is an emergency session for a UE. There are a few possible attitudes toward this issue:

- Simply ignores the drawback. The delay caused by LP-WUS is probably within 1000ms (waiting for RAN1 to decide), comparable with the delay caused by PO periodicity (and occasionally missing a couple of PO when monitoring). This may be acceptable.

- gNB begins to send Uu paging from the first available PO after it knows it shall page, rather than the first available PO after LP-WUS wake up period. Not only the UE with established emergency session but also UEs not monitoring LP-WUS due to other reason (e.g. out of enter condition) will benefit from such implementation. (Note that RAN2 has agreed that the gNB is assumed to be unaware of whether a UE monitors LP-WUS.)

- Assume that emergency session always come with the highest paging priority (an IE within NGAP paging message and/or NG-U DL packet). The gNB begins to send Uu paging from the first available PO for the highest paging priority.

- The UE updates its Uu capability (by UE Assistance Information maybe) as “not supporting LP-WUS” once an emergency session is established, fully aligned with the method in NAS. The gNB will of course act as legacy since it considers the UE not supporting LP-WUS.

- The core network indicates that a session is an emergency one, or an NG paging is triggered for an emergency session, in the corresponding NGAP messages.

The fourth attitude requires adequate design in RAN2, whereas the last attitude requires enhancement on RAN3 stage-3 specification—other attitudes are fully implementation-based and independent with each other (i.e. there would not be any extra issue if one gNB uses one implementation and another gNB uses another). We propose RAN3 to discuss this issue.

Proposal 5: To discuss the issue of emergency service handling based on the five attitudes listed above.

- Conclusion

Proposal 1: Introduce an “LP-WUS assistance information” IE into the NGAP PAGING message, the “Core Network Assistance Information for RRC INACTIVE” structure in NGAP, and the F1AP PAGING message. The “LP-WUS assistance information” IE includes a “CN Subgroup ID” IE with type “INTEGER (0..31, …)”.

Observation 1: The current 16-bit “Extended UE Identity Index Value” delivered in the “Core Network Assistance Information for RRC INACTIVE” structure may not be enough for determining the subgroup of LP-WUS.

Observation 2: The “Core Network Assistance Information for RRC INACTIVE” structure is available at every gNB which serves the UE. Further extending the UE Identity Index Value provided in this structure may cause concern on privacy (enabling attackers to track UEs) and thus requires allowance from SA2/SA3.

Observation 3: If SA2/SA3 rejects the proposal of further extending the UE Identity Index Value, RAN2 will probably have to find other solutions, which in turn will probably cost some time.

Proposal 2: Send an LS to SA2/SA3 (as a reply LS to the RAN2’s) pointing out the problem stated in Observation 1 and asking SA2/SA3 whether it is acceptable to introduce a further extended UE ID—20-bits at least—into the “Core Network Assistance Information for RRC INACTIVE” structure.

Proposal 3: For UE capability items which are needed on F1AP, assume similar IEs are introduced as they are in RRC.

Proposal 4: Assume that the entry/exit condition for LP-WUS monitoring should be delivered as separate F1AP IEs from the gNB-CU to the gNB-DU. This may be revisited upon further RAN2 discussion.

Proposal 5: To discuss the issue of emergency service handling based on the five attitudes listed above.

We draft an LS [2] according to Proposal 2.

- Reference

[1] R3-247007/R2-2409226; LS on LP-WUS subgrouping; RAN2.

[2] R3-247474; [Draft] Reply to LS on LP-WUS subgrouping; CATT.