3GPP TSG-RAN WG3 #124	R3-247474

Orlando, Florida, USA, 18th – 22nd November 2024

Title:	[Draft] Reply to LS on LP-WUS subgrouping

Response to:	LS on LP-WUS subgrouping (R3-247007/R2-2409226)

Release:	Rel-19

Work Item:	N/A

Source:	CATT [To be RAN3]

To:	RAN2, SA2, CT1, SA3

Cc:

Contact Person:

#### Name:	LIU Aijuan

E-mail Address:	liuaijuan (at) catt (dot) cn

1. Overall Description:

RAN3 thanks RAN2’s LS for providing information. RAN3 notices that one issue may need further coordination following RAN2’s conclusion in the LS and one working assumption achieved in RAN1:

1) The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. 5) For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

According to the current TS 38.304 where LP-WUS is not supported yet, the subgroup for PEI is determined by the (truncated) 5G-S-TMSI: up to 12 Least Significant Bits (LSBs) of 5G-S-TMSI are used for selecting the PO, and the next up to 3 bits are used for determining the PEI subgroup. Therefore, up to 15 LSBs of 5G-S-TMSI in total are used.

Now for LP-WUS, up to 5 bits (as in RAN1 working assumption) will be needed for determining the LP-WUS subgroup, and they will probably be truncated from 5G-S-TMSI too (as RAN2 conclusion). PEI and LP-WUS may be used together, so up to 20 LSBs of 5G-S-TMSI in total may be needed.

Therefore, if all of PEI, LP-WUS and RRC_INACTIVE state are supported by the UE and the NG-RAN, the 20 LSBs of 5G-S-TMSI of a given UE will be needed to be provided from the AMF to any gNB which serves the UE, and stored as a part of the UE context in the gNB, so that the NG-RAN can perform paging correctly after releasing the UE into RRC_INACTIVE state.

RAN3, however, is not sure whether it will be allowed to do so. RAN3 would like to invite SA2/SA3 for further check.

## 2. Actions:

To SA2 and SA3:

ACTION: RAN3 respectfully asks SA2/SA3 to check whether it is allowed that the AMF provides the 20 LSBs of the 5G-S-TMSI of a given UE toward every gNB which serves the UE, and that the receiving gNB stores it as a part of the UE context.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #127	17th – 21st February 2025	Athens, Greece

TSG-RAN3 Meeting #127-bis	07th – 11th April 2024	TBD, China