3GPP TSG-RAN WG3 Meeting #126	R3-247482

Orlando, USA, 18 – 22 November 2024

Agenda item:	12.3

Source:	Nokia

Title:	Completion of Functional aspects of NR Femto Architecture

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At the last RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

This paper evaluates the corresponding functional impacts and proposes stage 2 CR in [2] and stage 3 CR in [3], then analyses remaining open points and propose LS in [4] to progress the work.

# 2	Discussion

- Progress on stage 2 and stage 3

The following conclusion was reached in the TR 38.799 for the NG interface:

### 6.2.1	Architecture for NG interface

Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible.

In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase.

It is clear from the above text that the agreement to have an optional NR Femto Gateway in the architecture was taken in consideration of smooth integration for an operator who has deployed HeNBs using HeNB GW. Therefore, it is assumed that the protocol and functional aspects of the NR Femto GW to be specified to enable an equivalent level of functionalities to the operators as the one of the HeNB GW in 4g network.

The protocol and functional aspects of HeNB subsystem and HeNB GW can be found in details in TS 36.300 section 4.6. We therefore propose to take these functional aspects as a starting point to evaluate the necessary equivalent functions for the NR Femto, the NR Femto GW, and the AMF. They need not necessarily be the same because 5G has its own characteristics but an equivalent level of functionalities should be provided in our view.

Proposal 1: discuss the functional aspects of NR Femto, NR Femto GW and AMF presented in the TP for TS 38.300 draft CR in [2] to provide equivalent support of functionalities as the HeNB subsystem.

Proposal 2: discuss the corresponding stage 3 aspects with the TS 38.413 CR proposed in [3].

- Remaining open points

Similarly, the functional Part of the HeNB subsystem in TS 36.300 section 4.6 reveals several security verification checks for HeNBs performed either by the HeNB GW or by the MME to validate the HeNB.

It FFS pending SA3 study whether the same verification checks would apply to the 5G NR Femto architecture agreed by RAN3. This is why the functional aspects specified in the draft stage 2 CR [2] captures these verification checks as NOTEs and open issues to be further checked.

But in the meantime, to speed up the process, it would be beneficial to emphasize explicitly to SA3 those security points which needs checking for parity with 4g and are pending in NOTEs the CR [2] so that SA3 can give a quick feedback.

Proposal 3: send the LS in [4] to SA3 to check whether the verification aspects which applied to HeNB GW architecture apply to NR Femto GW architecture.

Another functional aspect which was available in the HeNB subsystem was the proximity indication. Indeed, in order to benefit from the services offered by the femto node an handover to the closed femto node is necessary to take place at the right moment i.e. when the end user nears its femto. It is indeed cumbersome for the macrocell and the UE to constantly report neighbor cell measurements and SIB acquisition even when the UE is far away from its femto.

In HeNB subsystem the functionality of proximity detection is specified in section 10.5.1.2 of TS 36.300:

1.	Proximity Estimation: in case the UE is able to determine, using autonomous search procedures, that it is near a CSG member cell, the UE may provide to the source eNB an indication of proximity. The proximity indication may be used as follows:

-	The source eNB may determine whether to perform other actions related to handover to HNB/HeNBs based on having received a proximity indication (for example, the source eNB may not configure the UE to acquire system information of the HNB/HeNB unless it has received a proximity indication).

In HeNB subsystem it is the UE which sends the proximity indication to the eNB. 5GS can work differently because it is the gNB which gets the UE Allowed CAG List and performs the access control, same as PNI NPN. Therefore, the proximity function can work differently as follows:

- On solution can be that a gNB receives from its neighbor NR femto node(s) over Xn its CAG IDs and associated PCI(s). The gNB when serving a UE knows the UE Allowed CAG List and therefore which neighbor femtos are relevant for this UE. It can therefore provide the corresponding PCI(s) to the UE so that the UE would only start acquiring the system information of the NR Femto when detecting one of the indicated PCI(s) or only report measurements of neighbor cells which match this PCI.

- Another possible solution can be that a gNB receives from its neighbor NR femto node(s) its CAG IDs and geographical coordinates. The gNB when serving a UE knows the UE Allowed CAG List and therefore which neighbor femtos are relevant for this UE. It can provide the geographical coordinates of the relevant femtos to the UE so that the UE would only start acquiring the system information of the NR Femto when approaching an NR femto which is relevant for it.

RAN3 should start discussing how proximity can work in 5G, also liaising RAN2 for support.

Proposal 4: initiate discussions with RAN2 on how proximity works with NR Femto in 5G and possible functional impacts for RAN3.

# 3	Conclusion and Proposals

This paper has evaluated the objectives and corresponding work to complete the NR Femto work item agreed in [1] and made the following proposals:

To progress the stage 2 and stage 3:

Proposal 1: discuss the functional aspects of NR Femto, NR Femto GW and AMF presented in the TP for TS 38.300 draft CR in [2] to provide equivalent support of functionalities as the HeNB subsystem.

Proposal 2: Discuss the corresponding stage 3 aspects with the TS 38.413 CR proposed in [3].

To address remaining open points:

Proposal 3: Send the LS in [4] to SA3 to check whether the verification aspects which applied to HeNB GW architecture apply to NR Femto GW architecture.

Proposal 4: Initiate discussions with RAN2 on how proximity works with NR Femto in 5G and possible functional impacts for RAN3.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-247486, [TP for BL CR NR Femto 38.300] Introduction of Functional aspects of NR Femto architecture

- R3-247487, Introduction of NR Femto architecture with optional NR Femto GW, TS 38.413

- R3-247488, LS on security verifications related to NR Femtos