3GPP TSG-RAN WG3 Meeting #126	R3-247496

# Orlando, U.S.A, 18th – 22nd November, 2024

Source:	Ericsson, T-Mobile US, InterDigital, Jio, Telecom Italia, Deutsche Telekom

Title:	On MDT enhancements for Network Slicing

Agenda item:     10.3.2

Document for:	Discussion, Decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

In meetings leading up to TSG-RAN WG3 #125-bis the following agreements and FFSs were made with respect to the future work on SON/ MDT enhancements for Network Slicing:

- For the immediate MDT, it is not needed to add measurements related to the slice unavailability and the lack of resources. - The slice ID will be added as a filter to the area scope (for the both, management- and signalling-based immediate MDT related to slice). The intention is to limit the number of reports sent to the OAM, thus the UE selection will not be amended. - Not to enhance MDT area scope for logged MDT for slice.

In addition to the above, discussion on proposals regarding MDT enhancements for logging denied slice service request at NAS level due to slice not being supported in the cell from which the UE service request was initiated and slice observability under different load conditions have not been opened or remain inconclusive. In this contribution, we share our views and proposals on these topics.

# 2	Discussion

## 2.1	New MDT measurements for observability of denied slice service request

An area where it is beneficial to have better observability is in the overlap between UE originated Service Request to establish sessions on certain slices and its relation to the existing slice coverage across the network. Indeed, configuring a perfect match between the coverage of the cells in which a slice is supported and available and the geographical areas where the users should have access to the services provided by a slice may be challenging in some cases and mismatches may occur. Another dimension of the same problem is to understand from which geographical areas UEs may request to access services over a certain slice, and if the slice deployment must be changed to satisfy such requests.

For example, let us consider the case of an operator providing services over a certain slice for a customer such that the employees of the customer can access the slice services, i.e., the enterprise slice, for work-related purposes.  The users may request access to slice services while working from the office premises of their employer but also while working remotely from home. While configuring a good overlap between the slice coverage and the customer’s office premises is rather easy, it may be comparatively more difficult to configure a good slice coverage overlapping with the remote working location of each employee.

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests at the RAN/OAM.

Below we describe solutions to the above problem and motivate our proposals based on the observed pros and cons. In general, the configuration problem is two-fold. The first is to configure the cells/tracking areas that overlap with the enterprise location. This part of the configuration task is relatively simple and can be achieved with careful network planning and correlating the network deployment with measurements using existing MDT measurements. The second is to configure the cells/tracking areas in remote locations, e.g., employee households, is a much harder and a dynamic task. Below we discuss solutions to this second configuration problem.

As one potential solution to support the configuration task described above, the RAN may attempt to configure all cells in a geographic area to support the enterprise slice, however, these slices may have zero resources allocated to them. As a consequence of the above, the RAN will become aware of each UE request trying to access the slice. The RAN after receiving one or more request to access the slice in a cell may decide to allocate resources to the enterprise slice in certain cells in the registration area. The drawback of the above solution is that one or more registration areas may need to advertise support for the enterprise slice, which has both a configuration overhead and an overhead during execution, i.e., UEs may handover to the cell with an expectation that the slice is supported, but the lack of allocated resources to the slice is only known after the UE tries to access resources on the target cell after the handover. Due to the above drawbacks, logging UE’s service request in the network by advertising slice support with no allocated resources to the enterprise slice is not a viable solution.

A second potential solution to the problem is to log the service requests at the core network at the time of a service request. The drawback of such an approach is that not all the information pertaining to the service request is available at the AMF/core network, for e.g., the AMF and core network entities are unaware, by design, of cells in the RAN, but rather are aware of the RAN only in terms of a Registration Area or a Tracking Area. Knowing the mapping between the UE’s service request and a cell in the TA/RA is of vital importance in performing resource allocation to the enterprise slice in specific cells. Therefore, logging at the core network is also not a viable option.

A third potential solution, which avoids the aforementioned drawbacks would be to enhance MDT to log users’ access request to slice services, especially the events where user’s request to a service provided by a slice is rejected.

For a UE initiated service request, if a service is to be requested over a slice that is currently in the Rejected S-NSSAI list, the UE NAS-layer will not allow such a request to be performed. For such situations, logging the slice access requests that are rejected at the UE NAS is the only means to obtain information about such events. On another hand if the UE initiated service request is requesting access to a service over a slice that is in the Configured S-NSSAI list, but not yet in the Allowed S-NSSAI list, the UE NAS will attempt establishment of such a session. Such a request may then be accepted/rejected by the AMF depending on the UE’s registration area, subscription, and other constraints. Such events too can be logged at the UE side via MDT, and complex procedures to exchange the information between the core and the RAN nodes are not required.

In both the above-described scenarios, the RAN and the OAM do not have immediate visibility into the access attempts from the UE as the rejection happens at the UE NAS layer, or at the NAS entity at the core network. By enabling logging via MDT, such information can be shared with the RAN/OAM and can expedite the configuration task at hand.

Proposal 1: RAN3 to agree adding new MDT measurements for detecting holes in the coverage of a slice.

With access to such MDT reports, the RAN and the OAM may, depending on the slice to which access was rejected in the service request, determine to make the slice partially allowed in certain Tracking Areas, and depending on future service requests allocate resources for the requested slice to each cell in the Tracking Area.

Proposal 2: RAN3 to enhance network slice coverage observability by adding support of new MDT measurements to log the UE’s slice service requests rejected by the NAS layer because the slice is not  supported in the cell from which UE is making the request.

## 2.2	Slice observability under different load conditions

- In management-based immediate MDT, OAM configures each individual RAN nodes and RAN nodes perform UE selection and collection of MDT measurements. Afterwards, the measurements are sent back to OAM. The UEs are selected based on the configuration provided by the OAM in the area scope after taking considerations of UE capability. However, the MDT report collection starts upon selection of a suitable UE.

Observation 2: In management-based MDT, upon reception of the MDT configuration from the OAM, RAN nodes selects suitable UEs in the provided  area scope taking consideration of UE capabilities and begins report collection.

- Similarly, in signalling-based immediate MDT, the OAM tasks the RAN to configure MDT measurements on a specific UE, and the UE is configured immediately. The measurement in this case too commences immediately at the UE and the results are then signalled back to the OAM.

- Hence, it may happen that the OAM collects MDT measurements from RAN when the slice utilization is low and the reported performance may be good, e.g., during non-peak utilization hours of a slice. It should however be noted that the peak utilization period of a slice may be different when compared to the peak utilization period of the overall RAN node. This is shown in the below table-

| - Time | - Slice A utilization | - Slice B utilization | - Overall RAN node utlization |
| --- | --- | --- | --- |
| - t1 | - 50% | - 70% | - 25% |
| - t2 | - 26% | - 65% | - 60% |
| - t3 | - 20% | - 10% | - 75% |
| - t4 | - 80% | - 90% | - 20% |

- The table shows possible different utilizations of different slices along with overall RAN node utilization. Different slices may have peak utilization at different times, and they can be uncorrelated with the overall RAN node utilization. Thus, if OAM collects MDT reports at t3, the slice-specific results would not reflect the actual load condition at t4.

Observation 3: If OAM collects MDT reports for a specific slice when its utilization is low, it would not be able to analyse any potential performance degradation of the slice.

- An alternative is to collect MDT measurements for a very long period; this, however, would lead to transmitting large amount of data to OAM of which very little would be of relevance.

- Thus, to increase slice specific observability and transmission of pertinent data, it is important that the OAM nodes collect MDT measurements based on some slice specific KPIs available in RAN. For example, if the RAN nodes are provided with a criterion to collect MDT measurements for a slice only when the slice specific load is very high or the overall RAN node utilization is very high, it would be able to observe if there is a degradation in the slice performance.

Observation 4: If OAM collects slice specific MDT reports during high slice utilization period, it would be able to analyse if there is degradation in slice performance under high load.

It is also important to note that the Slice utilization and the overall RAN utilization can fluctuate significantly during a short period of time. In order to guarantee that the collected measurements pertain to the specified collection criteria, it should be possible to activate/deactivate or pause/resume the slice specific MDT measurements without affecting the MDT measurement configuration or requiring a reconfiguration.

Observation 5: To obtain slice/RAN load specific measurements, whenever the system-associated or UE-associated metrics are outside the specified collection conditions, the measurement reports should not be collected.

Proposal 3: To increase slice performance observability under different load conditions and reduce amount of data sent to OAM, RAN3 to discuss enhancement of immediate MDT configuration.

# Conclusion

In the previous sections we made the following observations:

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests at the RAN/OAM.

Observation 2: In management-based MDT, upon reception of the MDT configuration from the OAM, RAN nodes selects suitable UEs in the provided i area scope taking consideration of UE capabilities and begins report collection.

Observation 3: If OAM collects MDT reports for a specific slice when its utilization is low, it would not be able to analyse any potential performance degradation of the slice.

Observation 4: If OAM collects slice specific MDT reports during high slice utilization period, it would be able to analyse if there is degradation in slice performance under high load.

Observation 5: To obtain slice/RAN load specific measurements, whenever the system-associated or UE-associated metrics are outside the specified collection conditions, the measurement reports should not be collected.

Based on the discussion in the previous sections we propose the following:

Proposal 1: RAN3 to agree adding new MDT measurements for detecting holes in the coverage of a slice.

Proposal 2: RAN3 to enhance network slice coverage observability by adding support of new MDT measurements to log the UE’s slice service requests rejected by the NAS layer because the slice is not  supported in the cell from which UE is making the request.

Proposal 3: To increase slice performance observability under different load conditions and reduce amount of data sent to OAM, RAN3 to discuss enhancement of immediate MDT configuration.

# References

RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC