3GPP TSG-RAN WG3 #126	R3-247499

Orlano, USA, 18th – 22nd November 2024

Agenda Item:	10.4

Source:	Ericsson, CATT, ZTE Corporation

Title: 	Further discussion on RACH optimization for SDT

Document for:	Discussion and Approval

# 1	Introduction

In relation to RACH optimization for SDT, at RAN3#125 meeting the following agreement was achieved:

The information that stored and reported in the RA report for SDT:

-	c. the time from the start of RA-SDT to the reporting of RA report

At the same meeting, the following FFS was captured: “FFS on other SDT related information.”.

At RAN3#125-bis, no consensus could be reached on which information should be stored and reported in the RA report for SDT.

In this contribution we further discuss relevant SDT related information that can be included in the RA report.

# 2	Discussion

According to MAC spec TS 38.321, clause 5.27.1, an SDT procedure initiated for MO-SDT can be performed either by RA-SDT or by configured grant Type 1 (i.e., CG-SDT). RRC configures a number of parameters for SDT, and two of them – sdt-DataVolumeThreshold and sdt-RSRP-Threshold - are checked both in case of MO-SDT performed by RA-SDT and in case of MO-SDT performed by CG-SDT.

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT cannot be performed by RA-SDT (i.e., RA-SDT is not applicable as specified in clause 5.1.1b), but can be performed either by Random Access procedure (i.e., with 2-step RA type or 4-step RA type) or by configured grant Type 1 (i.e., CG-SDT). RRC configures the following parameters for SDT procedure: - sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT; - sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT; - mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT; - cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT; - cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT; - cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT; - sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

A UE may attempt to perform MO-SDT and if one or both of the common criteria (on RSRP and data volume) used to evaluate whether the UE can perform CG-SDT or RA-SDT is not fulfilled, the UE will use common RACH resources.

At previous meetings, RAN2 has agreed that for SDT optimization, the UE includes RSRP and data volume related information in RA reports:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

However, for the MO-SDT case, including the RSRP and data volume related information is not enough for the network to understand if optimization is required for RA-SDT, since the related thresholds are in common for RA-SDT and CG-SDT. In other words, if the RSRP value and the buffered uplink data volume are included in RA report when SDT was evaluated, we can have two situations:

- SDT was not performed, and common RACH resources were used

- SDT was performed

In the first case, the common RACH resources were used because at least one of the RSRP value or the buffered uplink data volume was not fulfilling the respective condition. However, from the network does not know whether the UE selected the common RACH resources as a consequence of a “conventional” RA procedure, or because the conditions for the selection of CG-SDT or RA-SDT resources were not satisfied.

In the second case, there is still an ambiguity, because, according to MAC spec (same clause 5.27.1), if both CG-SDT and RA-SDT are configured, and both conditions on RSRP and data volume are satisfied, the UE first evaluates if CG-SDT is possible, and if not, it performs RA-SDT. This can happen for example, when a CG-SDT specific condition (e.g. TA for CG-SDT is not valid, next CG occasion is too far away in time, etc) was not fulfilled, in which case the UE uses RA-SDT. Hence, the network cannot distinguish whether RA-SDT was used because CG-SDT was not configured, or RA-SDT was used because CG-SDT was configured but could not be used (note that the reason why CG-SDT could not be used is not relevant for this discussion).

So, we can derive the following observations:

When at  least one of the RSRP-based or data volume-based condition is not fulfilled, from the RA information, the network is not able to distinguish between the following scenarios:

  - the UE evaluated RA-SDT but used common RACH resources

  - the UE evaluated CG-SDT but used common RACH resources

When both RSRP-based and data volume-based conditions are fulfilled, from the RA information, the network is not able to distinguish between the following scenarios:

  - the UE used RA-SDT after evaluating CG-SDT

  - the UE used RA-SDT without first evaluating CG-SDT

We note that, without knowing whether UEs evaluated CG-SDT before using RA-SDT, it becomes unclear how to dimension RA-SDT resources. For example, RA-SDT resources may have been dimensioned assuming that CG-SDT is used, but it is not (or at least not used as much as planned). So, an unexpected use of RA-SDT due to UEs evaluating CG-SDT but using RA-SDT can overload RA-SDT resources.

Unwanted use of RA-SDT after evaluation of CG-SDT can overload RA-SDT resources, but the network has no visibility of that.

RAN3 agreed to enhance RA report including the time from start of RA-SDT, so a possible assumption can be that knowing this information the network can deduce whether CG-SDT was at least configured. For this reason, one could assume that the ambiguity indicated in Observation 2 can be resolved by the network using the “time from start of RA-SDT” information. Still the ambiguity described in Observation 1 remains. Therefore, to enable appropriate RACH optimization for RA-SDT, we propose that the RA report is enhanced with some further information with which the UE informs the network if the UE has evaluated RA-SDT and/or CG-SDT.

If the UE evaluated to perform SDT prior to executing the random access in the commond RACH resources, the UE indicates in the RA Report whether RA-SDT and/or CG-SDT was evaluated.

If the UE evaluated to perform CG-SDT prior to executing the random access in RA-SDT resources, the UE indicates in the RA Report if CG-SDT was evaluated.

# 3 Conclusions

In this contribution the following observations and proposal were derived:

Observation 1	When at  least one of the RSRP-based or data volume-based condition is not fulfilled, from the RA information, the network is not able to distinguish between the following scenarios:

a.	the UE evaluated RA-SDT but used common RACH resources

b.	the UE evaluated CG-SDT but used common RACH resources

Observation 2	When both RSRP-based and data volume-based conditions are fulfilled, from the RA information, the network is not able to distinguish between the following scenarios:

a.	the UE used RA-SDT after evaluating CG-SDT

b.	the UE used RA-SDT without first evaluating CG-SDT

Observation 3	Unwanted use of RA-SDT after evaluation of CG-SDT can overload RA-SDT resources, but the network has no visibility of that.

Proposal 1	If the UE evaluated to perform SDT prior to executing the random access in the commond RACH resources, the UE indicates in the RA Report whether RA-SDT and/or CG-SDT was evaluated.

Proposal 2	If the UE evaluated to perform CG-SDT prior to executing the random access in RA-SDT resources, the UE indicates in the RA Report if CG-SDT was evaluated.