3GPP TSG-RAN WG3 Meeting #126	R3-247521

Orlando, USA, 18th Nov – 22nd Nov, 2024

Title: 	AI/ML support for CCO

Source: 	Ericsson, InterDigital, Deutsche Telekom

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we continue to discuss AI/ML support for CCO based on the agreements and FFS captured at last RAN3 meeting.

Stage3 signalling design:

Xn interface:

Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

The Coverage Modification Cause reflects the predicted CCO issue.

F1 interface:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Timing information:

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

Information to validate the prediction is needed.

# 2. Discussion

## CU/DU configuration update procedures

At the previous meeting a WA has been made on reusing CU/DU configuration update procedures:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

At the previous meeting, some proposals were made, to introduce new procedures for transferring the predicted CCO issue and the future CCO state, instead of reusing the existing procedures. According to these proposals, e.g., a DU would subscribe to receive a predicted CCO issue. 
In our view, this approach does not bring particular benefits and complicates the overall handling and implementation of CCO functionality. 
For example, by reusing the legacy signalling, if a DU receives a predicted CCO issue and it does not want to or cannot handle that the predicted CCO issue, it can fail the procedure. Later on, if the CU finally detects the CCO issue, the CU can still, using the legacy way, inform the DU that now a CCO issue is detected. 
The same would not be possible by using procedures similar to the Xn Data Collection Reporting Initiation and Xn Data Collection Reporting, as the gNB-DU cannot reply to the reception of a CCO Issue via a procedure similar to the Xn: Data Collection Reporting Update. Whether the gNB-DU decides to handle the predicted CCO issue or not, it will at some point need to inform gNB-CU of its decision. And for that it will need to use the legacy procedure, thus creating a mix of both legacy and new procedures to complete the task, something that creates unnecessary confusion and is more prone to error.

Using legacy procedures enables a simple implementation, because it is more straightforward to convey new and legacy CCO related information in a consistent way. Indeed, by using different procedures for legacy and AI/ML based CCO, the same CCO function will be handled in a more scattered way, where legacy information is signalled via gNB-DU/gNB-CU Configuration Update, while AI/ML based information is signalled via a new procedure. This complexifies implementations. In summary, we propose to confirm the WA, and reuse the existing CU/DU configuration update procedures.

Proposal 1: To support AI/ML based CCO, reuse the CU configuration update procedure to transfer the predicted CCO issue and reuse the DU configuration update procedure to transfer the Future Coverage State over F1.

## F1 and Xn impacts

At the last meeting it was agreed to signal separate “time for future coverage state” and “time for predicted CCO issue”, and simple definitions for them were also agreed.

Introducing separate time information for future coverage state and predicted CCO issue.
Time for future coverage state: The point in time when the future coverage state will be applied.
Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

Also, RAN3 agreed the following: “Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.” and that “The Coverage Modification Cause reflects the predicted CCO issue.”.

Based on the above, we describe our view and proposals for detailed F1 and Xn signaling.

If gNB1 is the node where the CCO issue is predicted, and gNB2 is neighbouring gNB1, to enable gNB2 to take appropriate actions for matching the future CCO State derived at gNB1, gNB2 needs to receive information concerning three aspects:

- What: the gNB2 needs to know which type of predicted CCO issue will affect gNB1, to be able to adjust its coverage accordingly. Given that, we think that the Coverage Modification Cause IE should indicate exactly the type of predicted CCO issue predicted by gNB1.

- How: to decide how to match the coverage modification at gNB1, gNB2 (DU2) needs to receive information that can help to find the proper match. It is the DU2 in gNB2 that will generate the matching coverage, so the future coverage state needs to be sent first from DU1 to CU1, then from gNB1 to gNB2, and finally from CU2 to DU2.

- When: to decide when it is appropriate to match the future coverage state, gNB2 (DU2) needs to receive the time at which the future coverage state at gNB1 will be applied, and the time for the predicted CCO issue inferred by gNB1 (after that time, the CCO issue is predicted to manifest, hence this timing information is useful to determine the maximum limit in time when a CCO State change at gNB2 should be applied). As for the future coverage state, the information is needed at DU2. Therefore, like the future coverage state, the time for the future state decided by DU1 needs to be sent first from DU1 to CU1, then from CU1 to CU2, and finally from CU2 to DU2. The predicted CCO issue and its timing is available at CU1, so this should be signalled first from CU1 to CU2, and then from CU2 to DU2.

Based on the discussion and the existing agreements, we propose the following F1 and Xn impacts concerning timing information:

- For the gNB predicting CCO issue (gNB1):

  - CU1 sends to DU1 the time for predicted CCO issue (and, as already agreed, the predicted CCO Issue)

  - DU1 sends to CU1 the time for future coverage state (and, as already agreed, the Future Coverage State)

- For the Xn from the gNB predicting CCO issue (CU1) to the neighbor gNB (gNB2 or CU2):

  - CU1 sends to CU2 the time for predicted CCO issue (of gNB1) and the time for future coverage state (of gNB1) (as well as the predicted Coverage Modification Cause and future Coverage State, as already agreed)

- For the target neighbour gNB (gNB2):

  - CU2 sends to DU2 the time for predicted CCO issue (of gNB1) (as well as the predicted CCO issue derived by gNB1), the future coverage state (of gNB1) and the time for future coverage state (of gNB1)

Proposal 2: To support AI/ML based CCO in a gNB1 where the CCO issue is predicted: 1) source CU sends to source DU the predicted CCO issue type and the time for predicted CCO issue; 2) source DU sends to source CU the future CCO state and the time for future CCO state.

Proposal 3: To support AI/ML based CCO in a target neighbour gNB2: 1) CU of gNB2 sends to DU of gNB2 the predicted CCO issue type predicted at gNB1 and the associated time for the predicted CCO issue, the future CCO state at gNB1 and the associated time for such future CCO state.

Proposal 4: To support AI/ML based CCO the gNB1 where the CCO issue is predicted signals to a neighbouring gNB2: the predicted CCO issue type inferred at gNB1 and the associated time for the predicted CCO issue, the future CCO state at gNB1 and the associated time for the future CCO state.

Looking at F1 impact in more details, considering the reuse of the CU/DU Configuration Update procedures, we think two options can be evaluated:

- CU to DU:

  - Option 1: The predicted CCO issue and the affected cells and beams are signalled by reusing and extending the existing CCO Assistance Information IE with a time for predicted CCO issue, implicitly indicating that the type of CCO issue is a predicted CCO issue, and the Affected Cells and Beams IE indicates the predicted affected cells and beams

  - Option 2: The predicted CCO issue, the predicted affected cells and beams and the time for predicted CCO issue are signalled with a new IE (e.g., Predicted CCO Assistance Information), containing all the information above.

- DU to CU:

  - Option 1: The future CCO state is signalled by reusing and extending the existing Coverage Modification Notification IE, indicating that the cells and SSB coverage states included in the IE will be applied at the time of future CCO state

  - Option 2: The future CCO state is signalled with a new IE (e.g., Future Coverage Modification Notification), containing the future cells and SSB coverage states and the time of future CCO state

We don’t have a strong preference, on the other hand the definition of separate IEs for AI/ML based CCO seems to be more future proof in case some more information needs to be added to extend the AI/ML based CCO without impacting legacy CCO.

Proposal 5: gNB-CU Configuration Update is extended with a new IE (e.g., Predicted CCO Assistance Information) containing: predicted CCO issue type, predicted affected cells and beams, time for predicted CCO issue.

Proposal 6: gNB-DU Configuration Update is extended with a new IE (e.g., Future Coverage Modification Notification) containing: future cells and beams coverage state, time of future CCO state.

Regarding the time when the CCO issue is predicted, if that is far in the future, there can be cases where the CU realizes that the predicted CCO issue is outdated (e.g., the traffic in the cells have changed considerably), and the coverage modification can be avoided. In this case, we think it is beneficial that CU informs the DU that a previously signalled predicted CCO issue is no longer valid (and the upcoming coverage modification should be cancelled).

Proposal 7: CU can inform DU that a previously predicted CCO issue in no longer valid.

## Overview of the solution for AI/ML based CCO

Considering the proposals above, we present an overview of the solution for AI/ML based CCO in the flow chart below.

Figure 1 – Overview of the solution for AI/ML based CCO

## Information to validate the prediction

At the last RAN3 meeting, the following open point was captured: “Information to validate the prediction is needed.”.

With AI/ML based CCO, a predicted CCO issue can be the output of AI/ML inference, and it can be noted that, regardless of whether a predicted CCO issue is “coverage” or “cell edge capacity” the action that will be taken to resolve/anticipate the issue is a change in coverage. When the CCO-related action results in one cell (or beam) shrinking and another one expanding, the fundamental effect that UEs will experience is a change of the radio channel. Some UEs that were served by a cell will be served by another one, and all UEs in the vicinity of the CCO issue area will measure different coverage and/or different quality of the radio channel. It can happen that the performance of the UE in the new cells are different, but this is an indirect effect potentially influenced by many other factors, like the user behavior or the RRM policy for a given UE. Instead, the change in radio environment does not depend on those factors, and radio measurements (RSRP, RSRQ, SINR) can provide a more precise and direct information to understand whether a CCO action resulted in a good or bad coverage configuration for the UEs.

For example, a CCO action leads to a contraction in coverage for cell 1 of gNB1 and to the expansion of coverage for a neighbor cell 2 of gNB2. Some UEs will be handed over from cell 1 to cell 2. To understand if the issue was prevented, UEs in cell 2 should not suffer poor coverage or high cross cell interference.

In order to determine if the CCO issue was prevented, gNB1 may request gNB2 to report UE measurements (RSRP/RSRQ/SINR) for those UEs which will be handed over to gNB2.

We focus our attention on the UEs that are handed over between cells involved in a CCO state change, because by doing this we can reuse the UE Performance reporting framework. Namely we can simply enhance the UE performance reported after a handover, hence collecting feedback information for CCO and limiting the impact on existing procedures

After gNB2 sends the UE radio measurements to gNB1, gNB1 uses them to understand if the predicted CCO issue was prevented by means of the CCO State changes. This also creates an understanding of whether the predicted CCO issue would have occurred if the CCO state changes were not applied.

For example, if the predicted CCO issue was an interference at cell edge, a good SINR reported by UEs in the area affected by the CCO state changes indicates that this was successfully prevented. If the predicted CCO issue was related to coverage, a good RSRP reported by UEs in the area affected by the CCO state changes indicates that this was successfully prevented.

We also would like to note that reporting of radio measurements on a per UE level, even if for a different purpose, is not new in RAN3. Over the X2, as part of Resource Status Reporting, RSRP measurements for a UE can be requested (and reported) between eNBs. We can reuse a similar approach for CCO, with a request to report UE radio measurements in the Data Collection Reporting. At handover preparation, the Data Collection ID is added to the Handover Request, and the target gNB collects the radio measurements for the handed over UEs. The reporting is UE-specific and can be provided in the Data Collection Update together as part of UE Performance.

An example of how the Report Characteristics can be extended to request UE radio measurements is in the table below.

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory, Tenth Bit = UE measured RSRP Eleventh Bit = UE measured RSRQ Twelfth Bit = UE measured SINR Other bits are ignored by the NG-RAN node2. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

An example of UE Measurement Report that can be used to extend UE performance contained in the Xn Data Collection Update is provided in the table below.

| UE Associated Info Result List |  | 0..1 |  |  | YES |
| --- | --- | --- | --- | --- | --- |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |
| >>UE Performance | O |  | 9.2.3.179 |  | – |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| UE Measured RSRP | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |
| UE Measured RSRQ | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |
| UE Measured SINR | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |

Proposal 8: To request feedback for AI/ML based CCO, reuse the Report Characteristics IE in the Data Collection Reporting Request to request for UE radio measurements.

Proposal 9: To report feedback for AI/ML based CCO, extend the UE Performance IE to include UE radio measurements.

# 3. Conclusions

Based on the discussion in this paper, we derived the following proposals:

Proposal 1: To support AI/ML based CCO, reuse the CU configuration update procedure to transfer the predicted CCO issue and reuse the DU configuration update procedure to transfer the Future Coverage State over F1.

Proposal 2: To support AI/ML based CCO in a gNB1 where the CCO issue is predicted: 1) source CU sends to source DU the predicted CCO issue type and the time for predicted CCO issue; 2) source DU sends to source CU the future CCO state and the time for future CCO state.

Proposal 3: To support AI/ML based CCO in a target neighbour gNB2: 1) CU of gNB2 sends to DU of gNB2 the predicted CCO issue type predicted at gNB1 and the associated time for the predicted CCO issue, the future CCO state at gNB1 and the associated time for such future CCO state.

Proposal 4: To support AI/ML based CCO the gNB1 where the CCO issue is predicted signals to a neighbouring gNB2: the predicted CCO issue type inferred at gNB1 and the associated time for the predicted CCO issue, the future CCO state at gNB1 and the associated time for the future CCO state.

Proposal 5: gNB-CU Configuration Update is extended with a new IE (e.g., Predicted CCO Assistance Information) containing: predicted CCO issue type, predicted affected cells and beams, time for predicted CCO issue.

Proposal 6: gNB-DU Configuration Update is extended with a new IE (e.g., Future Coverage Modification Notification) containing: future cells and beams coverage state, time of future CCO state.

Proposal 7: CU can inform DU that a previously predicted CCO issue in no longer valid.

Proposal 8: To request feedback for AI/ML based CCO, reuse the Report Characteristics IE in the Data Collection Reporting Request to request for UE radio measurements.

Proposal 9: To report feedback for AI/ML based CCO, extend the UE Performance IE to include UE radio measurements.

TPs reflecting the above proposals are provided in R3-247522 and R3-247523