3GPP TSG-RAN WG3 Meeting #126				                          R3-247551

Orlando, USA, 18th - 22th Nov 2024

Agenda Item:	17.3

Source:	ZTE Corporation

Title:		(TP to BL CR for 38.423) Further discussion on on-demand SIB1

Document for:	Discussions & Approval

1	Introduction

- This paper provides further discussion on on-demand SIB1.

2	Discussion

At the RAN#105 meeting, it was agreed to consider only Case 2 in Rel-19.

At the RAN3#125 meeting, the following agreement was reached for Case 2.

Case 2 can be supported with the below details: - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

At RAN3#125 bis meeting, the following agreement was reached.

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

During the RAN3#125 bis meeting, the following open issue was discussed.

Introduce the start/stop mechanism from NES cell to Cell A for UL WUS configuration broadcast? Further check on first activation of UL WUS configuration broadcast in Cell A?

Three options are provided in the SOD[R3-245737], see below:

Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.  (XnAP: Cell A gNB “confirm” means that it will broadcast received UL WUS configuration via its SIB right away ( i.e. not wait for 2 days) ; Cell A gNB “reject” means it will not perform the request.)

Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of the explicit “UL config provided” message from Cell A gNB.(The reason is, it may take up to 41 seconds to broadcast UL WUS even Cell A broadcast immediately, NES cell can stay as legacy until it knows that Cell A has broadcast its UL WUS.)

Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs. Then, the second message is sent from NES Cell gNB to Cell A gNB to indicate it is time to broadcast UL WUS configuration. And NES cell goes to OD-SIB Mode.

In these options ,we prefer Option 3 for the following reasons:

As agreed in the last meeting, "Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB", In other words, neighboring cells may have dynamic reasons for adjusting their assistance with broadcasting. Accordingly, the NES cell may need to change its OD-SIB1 state based on the UL WUS broadcast status of multiple Cell(s) A, furthermore, In practice, this OD-SIB1 state change might also depend on variations in the energy state of the NES cell or product implementation factors. Therefore, we believe that both the UL WUS broadcast status of Cell(s) A and the OD-SIB1 state of the NES cell may experience semi-dynamic changes.

Observation: Both the UL WUS broadcast status of Cell(s) A and the OD-SIB1 state of the NES cell may experience semi-dynamic changes.

Therefore, from our perspective, we prefer Option 3, as it is better suited to a dynamic energy-saving strategy, especially in cases where the NES cell may require assistance from multiple Cell(s) A , a change in the assistance of any cell A may result in a change to the NES cell's OD-SIB1 state.

Proposal 1: RAN3 selects option3 as a baseline, as it is better suited to a dynamic energy-saving strategy, especially in cases where the NES cell may require assistance from multiple Cell(s) A , a change in the assistance of any cell A may result in a change to the NES cell's OD-SIB1 state. (The corresponding TP for TS38.423 for option3 is provided in annex).

Additionally, we suggest waiting for the Xn interface solution to be finalized before discussing the impact on the F1 interface.

Proposal 2: RAN3 should wait for the Xn interface solution to be finalized before discussing impact on the F1 interface.

3 Conclusion

Based on the above discussion, we provide the following observation and proposals.

Observation: Both the UL WUS broadcast status of Cell(s) A and the OD-SIB1 state of the NES cell may experience semi-dynamic changes.

Proposal 1: RAN3 selects option3 as a baseline, as it is better suited to a dynamic energy-saving strategy, especially in cases where the NES cell may require assistance from multiple Cell(s) A , a change in the assistance of any cell A may result in a change to the NES cell's OD-SIB1 state. (The corresponding TP for TS38.423 for option3 is provided in annex).

Proposal 2: RAN3 should wait for the Xn interface solution to be finalized before discussing impact on the F1 interface.

Annex

TP for TS 38.423 for OD-SIB1 (Option3)

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.x.x	WRITE-REPLACE UL WUS Configuration

8.x.x.1	General

The purpose of the Write or replace UL WUS Configuration procedure is to enable an NG-RAN node to write or update UL WUS Configuration to neighbouring cell(s) at other NG-RAN node for the subsequent UL WUS configuration provision.

The procedure uses non UE-associated signalling.

8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Write-Replace, successful operation

The NG-RAN node1 initiates the procedure by sending a WRITE-REPLACE UL WUS REQUEST message to the peer NG-RAN node2, in order to writer or update an UL WUS configuration to one or more cells of the NG-RAN node2. Upon receipt, if supported, the NG-RAN node2 shall store or update the UL WUS configuration for the indicated cells for subsequent UL WUS configuration provision, and respond with the WRITE-REPLACE UL WUS RESPONSE message.

8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Write-Replace, unsuccessful operation

If the NG-RAN node2 cannot assist the broadcasting of the UL WUS configuration in one or more cells indicated in the WRITE-REPLACE UL WUS REQUEST message, it shall respond with the WRITE-REPLACE UL WUS FAILURE message with an appropriate cause value.

8.x.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

8.x.y	UL WUS Configuration Provision

8.x.y.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to start or stop the provisioning of an admitted UL WUS configuration in one or more cells, or to release the corresponding provisioning task and the admitted UL WUS configuration.

The procedure uses non UE-associated signalling.

8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS PROVISION REQUEST message to the peer NG-RAN node2 to start a provisioning, stop a provisioning, of an UL WUS configuration in one or more cells of the NG-RAN node2 , or to release the corresponding provisioning task and the admitted UL WUS configuration. Upon receipt, the NG-RAN node2:

-	shall initiate the requested provisioning according to the parameters given in the request in case the Request Type IE set to "start"; or

-	shall stop the indicated provisioning in case the Request Type IE is set to "stop".

-  shall release the indicated provisioning task and the corresponding stored UL WUS configuration in case the Request Type IE is set to "cancel".

8.x.y.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

8.x.z	UL WUS Provision Failure Indication

8.x.z.1	General

The purpose of the UL WUS Configuration Provision failure indication procedure is to enable an NG-RAN node to inform another NG-RAN node, which initiated the UL WUS configuration provision request, that the ongoing provision has failed in one or more cells.

The procedure uses non UE-associated signalling.

8.x.z.2	Successful Operation

Figure 8.x.z.2-1:UL WUS Configuration Provision Failure indication, successful operation

The NG-RAN node2 initiates the procedure by sending a UL WUS PROVISION FAILURE INDICATION message to the peer NG-RAN node1 to inform that the ongoing UL WUS configuration provision has failed in one or more cells.

x.x.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.x.x.x	WRITE-REPLACE UL WUS REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to write or update UL WUS Configuration for subsequent provision of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. | YES | reject |
| Assisting Cells List |  | 1 |  | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. | YES | reject |
| >Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR_CGI at the NG-RAN node2 | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.x.x.y	WRITE-REPLACE UL WUS RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be stored or updated for the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.x.x.z	WRITE-REPLACE UL WUS FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration provision cannot be supported in the one or more requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.x.x.t	UL WUS PROVISION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to request NG-RAN node2 to start or stop the provisioning of an admitted UL WUS configuration in one or more cells, or to release the corresponding provisioning task and the admitted UL WUS configuration.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Request type |  |  | ENUMERATED (start, stop,cancel, …) |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<<NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

9.x.x.z	UL WUS PROVISION FAILURE INDICATION

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the ongoing UL WUS configuration provision has failed in one or more cells..

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<<END OF CHANGES >>>>>>>>>>>>>>>>>>>>