3GPP TSG-RAN3 Meeting # 126	R3-247580

# Orlando, US,  18-11-2024 – 22-11-2024

Agenda Item:	31.2

Source:	Ericsson, NTT DOCOMO, Nokia, Polaris Wireless

Title:	Discussion on improving E-CID positioning accuracy

Document for:	Discussion, Decision

- Introduction

In this contribution, we detail how the E-CID procedures in NRPPa and F1AP can be extended by including a new measurement report granularity to improve the accuracy of E-CID-based positioning.

# 2	Discussion

In the E-CID positioning method, the position of the UE is estimated with the knowledge of measurements information coming from the UE’s serving cell. Such measurements may include UE measurements aggregated at cell level or measured per SSB or CSI-RS resource, and/or also gNB measurements such as UL-AoA and NR Timing Advance (TA).

Currently, E-CID framework in NRPPa (and F1AP) is designed on the concept of “one cell level measurement”. However within a cell several TRPs can be associated to the same NR CGI and can be deployed with different beam information. It should be possible then to report the gNB’s measurements from several TRPs which could help improving the quality of the E-CID measurement report. For instance, several angles per TRP could be reported in the E-CID measurement report, each with a measurement quality indicator. This would help LMF to decide how to weight-factor the different reported measurements for calculating UE's position.

Observation 1: Current E-CID framework in NRPPa (and F1AP) is designed on the concept of “one cell level measurement”, however one cell can comprise several TRPs which can report different measurements.

To improve E-CID, it should be possible to add TRP level measurements by combining the E-CID’s best effort reporting design (i.e., gNB decides what to report based on what is available) and indicating which TRPs should the gNB report the measurements from, if they are available. A signalling flow is provided below:

Fig. 1: Addition of TRP list in the E-CID Measurement Initiation procedure

Fig.1 shows the signalling in case of on-demand E-CID measurement request. In case of periodical reporting, the E-CID MEASUREMENT REPORT message can also be extended to provide the list of E-CID TRP Measurement Result List.

Proposal 1: To allow for multiple reporting of measurements, it is proposed to add TRP level of measurement in the E-CID Measurement procedures, so that more measurements can be collected from the RAN side and reported to the LMF. This will improve the accuracy of E-CID positioning method.

Regarding the impacted measurements, UL-AoA can differ from one TRP to another. So it is proposed to start with this measurement and discuss if others (e.g., NR TA) could also be affected.

In terms of encoding, to avoid impacts on legacy procedures, where a value of UL-AoA per cell needs to be reported if requested as part of the E-CID Measurement Quantities value, a new IE E-CID TRP Measurement Request Info can be added in the E-CID MEASUREMENT INITIATION REQUEST message. This IE can be defined by a list of E-CID TRP Measurement Quantities IE, which includes the list of measurements requested to be reported per TRP, and a E-CID TRP Measurement Request List containing the list of requested TRPs to measure.

Example below for NRPPa spec of new IE definition:

### 9.2.x1 E-CID TRP Measurement Request Info This information element contains the requested E-CID TRP measurement information. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. E-CID TRP Measurement Quantities; 1. >E-CID TRP Measurement Quantities Item; 1 .. <maxnoMeas>. >>E-CID Measurement Quantities Value; M; ENUMERATED (NR Angle of Arrival, …). E-CID TRP Measurement Request List; 1. >E-CID TRP Measurement Request Item; 1..<maxnoofMeasTRPs>. >>TRP ID; M; 9.2.24. >>Cell ID; O; NR CGI 9.2.9; The Cell ID of the TRP identified by the TRP ID IE.. Range bound; Explanation. maxnoMeas; Maximum no. of measured quantities that can be configured and reported with one message. Value is 64.. maxnoofMeasTRPs; Maximum no. of TRPs that can be included within one message. Value is 64..

Note: LMF can be configured by OAM to know which TRPs are belonging to which cells, or by triggering the TRP Information Exchange procedure.

Observation 2: LMF can be configured by OAM to know which TRPs are belonging to which cells, or by triggering the TRP Information Exchange procedure.

For the reporting, a new E-CID TRP Measurement Result IE can be added in the NRPPa and F1AP E-CID MEASUREMENT INITIATION RESPONSE and E-CID MEASUREMENT INITIATION REPORT messages, which would include a list of results for a list of TRPs and follow similar structure as the E-CID Measurement Result IE (where the list is contained within the Result IE).

Example below for NRPPa spec of how to define such new list:

### 9.2.x1	E-CID TRP Measurement Result

This information element contains the E-CID TRP measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-CID TRP Measurement Response List |  | 1 |  |  |
| >E-CID TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |
| >>E-CID Measured Result Item | M | 1 ..<maxnoMeas> |  |  |
| >>>CHOICE E-CID Measured Results Value | M |  |  |  |
| >>>>UL Angle of Arrival |  |  | 9.2.38 |  |
| >>>Time Stamp | M |  | 9.2.42 | Time Stamp of the corresponding measured result. |
| >>>Measurement Quality | O |  | 9.2.43 | Measurement Quality of the corresponding measured result. |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

Proposal 2: RAN3 to discuss the proposed encoding to improve E-CID requesting multiple TRP measurements, by adding an E-CID TRP Measurement Request Info in the E-CID MEASUREMENT INITIATION REQUEST message, containing two lists of measurements requested, and list od TRPs requested to measure.

Proposal 3: RAN3 to discuss the proposed encoding to improve E-CID reporting multiple TRP measurements, by adding a list of reported E-CID TRP Measurement Result IE containing a list of results per TRP in the E-CID MEASUREMENT INITIATION RESPONSE and E-CID MEASUREMENT INITIATION REPORT messages, in NRPPa and F1AP.

A CR to NRPPa is provided in [1].

A CR to F1AP is provided in [2].

# 3	Conclusions

Observation 1: Current E-CID framework in NRPPa (and F1AP) is designed on the concept of “one cell level measurement”, however one cell can comprise several TRPs which can report different measurements.

Proposal 1: To allow for multiple reporting of measurements, it is proposed to add TRP level of measurement in the E-CID Measurement procedures, so that more measurements can be collected from the RAN side and reported to the LMF. This will improve the accuracy of E-CID positioning method.

Observation 2: LMF can be configured by OAM to know which TRPs are belonging to which cells, or by triggering the TRP Information Exchange procedure.

Proposal 2: RAN3 to discuss the proposed encoding to improve E-CID requesting multiple TRP measurements, by adding an E-CID TRP Measurement Request Info in the E-CID MEASUREMENT INITIATION REQUEST message, containing two lists of measurements requested, and list od TRPs requested to measure.

Proposal 3: RAN3 to discuss the proposed encoding to improve E-CID reporting multiple TRP measurements, by adding a list of reported E-CID TRP Measurement Result IE containing a list of results per TRP in the E-CID MEASUREMENT INITIATION RESPONSE and E-CID MEASUREMENT INITIATION REPORT messages, in NRPPa and F1AP.

# 4	References

- R3-247581, E-CID measurement enhancement, NRPPa CR, Ericsson, DCM, Nokia, Polaris W

- R3-247582, E-CID measurement enhancement, F1AP CR, Ericsson, DCM, Nokia, Polaris W