3GPP TSG-RAN WG3 Meeting #126	R3-247585

Orlando, Florida, U.S., 18th – 22nd November 2024

Agenda Item:	16.2

Source:	Ericsson

Title:	Further aspects on AIoT Architecture

Document for:	Discussion and Approval

# 1	Introduction

This document discusses known, but not yet fully understood/documented and some new aspects on the AIoT architecture aspects.

# 2	Discussion

## 2.1	Topics related to architectural principles

### 2.1.1	Direct versus Indirect Communication of AIOTF with AIoT-RAN for topology 1

For Topology 1, SA2 had produced an “interim conclusion” (whatever that actually means) stating that the newly introduced AIOTF communicates with the BS Reader (i.e. AIoT-RAN) either directly or via an AMF, to trigger e.g., inventory request and command request).

Disregarding the rather confusing terminology “BS Reader”, we would like to discuss which kind of consequences such a conclusion would have, if its status changes from “interim” to “final” or “for real”:

Given the current status of agreements / interim conclusions in SA2 and RAN3

1.	The signalling transport for XXAP at the AIoT RAN node is SCTP/IP (TR 38.769 [1])

2.-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality ... with the following features for topology 1: Communicate with BS Reader (i.e. AIoT RAN), either directly or via an AMF, to trigger e.g., inventory request and command request). (TR 23.700-13 [2])

3.-	When a BS Reader and the AIoTF communicate indirectly via an AMF ... NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services. (TR 23.700-13 [2])

We can see that we are potentially only one step away from having the protocol stack defined for topology 2 in its entirety, i.e. we are missing a developed view on which protocol to use between the AIoT CN (to still speak in RAN3 terms) and the AIoT RAN in case of direct connectivity (without an intermediate AMF).

But let us first consider on how an NG-RAN node is supposed to connect to an AMF:

-	The NG-RAN node is configured with basic parameters defining which information to broadcast in its cells: Cell-ID (with the node id being part of it), Tracking Area and PLMN ID (not going into any details regarding e.g. slicing and shared network details and others).

-	Further the NG-RAN node is configured with information on how to connect to the 5GC, i.e. IP addresses to AMFs towards which a signalling association shall be established.

-	The successful exchange of application-level information between the NG-RAN node and the AMF(s) by means of NGAP NG Setup procedure (success defined by a situation where at least one PLMN being supported by both, NG-RAN and the AMF) establishes a working NG-C interface instance, ready to take up user traffic.

In case of indirect communication between the AIoT RAN and the AIOTF for topology 1, we would expect the AIoT RAN node (which could be an NG-RAN node already supporting “legacy” 5G services) to connect to the AMF by means of the NGAP NG Setup procedure.

Looking at the protocol features of the NGAP NG Setup procedure, by which means an NG-RAN node “announces itself” towards an AMF, one can see that an AIoT RAN node would need to provide

-	A Global RAN Node ID, supported TAs (TAC+PLMN(s)) and a Default Paging DRX.

With the possibility to indicate AIoT support (e.g. by means of defining a new Global RAN Node ID for AIoT), to define a “virtual” TA for AIoT purposes and ignoring Default Paging DRX for AIoT purposes, it can be seen that it is not “impossible” to re-use the NGAP NG Setup procedure.

The same for the response from the AMF, which requires the indication of an AMF name, a served GUAMI List, A Relative AMF Capacity and a PLMN Support List.

Observation 1.1.-1:	For topology 1, for indirect communication of the AIOTF with the AIoT RAN node via the AMF, it is possible to re-use the NGAP NG Setup procedure: introducing the possibility to “negotiate” the support of AIoT Such re-use seems to require the possibility to define under which circumstance certain mandatory IEs are either to be ignored (e.g. Default Paging DRX) or would need to be regarded (from RAN p.o.v.) as “virtual” parameters (e.g. supported TAC).

If it is possible to re-use the NGAP NG Setup procedure, and, by that, other NG-C interface management procedure, in case of indirect communication (just think of an AIoT RAN node only supporting AIoT radio), then it should be also possible to support direct communication with the AIOTF by NGAP protocol means. And indeed, we would regard direct communication with the AIOTF from an AIoT RAN point of view as being equivalent to communicating via the AMF, so the AIoT RAN would see an AIOTF as just another “sort of AMF”.

Observation 1.1.-2:	For topology 1, for direct communication of the AIOTF with the AIoT RAN node, it is possible to define communication of the AIoT RAN node with the AIOTF by means of modified NGAP. For interface management procedures, it is possible to apply the same modifications to NGAP as for the indirect communication.

Observation 1.1.-3:	For topology 1, from an AIoT RAN node point of view, applying direct communication with the AIOTF would appear like communicating with an “AIoT enabled” AMF.

If an AIoT RAN node would not recognise any difference as to whether the 5GC applies direct or indirect communication towards the AIOTF, respective AIoT specific NGAP modifications should be agnostic towards the communication option applied.

Observation 1.1.-4:	For topology 1, if NGAP is applied irrespective the communication option, an AIoT RAN node would (and should) not recognise any difference as to whether direct or indirect communication is applied. AIoT specific modifications of NGAP should be agnostic with respect to the communication option.

Proposal 1.1-5:	For topology 1, agree to extend NGAP protocol functions for support of topology 1.

Proposal 1.1-6:	For topology 2, agree to extend NGAP protocol functions in an agnostic way as to whether direct or indirect communication is applied between the AIoT RAN node and the AIOTF.

### 2.1.2	Nature of communication paths in various topology options

At RAN3#125bis we made several attempts to put into figure and words the properties of various topology options, but didn’t yet achieve to create reasonable input for TR 38.769 for all options in all necessary details.

Following an attempt to take the agreed architecture as a starting point protocol stacks were drafted to depict for topology 2 the 3 options, nick named “RRC”, “NAS”, “UP”. The main difference between the RRC option as compared to the NAS and UP options is the conveyance of “AIoT upper layer information” to the AIoT enabled UE (“UE Reader” for short). While the RRC option foresees that the AIoT enabled gNB is aware that transport of “AIoT upper layer information” is ongoing by terminating the communication stemming from the AIoT CN, this is not the case with the NAS or UP option.

Observation 1.2-1:	Per definitions of the topology 2 options in TR 38.769 [1] , only the RRC option foresees the AIoT enabled gNB to be aware when AIoT upper layer information is conveyed along with details about its nature necessary to execute respective AIoT RAN node functions; 
in case of the NAS or UP option, the AIoT enabled gNB is not aware when AIoT upper layer information is conveyed and can therefore also not deduce the nature of such information.

All topology 2 options (are assumed to) try to achieve the same result in terms of executing an AIoT transaction as requested by AIoT CN, i.e. provide all involved entities with necessary information in order to execute AIoT functions towards the AIoT devices targeted by an AIoT transaction, and, per RAN3 agreement, to enable the coordination of the usage of the A-IoT radio resources among readers, for both T1 and T2.

One important aspect not yet sufficiently outlined in discussions is the scope of communication in various topology options, i.e. in case of the topology 2 options, whether the recipient of a communication item is a single AIoT enabled UE/UE reader (in case of direct communication between AIOTF and UE Reader) or whether the AIoT enabled gNB duplicates a communication request towards multiple UE readers (as of the indicated area within which an AIoT transaction should take place.)

Observation 1.2-2:	Different topology 2 options (are assumed to) to try to achieve the same result in terms of executing an AIoT transaction as requested by AIoT CN. One important aspect not yet sufficiently outlined in discussions is the scope of information provision in various topologies/options, i.e. whether on a certain communication path the information is destined for a single or multiple readers.

In order to depict and finally to compare different options, we will look into details of topology 1 and the various topology 2 options in the following:

#### 2.1.2.1	General explanations on depiction of communication paths

The RAN3 architectural entities are used: AIoT RAN (short for for AIoT enabled gNB in T2), AIoT CN, UE Reader (short for AIoT enabled UE), for T2/UP also the UPF is shown

In addition, where appropriate, assuming direct and indirect connectivity between AIOTF and AIoT RAN as discussed in SA2 applies for T1 and T2 alike, AMF, AIOTF and SMF are shown for illustrative purposes.

The following elements for communication are depicted:

-	AIoT upper layer information: this item denotes communication means to transport AIoT transaction information to the AIoT device via the readers; for T2 it also contains control information for the AIoT transaction destined for the AIoT enabled UE.

-	Note, that content and scope of this communication item will differ, depending on the option and the communication path it takes. Note also, that from an AIoT RAN this information may be transparent or contains explicit information to be processed by AIoT RAN.

-	AIoT session resource control: this item denotes communication means to trigger allocation/control of AIoT radio resources by the AIoT RAN

-	AIoT radio resource control: this item denotes protocol means for configuring the AIoT enabled UE for usage of AIoT radio resources.

#### 2.1.2.2	Topology 1

The figure below gives a starting point of the discussions, depicting the communication path between the AIoT CN and the AIoT RAN. The intended purpose of the AIoT CN triggered communication is to request the AIoT RAN to perform allocation and coordination of AIoT radio resources and to convey AIoT upper layer information to the targeted AIoT devices.

Figure 2.1.2-1.1: Communication paths for Topology 1 (RAN3 agreed architectural entities)

The scope of the AIoT RANAIoT CN communication is on a per AIoT transaction basis.

Connectivity options (direct from AIOTF or via AMF) do not reveal additional insights for the purpose of this discussion.

#### 2.1.2.3	Topology 2 - RRC

Topology 2 options bear the need to consider the relation between AIoT RANAIoT CN and UE ReaderAIoT RAN communication.

There is no difference in terms of the AIoT RANAIoT CN communication between topology 1 and topology 2/RRC and it can be expected that respective NG/XX protocol functions are equivalent.

AIoT RAN duplicates/relays AIoT upper layer information towards the UE Readers involved in the AIoT transaction and is in an excellent position to coordinate the usage of AIoT radio resources among by the involved UE Readers.

Figure 2.1.2-2.RRC.1: Communication paths for Topology 2 – RRC (RAN3 agreed architectural entities)

Assumed connectivity options (direct from AIOTF or via AMF) do not reveal additional insights for the purpose of this discussion.

#### 2.1.2.4	Topology 2 – NAS – AIoT CN triggered AIoT resource control

While topology 1 and topology 2/RRC is quite straight forward and resembles existing RAN-CN signalling for dedicated (MBB) radio resources, like NG signalling for PDU Session Resource control, the guiding principle of the NAS option is to allow direct communication between the AIoT enabled UE and the AIoT CN. This is depicted by the red dot-and-dash line, going transparently through the AIoT RAN node.

Following this principle of course comes with a cost. As The RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers, for both T1 and T2 there is the necessity to trigger AIoT radio resource allocation (for a particular UE Reader) and coordination (among many UE Readers). Consequently, AIoT RAN would need receive an explicit trigger, in this case, following the principle of triggering resource setup always by the network, the trigger comes from the AIoT CN.

For the AIoT RAN to understand how to allocate/coordinate AIoT radio resources the information contained within the AIoT session resource control would need to – at least partly – duplicate information contained in the AIoT upper layer information.

It is expected that the AIoT upper layer information is signalled within legacy NG/RRC NAS transport procedures, but there is no processing performed in AIoT RAN apart from relaying a NAS PDU.

Figure 2.1.2-2.NAS/CN.1: Communication paths for Topology 2- NAS – CN triggered AIOT session resource control (RAN3 agreed architectural entities)

Looking into the “SA2” view of things, one can see that nothing dramatic changes, apart from the fact, that the AIoT upper layer information is also transparent to the AMF, which would, like the AIoT RAN, only relay information provided by the AIOTF.

The costly aspect of that approach – as compared to the T2/RRC option – is that within the AIoT CN, AIOTF would need to “feed” AIoT upper layer information on per UE Reader basis, so AIoT CN would first need to gain awareness of the involved UE Readers in the AIOTF (which is not necessary in the T1 and T2/RRC option) so a per AIoT transaction AIoT session resource control needs to be issued towards AIoT RAN, which has also to find out and activate UE Readers (and probably other AIoT radio equipment, like e.g. equipment just providing the CW) which are necessary to perform the AIoT transaction within the requested area.

This require respective information to be passed on the per-UE Reader signalling the AIoT session resource signalling in order to allow association at the UE Reader.

Figure 2.1.2-2.NAS/CN.2: Communication paths for Topology 2- NAS – CN triggered AIoT session resource control (decomposed 5GC, connectivity options)

Observation 1.2-3:	The guiding principle of the Topology 2/NAS option is to enable direct communication between the AIoT enabled UE and the AIoT CN. This comes to the cost of 
1) establishing per UE Reader communications paths from the AIoT CN, with the necessity of the AIOTF being aware of the involved This is depicted by the red dot-and-dash line, going transparently through the AIoT RAN nodethe Different topology 2 options (are assumed to) to try to achieve the same result in terms of executing an AIoT transaction as requested by AIoT CN. One important aspect not yet sufficiently outlined in discussions is the scope of information provision in various topologies/options, i.e. whether on a certain communication path the information is destined for a single or multiple readers.

#### 2.1.2.5	Topology 2 – NAS – AIoT enabled UE triggered AIoT resource control

The UE triggered AIoT resource control represents an attempt to escape the need for AIoT CN triggered AIoT session resource control and “transpose” such information provision to the UE Reader side.

This means that the per AIoT session resource control is replaced by a per UE Reader AIoT session resource control, resulting in a situation where the AIoT RAN would first needs to learn about the requested AIoT transaction gradually by means of collection information from all the UE Readers contacting the AIoT RAN.

By that, the effectiveness of per AIoT transaction communication is completely lost (if one compares it to the T2/RRC solution), as both, the AIoT upper layer information and the AIoT session resource request is performed on a per UE Reader basis.

By that also the AIoT RAN would lose a huge part of its role in radio resource allocation and control, and would bring the network side into a situation which should not be further pursued,

- as the AIoT RAN, being responsible for resource control and coordination for the UE Reader would be dependent from that very entity (the UE Reader) to which RAN is assumed to grant resources.

- as AIoT RAN would be dependent on the AIoT CN selection of UE Readers w/o any possibility to influence the AIoT CN’s selection of UE Readers.

Of course, there are ways of amending this situation with work arounds, but the main design error lies already in the paradigm of introducing an end-to-end communication for AIoT upper layer information transparent to AIoT RAN.

Figure 2.1.2-2.NAS/UE.1: Communication paths for Topology 2- NAS – UE Reader triggered AIoT session resource control (RAN3 agreed architectural entities)

The following figure only depicts the fact that like for the CN triggered approach, the AIoT CN consists of an AMF and AIOTF, with the necessity of the AIOTF to get hold of UE Readers topology for performing an AIoT transaction in order to feed AIoT upper layer information into NAS signalling on a per UE Reader basis.

Figure 2.1.2-2.NAS/UE.2: Communication paths for Topology 2- NAS – UE triggered AIoT session resource control (decomposed 5GC, indirect connectivity option)

Observation 1.2.-4:	The guiding principle of the Topology 2/NAS option is to enable direct communication between the AIoT enabled UE and the AIoT CN. This comes to the cost of 
1) establishing per UE Reader communications paths from the AIoT CN, with the necessity of the AIOTF being aware of the involved UE Readers
2) enabling the AIoT RAN to allocate and coordinate the usage of AIoT radio resources for and among the involved UE Readers, which requires
2a) either additional per AIoT transaction communication between the AIoT CN and AIoT RAN
2b) or additional per UE Reader communication with AIoT RAN, with AIoT RAN to coordinate the UE Readers’ requests.

Observation 1.2-5:	While the CN triggered AIoT session resource control for the NAS option might represent a theoretical possibility, the AIoT enabled UE resource control should not be further pursued, as it would change basic system and protocol principles, with the RAN not being in full control and receiving timely complete session information from the CN and w/o the possibility to modify the involved UE Readers.

#### 2.1.2.6	Topology 2 – UP – AIoT CN triggered AIoT resource control

The “UP” option for Topology 2 differs from the “NAS” option in terms of even increased complexity. While the guiding principle to keep AIoT RAN “blind” against control information exchanged directly between the AIoT CN and the UE Reader, this option also requires the AIoT CN to associate a per UE Reader communication path with UP resources for a PDU Session established to deliver AIoT upper layer information.

Figure 2.1.2-2.UP/CN.1: Communication paths for Topology 2 – UP – CN triggered AIoT session resource control (RAN3 agreed architectural entities)

Although probably an SA2 item, it would be interesting to know, whether SA2 spent a single thought on the complexity of associating the PDU session (UP) resources with the per-UE-Reader communication originated at the AIoTF – when the respective “interim conclusion” was captured in [2].

Figure 2.1.2-2.UP/CN.2: Communication paths for Topology 2 – UP – CN triggered AIoT session resource control (decomposed 5GC, indirect connectivity option)

#### 2.1.2.7	Topology 2 – UP – AIoT enabled UE triggered AIoT resource control

The AIoT enabled UE triggered AIoT resource control for the “UP” option does not reveal any additional insight as compared to the discussion already lead for the NAS and the UP option above. The respective  communication scheme is depicted below and is self-explanatory:

Figure 2.1.2-2.UP/UE.1: Communication paths for Topology 2 – UP – AIoT enabled UE triggered AIoT session resource control (RAN3 agreed architectural entities)

Observation 1.2-6:	The guiding principle of the Topology 2/UP option is, like for the NAS option, to enable direct communication between the AIoT enabled UE and the AIoT CN. The cost for this option is as the cost for the NAS option, with the assumed additional cost for involving PDU session UP entities.

Finally the following is proposed:

Proposal 1.2-7:	It is proposed to capture the discussion on section 2.1.2 in the TR.

Proposal 1.2-8:	It is proposed to abstain from architectural approaches foreseeing the UE Reader to trigger AIoT radio resource allocation.

### 2.1.3	Support of mixed deployments – AIoT RAN node supporting topology 1 and topology 2

At RAN3#125bis we made the following agreement:

An AIoT enabled gNB could support both topologies, this is an implementation matter.

Putting this agreement into perspective of the proposed approach for topology 1, i.e. to support modification of NGAP in order to enable communication between an AIoT RAN node supporting both topologies and the AIoT CN (i.e. an AIoT enabled 5GC), we can raise the question whether such AIoT RAN node (i.e. an AIoT enabled gNB) would connect to the AIoT CN/AIoT enabled 5GC in a different way for topology 2 as compared to topology 1.

Observation 1.3-1:	In order to answer whether mixed deployments are supportable in an efficient way, the nature of necessary communication paths would need to be compared between topology 1 and the topology 2 options.

The discussions on the “nature of communication paths” in various topology 2 options in the previous chapter have shown, that the guiding principle of the NAS and the UP option, i.e. establishing a direction communication path between the UE Reader and the AIOT CN comes along with quite some costs in complexity and unnecessary additional signalling for enabling AIoT radio resource control by the AIoT RAN.

If – for once – simplicity and robustness would be the guiding principles for a decision made by 3GPP, aligning topology 2 with topology 1 in terms of RAN-CN communication would come along with comparable complexity and robustness from UE Reader point of view.

In addition, choosing a topology 2 option which is “compatible” with topology 1 solution would allow the RAN-CN communication – to a certain extent – topology-agnostic (aspects of paging UE Readers in RRC_INACTIVE/IDLE would need to be considered).

Observation 1.3-2:	It is apparent that the topology 2/RRC option is the most simplest and robust option and that the direct communication between the UE Reader and the AIoT CN comes with high costs in complexity. In addition, the topology 2/RRC option is “compatible” with the topology 1 option and allows straight forward deployment both topologies within a single AIoT RAN node.

### 2.1.4	Re-attempt to draw yet missing protocol stacks topology 2 options

Once a deeper understanding of the various options is established, we can start an attempt to draw agreeable and protocol stacks for the NAS and UP option, having in mind that protocol stacks alone are not able to cover all aspects of the various approaches.

#### 2.1.4.1	Topology 2 – NAS option – protocol stack

Two communication paths on the RAN-CN interface as discussed in section 2.1.2 are necessary:

1)	transparent provision of AIoT upper layer information to the AIoT enabled UE via the AIoT enabled gNB: A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s DL/UL NAS packets, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport procedures over NGAP.

2)	execution of AIoT session resource control between the AIoT CN and the AIoT enabled gNB by means of new signalling.

Figure 2.1.4-NAS.1: Protocol stack for NAS based solution of Topology 2 – provision of AIoT upper layer information

Figure 2.1.4-NAS.2: Protocol Stack for topology 2 –NAS based solution of Topology 2 – execution of AIoT session resource control

Figure 2.1.4-NAS.1 assumes an “XXAP*” protocol, which, together with the assumed XXAP protocol functions for the AIoT session resource control results in the same set of protocol functions as the assumed XXAP protocol functions for the RRC option:

-	UE Reader specific protocol functions contained in XXAP for the RRC option (see Figure 6.4.2.1.1-1 in [1]) are replicated in XXAP* for the NAS option

-	AIoT session resource control specific functions contained in XXAP for the RRC option can be found in XXAP for the NAS option (see Figure 2.1.3-NAS.2).

-	Due to the disassociation of communication paths of the NAS option, which is not given for the RRC option, additional protocol functions are needed in order to associate RRC, NAS and RAN-CN signalling to the same AIoT transaction.

Whether the duplication of protocol functions for the NAS option in XXAP and XXAP* is desirable, or at least calls for a single point of specification (assuming that normative work is agreed) needs further consideration.

Observation 1.4-1:	The depiction and description of the NAS option in section 2.1.4 along with discussions led in section 2.1.2, provide sufficient justification and a nice foundation for inclusion in the RAN3 parts of TR 38.769 [1].

#### 2.1.4.2	Topology 2 – UP option – protocol stack

The communication paths on the RAN-CN interface as discussed in section 2.1.2 is 2-fold:

1)	transparent provision of AIoT upper layer information to the AIoT enabled UE via a protocol entity (“AIoT control(XXAP*)”) that is signalled inband through PDU Session resources (NG-U GTP-U tunnel / DRB) to the AIoT enabled UE.

2)	execution of AIoT session resource control between the AIoT CN and the AIoT enabled gNB by means of new signalling.

Figure 2.1.4-UP.1: Protocol stack for NAS based solution of Topology 2 – provision of AIoT upper layer information

Figure 2.1.4-UP.2: Protocol Stack for topology 2 –NAS based solution of Topology 2 – execution of AIoT session resource control

Figure 2.1.4-UP.1 assumes an “XXAP*” protocol, which, together with the assumed XXAP protocol functions for the AIoT session resource control results in the same set of protocol functions as the assumed XXAP protocol functions for the RRC option:

-	UE Reader specific protocol functions contained in XXAP for the RRC option (see Figure 6.4.2.1.1-1 in [1]) are replicated in XXAP* for the NAS option

-	AIoT session resource control specific functions contained in XXAP for the RRC option can be found in XXAP for the NAS option (see Figure 2.1.3-UP.2).

-	Due to the disassociation of communication paths of the NAS option, which is not given for the RRC option, additional protocol functions are needed in order to associate RRC, NAS and RAN-CN signalling to the same AIoT transaction.

Whether the duplication of protocol functions for the UP option in XXAP and XXAP* is desirable, or at least calls for a single point of specification (assuming that normative work is agreed) needs further consideration.

Observation 1.4-2:	Also the depiction and description of the UP option in section 2.1.4 along with discussions led in section 2.1.2, provide sufficient justification and a nice foundation for inclusion in the RAN3 parts of TR 38.769 [1].

Proposal 1.4-2:	Capture protocol stack depictions for the NAS and UP option as shown in section 2.1.4 in TR 38.769 [1].

## 2.2	Further topics

### 2.2.1	Reader Selection

3GPP specified systems foresee a clear functional split between RAN and CN. Such functional division in conjunction with the specification of the respective RAN-CN interface proofed to be a success factor of 3GPP systems.

Readers, irrespective of the applied topology, are undoubtedly part of the RAN ecosystem. They are part of the deployed radio infrastructure and we have agreed that respective AIoT resource allocation and co-ordination needs the supervision of the AIoT RAN.

While, for MBB deployments, the CN may be in possession of RAN internal information down to cell-level, this information is either used as assistance information (e.g. paging optimisation) or it is used as means to express geographical (area) information to indicate (e.g. for NR MBS) service area information (for NTN we have also introduced the possibility to define “mapped” TA/cell information, which do not relate to any “physical” entity but only geographical information).

Observation 2.1-1:	While RAN may receive cell-level assistance information or geographically connotated cell-level service area information for legacy (MBB) features, it is expected that the same principles are also kept for AIoT, retaining the fundamental functional split between RAN and CN, that RAN is ultimately responsible for determining the appropriate radio resources for supporting the requested service.

The above-mentioned functional division results in keeping any kind of topological information within RAN, e.g. despite of cell-level assistance/service area information, RAN may decide to serve a UE on a different carrier, apply CA or DC. The same principle is expected to be applicable also for AIoT, where RAN internal topology may need considerations of e.g. bi-static configurations etc.

Proposal 2.1-2:	Agree that CN may provide AIoT service/transaction area and assistance information for an AIoT transaction to RAN, but RAN is the entity to finally determine the set of readers to perform the requested service.

### 2.2.2	Related to latest SA2 “interim” conclusions

There are certain aspects in the latest SA2 “interim” conclusions, that produced a bit of surprise, given the obvious lack of detailed discussions that led to such study results.

The most astonishing part of these results is the proclamation that for topology 2 two options will be specified. The user plane option and the RRC option, with the NAS option still FFS.

As could be expected, the definition of both options is (currently) confined to 2 figures, an “architecture” and a “protocol stack”. As figures are obviously able to tell more than 1000 pages of descriptions, it appears that the conclusion is deemed to be made in a conscious way on stable ground.

Observation 2.2-1:	SA2 interim conclusions on performing normative work for at least 2 out of options for topology 2, a single meeting after having been identified by RAN3, may be seen by some as the proof of 3GPP work groups’ discussion effectiveness, by others as the continuation of 5G’s decision culture, to rather decide multiple options than spending effort on a thorough examination allowing necessary time for discussions.

Proposal 2.2-2	It is propose to seriously consider whether hypothetical normative work should be really carried out for multiple topology 2 options.

### 2.2.3	Related to latest RAN2 agreements

RAN2 made some interesting agreements at the last meeting where we would like to point out one set in particular related to topology 2, which was generated partly while discussion (temporarily) out of coverage UE readers:

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

We interpret the first agreement as a RAN2 discussion result regarding pre-determined AIoT radio resources, that e.g. might be communicated by OAM means to the UE Reader(s), as assumed in one of the recently discussed and captured text for the TR 38.769 [1].

Given the discussion on AIoT radio configuration by the AIoT enabled gNB triggered by the AIoT enabled UE in section 2.1.2, we would also suggest to alter the respective text in section 6.5.1.3 respectively.

Proposal 2.3-1:	It is proposed to remove aspects in TR 38.769 §6.5.1.3 concerning UE triggered AIoT resource allocation and AIoT radio allocation preconfigured by OAM.

### 2.2.4	DO-A aspects (Device originated autonomous)

The focus for traffic types in the SID scope is DO-DTT and DT so far. It is, however, mentioned in the SID [1] that:

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

For DO-A use cases, a device typically triggers an UL transmission autonomously. Whereas for DO-DTT and DT use cases, for any UL transmission of devices, the network needs to send a triggering message beforehand which may comprise resource assignment information based on which the devices can perform UL transmissions either in a backscatter manner or actively generated by the device.

It is assumed that DO-A traffic (sensory data) typically has stricter end-to-end latency requirement compared to non DO-A use cases. This means devices delivering DO-A traffic cannot wait too long for the network to trigger them by means of (periodic) inventory/command rounds so that the device can deliver pending data within the latency requirements (in that case the network triggered AIoT transaction would need to be performed by the network way too often just to meet the tight latency requirements). RAN2 will/might study how it is possible to inform devices (most likely on a regularly basis, similar to SIB1) which AIoT resources to use for DO-A traffic, so that collisions stemming from other devices/traffic can be coped with.

From RAN3 perspective, i.e. on the XX interface, DO-A traffic characteristics are expected to be as follows:

-	DO-A traffic is not triggered by the CN

-	DO-A traffic would typically be (required to be) delivered immediately from RAN to the CN without any optimizing accumulation of DO-A traffic from other devices, resulting in a single-device transaction on XX.

-	Assuming no interaction with CN initiated AIoT procedures, providing data from the device in DO-A fashion would require just an UL/D2R communication followed by an acknowledgment of the network in DL/R2D direction, some aspects appear to be similar to SDT.

-	Such device triggered DO-A procedure on XX will not need to be identified by a CN allocated transaction ID, but rather with a device ID.

-	After the successful execution of such simple 2-step communication, the DO-A procedure would terminate successfully.

Observation 2.4-1:	Latency requirement for DO-A traffic types are assumed to be more strict than for DO-TT traffic.

Proposal 2.4-2:		Agree to study at least DO-A traffic characteristics from RAN3 (XX interface) point of view and capture basic findings in the RAN TR [1].

# 3	Conclusion and Proposals

We have discussed several architecture aspects and observed and propose the following:

#### On connectivity the (AIoT/5G) CN

Observation 1.1.-1:	For topology 1, for indirect communication of the AIOTF with the AIoT RAN node via the AMF, it is possible to re-use the NGAP NG Setup procedure: introducing the possibility to “negotiate” the support of AIoT Such re-use seems to require the possibility to define under which circumstance certain mandatory IEs are either to be ignored (e.g. Default Paging DRX) or would need to be regarded (from RAN p.o.v.) as “virtual” parameters (e.g. supported TAC).

Observation 1.1.-2:	For topology 1, for direct communication of the AIOTF with the AIoT RAN node, it is possible to define communication of the AIoT RAN node with the AIOTF by means of modified NGAP. For interface management procedures, it is possible to apply the same modifications to NGAP as for the indirect communication.

Observation 1.1.-3:	For topology 1, from an AIoT RAN node point of view, applying direct communication with the AIOTF would appear like communicating with an “AIoT enabled” AMF.

Observation 1.1.-4:	For topology 1, if NGAP is applied irrespective the communication option, an AIoT RAN node would (and should) not recognise any difference as to whether direct or indirect communication is applied. AIoT specific modifications of NGAP should be agnostic with respect to the communication option.

Proposal 1.1-5:	For topology 1, agree to extend NGAP protocol functions for support of topology 1.

Proposal 1.1-6:	For topology 2, agree to extend NGAP protocol functions in an agnostic way as to whether direct or indirect communication is applied between the AIoT RAN node and the AIOTF.

#### On the nature of communication paths in various topologies and options

Observation 1.2-1:	Per definitions of the topology 2 options in TR 38.769 [1] , only the RRC option foresees the AIoT enabled gNB to be aware when AIoT upper layer information is conveyed along with details about its nature necessary to execute respective AIoT RAN node functions; 
in case of the NAS or UP option, the AIoT enabled gNB is not aware when AIoT upper layer information is conveyed and can therefore also not deduce the nature of such information.

Observation 1.2-2:	Different topology 2 options (are assumed to) to try to achieve the same result in terms of executing an AIoT transaction as requested by AIoT CN. One important aspect not yet sufficiently outlined in discussions is the scope of information provision in various topologies/options, i.e. whether on a certain communication path the information is destined for a single or multiple readers.

Observation 1.2-3:	The guiding principle of the Topology 2/NAS option is to enable direct communication between the AIoT enabled UE and the AIoT CN. This comes to the cost of 
1) establishing per UE Reader communications paths from the AIoT CN, with the necessity of the AIOTF being aware of the involved This is depicted by the red dot-and-dash line, going transparently through the AIoT RAN nodethe Different topology 2 options (are assumed to) to try to achieve the same result in terms of executing an AIoT transaction as requested by AIoT CN. One important aspect not yet sufficiently outlined in discussions is the scope of information provision in various topologies/options, i.e. whether on a certain communication path the information is destined for a single or multiple readers.

Observation 1.2.-4:	The guiding principle of the Topology 2/NAS option is to enable direct communication between the AIoT enabled UE and the AIoT CN. This comes to the cost of 
1) establishing per UE Reader communications paths from the AIoT CN, with the necessity of the AIOTF being aware of the involved UE Readers
2) enabling the AIoT RAN to allocate and coordinate the usage of AIoT radio resources for and among the involved UE Readers, which requires
2a) either additional per AIoT transaction communication between the AIoT CN and AIoT RAN
2b) or additional per UE Reader communication with AIoT RAN, with AIoT RAN to coordinate the UE Readers’ requests.

Observation 1.2-5:	While the CN triggered AIoT session resource control for the NAS option might represent a theoretical possibility, the AIoT enabled UE resource control should not be further pursued, as it would change basic system and protocol principles, with the RAN not being in full control and receiving timely complete session information from the CN and w/o the possibility to modify the involved UE Readers.

Observation 1.2-6:	The guiding principle of the Topology 2/UP option is, like for the NAS option, to enable direct communication between the AIoT enabled UE and the AIoT CN. The cost for this option is as the cost for the NAS option, with the assumed additional cost for involving PDU session UP entities.

Proposal 1.2-7:	It is proposed to capture the discussion on section 2.1.2 in the TR.

Proposal 1.2-8:	It is proposed to abstain from architectural approaches foreseeing the UE Reader to trigger AIoT radio resource allocation.

#### On support of mixed deployments

Observation 1.3-1:	In order to answer whether mixed deployments are supportable in an efficient way, the nature of necessary communication paths would need to be compared between topology 1 and the topology 2 options.

Observation 1.3-2:	It is apparent that the topology 2/RRC option is the most simplest and robust option and that the direct communication between the UE Reader and the AIoT CN comes with high costs in complexity. In addition, the topology 2/RRC option is “compatible” with the topology 1 option and allows straight forward deployment both topologies within a single AIoT RAN node.

#### On protocol stacks

Observation 1.4-1:	The depiction and description of the NAS option in section 2.1.4 along with discussions led in section 2.1.2, provide sufficient justification and a nice foundation for inclusion in the RAN3 parts of TR 38.769 [1].

Observation 1.4-2:	Also the depiction and description of the UP option in section 2.1.4 along with discussions led in section 2.1.2, provide sufficient justification and a nice foundation for inclusion in the RAN3 parts of TR 38.769 [1].

Proposal 1.4-2:	Capture protocol stack depictions for the NAS and UP option as shown in section 2.1.4 in TR 38.769 [1].

#### On Reader selection

Observation 2.1-1:	While RAN may receive cell-level assistance information or geographically connotated cell-level service area information for legacy (MBB) features, it is expected that the same principles are also kept for AIoT, retaining the fundamental functional split between RAN and CN, that RAN is ultimately responsible for determining the appropriate radio resources for supporting the requested service.

Proposal 2.1-2:	Agree that CN may provide AIoT service/transaction area and assistance information for an AIoT transaction to RAN, but RAN is the entity to finally determine the set of readers to perform the requested service.

#### On SA2 “interim” conclusions

Observation 2.2-1:	SA2 interim conclusions on performing normative work for at least 2 out of options for topology 2, a single meeting after having been identified by RAN3, may be seen by some as the proof of 3GPP work groups’ discussion effectiveness, by others as the continuation of 5G’s decision culture, to rather decide multiple options than spending effort on a thorough examination allowing necessary time for discussions.

Proposal 2.2-2	It is propose to seriously consider whether hypothetical normative work should be really carried out for multiple topology 2 options.

#### On RAN2 agreements

Proposal 2.3-1:	It is proposed to remove aspects in TR 38.769 §6.5.1.3 concerning UE triggered AIoT resource allocation and AIoT radio allocation preconfigured by OAM.

#### On DO-A aspects

Observation 2.4-1:	Latency requirement for DO-A traffic types are assumed to be more strict than for DO-TT traffic.

Proposal 2.4-2:		Agree to study at least DO-A traffic characteristics from RAN3 (XX interface) point of view and capture basic findings in the RAN TR [1] .

#### Final proposal

Final Proposal:	Capture in the RAN TR 38.769 conclusive content as provided in R3-247586 [3].

# 4	References

[1]	TR 38.769 "Study on solutions for ambient IoT (Internet of Things)", latest version with applied agreed text from RAN3#125bis

[2]	TR 23.700-13 "Study on Architecture support of Ambient power-enabled Internet of Things", latest version

[3]	R3-247586 "[TP for TR 38.769] on AIoT Architecture aspects", input paper for RAN3#126