3GPP TSG-RAN WG3 Meeting #126	R3-247609

# Orlando, USA, November 18 – 22, 2024

Agenda item:	15.2

Source: 	Samsung

Title: 	Discussion on support of regenerative payload for IoT NTN

Document for:	Discussion and decision

Introduction

Last meeting primarily discussed the support of regenerative payload for IoT NTN, and the following agreement is achieved,

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

Check progress in RAN1/2…

In this contribution, we continue to discuss the support of regenerative payload for IoT NTN.

Discussion

The first open issue is about whether the eNB only onboard architecture can be further considered.

In the latest version of TS 23.700-29, the conclusion has drawn by SA2 that to support the S&F satellite operation for regenerative payload, only the full CN onboard architecture and split MME architecture are further considered and worked on. So we see no motivation to continue to discuss the support of eNB only onboard architecture in this release.

Proposal 1: The eNB only onboard architecture to support the S&F satellite operation is not discussed in this release.

As indicated by the WID in RP-234077, the support of S&F satellite operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure. And a typical scenario can be illustrated as follows,

Assuming the full eNB is on board the satellite, at time instance T1, the satellite can provide the service link with the UE without the feeder link. After obtaining UL data from UE which is to be transmitted to the CN, the Satellite needs to store such UL data during T1->T2->T3. At time instance T3, when the satellite establishes the feeder link with the NTN GW, such UL data from UE can be further transmitted to the CN for further operations.

It should be noted that in TR 23.700-29, SA2 has discussed the possibility of the support of S&F satellite operation with both single satellite deployment and multiple satellite deployment. More specifically, for single satellite deployment, only one regenerative payload serves a specific UE; while for multiple satellite deployment, multiple regenerative payloads serve a specific UE. Consequently, RAN3 can also consider both scenarios with single satellite deployment and multiple satellite deployment.

According to the discussion last time, operators are quite interested in the multiple satellite deployment, depending on the fact that the regression time of some satellite will be as long as 12 hours. Although the service is non real time and delay tolerant, it is still too long for a UE to perform MO/MT data transmission with such long regression time satellite with full CN/split MME onboard. So it is reasonable to consider both single and multiple satellite deployment to support S&F operation for a UE.

Proposal 2: Consider both single satellite deployment and multiple satellite deployment on support of S&F operation serving a UE.

With the above understanding, the next is to check whether there has to be any update on the current basic procedures, and we believe such basic procedures may include the following ones,

- (Initial) Attach procedure

- TAU procedure

- Control Plane CIoT EPS Optimisation

- User Plane CIoT EPS Optimisation

Moreover, for procedures of CP/UP CIoT EPS Optimisation, more detailed procedures including MO-EDT, MT-EDT and transmission using PUR can be further investigated.

Firstly, it should be noted that, according to the conclusion of TR 23.700-29, during Attach or TAU procedure many information are defined to be transferred including,

- The information indicating the attach or TAU procedure cannot be completed because of the S&F operation.

- Wait timer: The time UE should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN.

- The list of satellite IDs over which the UE may re-attempt the Attach/TAU procedure

- The extimated delivery time

It can be observed that such information are all included in NAS messages.

In our opinion, the reason why the MME onboard regenerative payload is able to generate the list of satellite IDs as well as the wait timer, is because MME onboard knows exactly the information on which satellites are providing S&F operation. Moreover, it is also possible that the MME onboard, with which the UE re-attempts the Attach/TAU procedure, have already progressed a little with the MME ground (e.g. have already communicated with HSS on the ground) before the UE re-attempt, in order to make some progress on Attach/TAU when UE re-attempts instead of merely storing the Attach Request or TAU Request message in another MME onboard.

From another aspect, we need to check whether the above information should also be indicated to eNB onboard from MME onboard.

For the information indicating the procedure cannot be completed because of the S&F operation, our understanding is that the eNB onboard needs to send RRC Release message to the UE. Then the question becomes how the eNB onboard knows it is time to send RRC Release message, and in our opinion, an indication is needed over S1AP to trigger the eNB onboard to send RRC Release message. Since at this stage the UE context has not been established over S1AP, the indication can be included in the DL NAS Transport message.

For the wait timer, we think the information is only useful to the UE. No need to also send it to the eNB onboard.

For the list of satellite IDs, SA2 conclusion is that the Satelllite IDs are based on SIB information broadcasted by eNB. So whether such information is needed to be transferred from MME onboard to eNB onboard depends on RAN2 progress.

For the estimated delivery time, we think the information is only useful to the UE. No need to also send it to the eNB onboard.

Proposal 3: RAN3 is kindly asked to discuss whether the indication related to the information indicating the procedure cannot be completed because of the S&F operation is needed over S1AP.

For CP/UP CIoT EPS optimization, our understanding is that for single satellite deployment, the current call flows for CP CIoT can be reused without further enhancement.

While for the UP CIoT,

Since SA2 has concluded that the S-GW is on the ground, so for S&F operation, the step 1 is performed when the service link is available while the feeder link is unavailable, and the steps (2/)3-7 can be performed when the feeder link is available while the service link is unavailable. So the question is,

- For UP CIoT, is there any need to send some information from eNB onboard to UE after step 1 and before step 2/3?

Another thing we need to notice is how to support the multiple satellite deployment, more specifically, the following questions need to be considered,

- For CP CIoT, whether the UE is able to send different RRC Early Data Request message to different eNBs onboard at different time?

- For UP CIoT, whether the UE is able to send different RRC Resume Request message to different eNBs onboard at different time?

- Whether and how to fetch the UE context between eNBs onboard?

Proposal 4: RAN3 is kindly asked to discuss the following open issues,

For single satellite deployment,

- For UP CIoT, is there any need to send some information from eNB onboard to UE after step 1 and before step 2/3?

For multiple satellite deployment,

- For CP CIoT, whether the UE is able to send different RRC Early Data Request message to different eNBs onboard at different time?

- For UP CIoT, whether the UE is able to send different RRC Resume Request message to different eNBs onboard at different time?

- Whether and how to fetch the UE context between eNBs onboard?

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for IoT NTN. The following proposals are provided,

Proposal 1: The eNB only onboard architecture to support the S&F satellite operation is not discussed in this release.

Proposal 2: Consider both single satellite deployment and multiple satellite deployment on support of S&F operation serving a UE.

Proposal 3: RAN3 is kindly asked to discuss whether the indication related to the information indicating the procedure cannot be completed because of the S&F operation is needed over S1AP.

Proposal 4: RAN3 is kindly asked to discuss the following open issues,

For single satellite deployment,

- For UP CIoT, is there any need to send some information from eNB onboard to UE after step 1 and before step 2/3?

For multiple satellite deployment,

- For CP CIoT, whether the UE is able to send different RRC Early Data Request message to different eNBs onboard at different time?

- For UP CIoT, whether the UE is able to send different RRC Resume Request message to different eNBs onboard at different time?

- Whether and how to fetch the UE context between eNBs onboard?

References