3GPP TSG-RAN WG3 Meeting #126	R3-247620

Orlando, USA, 18 – 22 November, 2024

Agenda item:	11.4

Source:	Nokia

Title:	Procedures for AI/ML Mobility Optimization in NR-DC

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #125bis, we made a simple agreement related to the procedures to trigger the collection of UE Performance measurements for NR-DC, namely:

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

According to this agreement, if the Data Collection Reporting Initiation procedure is configured between two nodes and if a node decides to select the other node for Dual Connectivity, UE Performance collection can be triggered using the Dual Connectivity SN Addition and MN-initiated SN change procedures.

There is an open point that needs to be addressed for the general case where Data Collection Reporting Initiation procedure might not be configured in advance between two nodes that establish Dual Connectivity with each other, namely:

Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion.

In TR 38.743 we have captured that the potential standards impacts from Mobility Optimization for NR-DC relate to enhancing Dual Connectivity procedures (e.g., SN Addition, MN-initiated SN Change) to trigger the collection of measured UE performance. In this contribution we provide our views on the introduced procedures to enable the collection and reporting of UE Performance measurements in NR-DC.

# 2	Discussion

## 2.1	Procedures to trigger the Collection and Reporting of UE Performance in Dual Connectivity

In TR 38.743 we have agreed that Mobility Optimization for NR-DC only assumes inference in the MN. This means that the main use case is limited to Dual Connectivity procedures such as SN Addition and MN-Initiated SN Change, without taking into account Conditional Dual Connectivity procedures and without considering inference in the SN. In addition, according to TR 38.743, the standards impacts of the NR-DC Mobility Optimization topic is focused on enhancing Dual Connectivity procedures e.g., SN Addition and MN-initiated SN Change in order to trigger the collection of measured UE Performance.

The basic scenario is illustrated in the example of Figure 1. In this example, a UE is connected to gNB 2. Even though for simplicity the figure doesn’t illustrate such details, a UE may be connected to gNB 2 after it has been handed over from another gNB 1. gNB 2 may also have configured Data Collection Reporting Initiation procedures to gNB 3. gNB 2 may initiate SN Addition by requesting gNB 3 to be added as an SN node. Upon the completion of the SN Addition procedure, the relationship between gNB2 and gNB3 will change to MN and SN respectively. The MN (gNB 2) may also decide subsequently to change SN (gNB3) to another gNB 4 by initiating SN Change procedures. Upon completion of the SN Change procedure the new SN for MN (gNB 2) will be gNB4.

Figure 1 Example of SN Addition Request and SN Addition Change

In the last meeting we agreed that if gNB 2 has configured a Data Collection Reporting Initiation procedure to gNB 3 then UE Performance collection can be triggered with SN Addition and MN-initiated SN Change procedures (same if gNB 3 has configured a Data Collection Reporting Initiation procedure to gNB 4).

Another scenario, that is still part of a “basic” Dual Connectivity scenario requiring standards support is when a UE is handed over. In the example of Figure 2, gNB1 has sent a DATA COLLECTION REQUEST message to gNB 2. The request may include for instance UE Performance measurements. gNB2 has acknowledged the request by sending a DATA COLLECTION RESPONSE to gNB1. In the course of time, a UE served by gNB 1 is handed over to gNB 2. In this example, after the handover, gNB2 sends a DATA COLLECTION UPDATE message with measurements, that may include for instance UE Performance measurements and subsequently decides to initiate SN Addition. This means that subsequent reports from gNB 2 will include measurements measured under Dual Connectivity.

Figure 2 Example of UE Handover where Dual Connectivity is Initiated by gNB 2 at a later point time after a UE is handed over to gNB 2. A mechanism to configure and report UE performance measurements from gNB3 to gNB2 is needed in this scenario.

Another example is illustrated in Figure 3. In this example, gNB 2 establishes Dual Connectivity to serve immediately the UE when it is handed over to its cell. The UE is served in Dual Connectivity in gNB 2 throughout the time it is served by the target cell.

Figure 3 Example where Dual Connectivity is initiated immediately when UE is handed over to gNB 2. A mechanism to configure and report UE performance measurements from gNB3 to gNB2 is needed in this scenario.

In all three scenarios, the MN will need to trigger the collection of UE Performance measurements from the SN and aggregate the total UE Performance to evaluate whether the Dual Connectivity decision was a good one or not. In the handover scenario, the target node (MN) would also need to report the aggregated UE Performance measurement to the source node, but this reporting would follow the Rel-18 mechanism.

Observation 1: In all the described scenarios, the MN will need to trigger the collection of UE Performance measurements from the SN and aggregate the total UE Performance to evaluate whether the Dual Connectivity decision was a good one or not.

In all three scenarios, the MN will be aware of the UE Performance measurements it needs to collect from SN. This can be either received through the Data Collection Request message from another node (gNB 1 in the above examples) or it can be determined internally by the MN. However, it is possible, as can be seen in the example of Figure 1 above, that in the course of Data Collection the SN chosen by MN changes and Data Collection procedures may not be configured towards all those SN nodes.

Observation 2: In the course of Data Collection, the MN may choose different SN nodes from which UE Performance needs to be collected and Data Collection procedures may not be configured towards all those SN nodes.

In the rest of the paper, we provide our views with respect to the procedures that we think are needed to address the above aspects. The following two approaches could be considered in order to configure the reporting of UE Performance measurements from SN serving a UE:

- Reuse Data Collection Reporting Initiation procedure defined over Xn for the configuration of UE Performance measurements between an MN and an SN. Reuse Data Collection Reporting procedure for the reporting of the collected UE Performance measurements by an SN node.

- Reuse existing Dual Connectivity (SN Addition and MN-initiated SN Change) procedures for the configuration of UE Performance between an MN and an SN. Introduce a new Class 2 procedure (or reuse Data Collection Reporting procedure) for the reporting of the collected UE Performance measurements by an SN node.

In our view, reusing the Data Collection Reporting Initiation procedure would be an overly complex way of configuring UE Performance measurements from SN. This is because an NR cell, e.g. the target cell served by gNB 2 in Figure 2 and Figure 3, may serve a wide geographical area and hence there may be a high number of candidate SNs, depending on geographical location of the served UEs or the UEs handed over to this cell. Furthermore, since the UE performance collection parameters (reporting duration, reporting periodicity) will be those provided by the source node (gNB 1), the gNB 2 may need to handle UE performance reporting configured by a possibly large number of neighbour nodes, providing possibly different reporting configurations. It seems not feasible to preconfigure each of the SNs under the gNB 2’s coverage with these different configuration parameters. Additionally, since the MN-SN relationship may be dynamic and change with time this would increase the number of possible configurations. Configuring Data Collection procedures from a gNB to all possible SNs that the gNB may choose for Dual Connectivity may be wasting resources, especially since UE Performance is UE-associated and the SN decision may be taken by the MN on a per UE-basis.

Observation 3: Configuring Data Collection procedures from a gNB to all possible SNs that the gNB may choose for Dual Connectivity may be wasting resources because UE Performance is UE-associated and the MN may select the SN on a per UE-basis.

On the other hand, reusing existing Dual Connectivity procedures such as SN Addition and SN Change could be a simple way in which an MN, that needs to receive UE Performance related to a specific UE, can configure, in a UE-associated way, UE Performance measurements from the SN that it decides to serve the UE.

Proposal 1: MN configures the collection of UE Performance measurements to an SN by reusing existing Dual Connectivity procedures, such as SN Addition and SN Change procedures.

Another aspect to address is how to enable the actual reporting of UE Performance measurements from SN to MN. Even though we were originally thinking that a new UE-associated Class 2 procedure is needed for the reporting of UE Performance measurements from Target SN to Target MN, we think that a simpler solution is to reuse Data Collection Reporting procedure for that. The reporting of UE Performance measurements in the Data Collection Reporting procedure can be associated with the configuring SN Addition and SN Change procedures through measurement IDs allocated via these procedures.

Proposal 2: Reuse the Class 2 Data Collection Reporting procedure for the reporting of UE Performance measurements from Target SN to Target MN.

## 2.2	UE(s) with Dual Connectivity in a Target Node

In NR-DC, after a gNB hands over a UE to another node (target node) and if the latter configures Dual Connectivity, the UE is served by two legs, the Master Leg and the Secondary Leg. If the target node does not configure Dual connectivity, the UE is served by only one leg. With MN-Terminated Dual Connectivity, the Core Network Termination and PDCP layer is hosted in MN and data is transmitted over both Master and Secondary legs. Accordingly, the measured UE Performance (e.g, Average Throughput DL) varies depending on whether UE is configured with Dual Connectivity or not.

The collection of UE Performance measurements is started at the target gNB after a successful handover to enable the source node to evaluate the performance of a handover decision based on the UE Performance at the target node. The target may configure Dual Connectivity immediately after a UE is handed over to the target node or at a later point in time. Clearly, the reported UE Performance measurements at the target node will be impacted by the configuration of Dual Connectivity.

Observation 4: The measured (reported) UE Performance varies depending on whether the target node configures Dual Connectivity for a UE that is handed over to it.

Since the decision to configure Dual Connectivity is totally under the control of the target node, the latter may decide to only configure some of the handed over UEs with Dual Connectivity and the rest with Single Connectivity. In this case, the source will observe a difference in the UE Performance feedback for the set of UE(s) configured with Dual Connectivity. Since the source node is completely unaware of those configurations at the target node, it cannot evaluate correctly the different UE Performance results received from the target. In MN-Terminated Dual Connectivity, the reporting node will always include the UE Performance measurements from both MN and SN.

Observation 5: In MN-Terminated Dual Connectivity, the MN will always need to aggregate the UE Performance measurements from both MCG and SCG.

Good values of measured UE performance may imply that the handover decision was indeed a good one, but it may also be that the handover decision was not optimal, though still the target node improved UE Performance by configuring Dual Connectivity. Clearly, the latter handover would be inferior to a handover with the same performance where the target does not configure Dual Connectivity, since no additional resources would need to be configured at the target for the same observed UE Performance.

Observation 6: Aggregating UE Performance measurements from MN and SN may hide from the receiving source node important information that can be used to determine the performance of a Dual Connectivity decision.

Consider the following scenarios:

- No NR-DC is configured in source node, but there is NR-DC configured in the target node: If a target node configures Dual Connectivity for a certain UE, there will be a change in the measured UE Performance at the target compared to the one at the source due to the additional resources that the target configures to serve the UE. In this case, the source may decide it needs UE Performance measurements only from MN (from MCG bearer).

- NR-DC is configured in both the source node and in the target node: If the UE was configured with Dual Connectivity in the source node (prior to the handover), the source node may only be interested in the UE Performance measurement for the UE only after the Dual Connectivity configuration is successfully completed. Otherwise, part of the measurements are collected from MN only while the rest include both MN  and SN.

Based on the above, we propose the following.

Proposal 3: Enable the reporting of UE Performance measurements on different node granularities, e.g., per a) Target MN only, b) per Target SN only, c) overall UE Performance, or d) overall UE Performance with information per MCG bearers, SCG bearers, and split bearers.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: In all the described scenarios, the MN will need to trigger the collection of UE Performance measurements from the SN and aggregate the total UE Performance to evaluate whether the Dual Connectivity decision was a good one or not.

Observation 2: In the course of Data Collection, the MN may choose different SN nodes from which UE Performance needs to be collected and Data Collection procedures may not be configured towards all those SN nodes.

Observation 3: Configuring Data Collection procedures from a gNB to all possible SNs that the gNB may choose for Dual Connectivity may be wasting resources because UE Performance is UE-associated and the MN may select the SN on a per UE-basis.

Proposal 1: MN configures the collection of UE Performance measurements to an SN by reusing existing Dual Connectivity procedures, such as SN Addition and SN Change procedures.

Proposal 2: Reuse the Class 2 Data Collection Reporting procedure for the reporting of UE Performance measurements from Target SN to Target MN.

Observation 4: The measured (reported) UE Performance varies depending on whether the target node configures Dual Connectivity for a UE that is handed over to it.

Observation 5: In MN-Terminated Dual Connectivity, the MN will always need to aggregate the UE Performance measurements from both MCG and SCG.

Observation 6: Aggregating UE Performance measurements from MN and SN may hide from the receiving source node important information that can be used to determine the performance of a Dual Connectivity decision.

Proposal 3: Enable the reporting of UE Performance measurements on different node granularities, e.g., per a) Target MN only, b) per Target SN only, c) overall UE Performance, or d) overall UE Performance with information per MCG bearers, SCG bearers, and split bearers.