3GPP TSG-RAN WG3 Meeting #126	R3-247622

Orlando, USA, 18 – 22 November, 2024

Agenda item:	11.4

Source:	Nokia

Title:	Discussion on Rel-18 UE Performance measurements

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #125 we made the following agreement:

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3#124.

In this contribution we provide our views regarding the definitions of the introduced UE Performance measurements in Rel-18. We think that the introduced UE Performance reporting should be consistent across use cases, that is for AI/ML slicing and AI/ML Mobility Optimization for NR-DC, enabling a common solution with respect to the entity that measures the different UE Performance measurements.

# Discussion

## 2.1	UE Performance Measurements

### 2.1.1	Packet Delay

According to TS 38.314, the Average Packet Delay measurement comprises different components when measured in UL and DL directions. Here, we only consider the RAN part of the Average Packet Delay in UL and DL.

Specifically, the Average Packet Delay in DL is the sum of D1+ D2+ D3+ D4, which are defined (in TS 28.558) as follows:

  - D1 (DL delay in over-the-air interface): This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface. It is measured at the gNB-DU. It is described in TS 28.558, clause 6.3.1.1.1.

  - D2 (DL delay on gNB-DU): This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. It is described in TS 28.558, clause 6.3.1.1.2.

  - D3 (DL delay on F1-U): This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. It is described in TS 28.558, clause 6.3.1.1.3.

  - D4 (DL delay in CU-UP): This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. It is described in TS 28.558, clause 6.3.1.1.4.

D1, D2, D3 and D4 measurements should be measured per DRB per UE.

Similarly, the Average Packet Delay in UL is the sum of D1+ D2.1+ D2.2+ D2.3+ D2.4, which are defined as follows:

  - D1 (UL PDCP packet average delay): This is the average PDCP delay in the UL. It is defined in TS 38.314, clause 4.3.1.1. It is a measurement performed by the UE to measure UL buffering delay in the PDCP layer, i.e. the average time a packet is buffered in the UE PDCP layer before it is transmitted. This measurement is obtained according to the definition in clause 4.3.1.1 of TS 38.314, named "UL PDCP Packet Average Delay per DRB per UE".

  - D2.1 (average over-the-air interface packet delay): This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2 (and in TS 28.558, clause 6.3.1.1.6). It is measured at the gNB-DU.

  - D2.2 (average RLC packet delay): It measures RLC packet delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.3 (and in TS 28.558, clause 6.3.1.1.7). It is measured at the gNB-DU.

  - D2.3 (average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured using the same metric as the average delay DL on F1-U defined in TS 28.558, clause 6.3.1.1.3. It is measured by the gNB-CU-UP.

  - D2.4 (average PDCP re-ordering delay in the UL): It measures PDCP re-ordering delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.4 (and in TS 28.558, clause 6.3.1.1.8). It is measured at the gNB-CU-UP.

In a split-architecture, some of the above delay components are measured in gNB-DU while some others are measured in gNB-CU-UP.  For example, for DL Packet Delay, D1 and D2 are measured at the gNB-DU while D3 and D4 are measured at gNB-CU-UP. In case of  UL Packet  Delay, D2.1 and D2.2 are measured in gNB-DU while D2.3, D2.4 are measured in gNB-CU-UP. It should also be noted that D1 is measured in the UE and gNB-CU-UP also needs this measurement to calculate the total UL packet delay.

Observation 1: Some delay components are measured at the gNB-CU-UP while some others in the gNB-DU.

In RAN3 #125, companies proposed two approaches to enable gNB-CU-CP to receive the overall Average Packet Delay measurements:

- gNB-CU-UP centric approach: gNB-DU sends the delay components measured by itself to gNB-CU-UP (over F1-U) and gNB-CU-UP forwards all the delay components (also measured by itself) to gNB-CU-CP (over E1). In this case, gNB-CU-UP aggregates all the delay components and sends them to gNB-CU-CP.

- gNB-CU-CP centric approach: gNB-DU sends the delay components measured by itself to gNB-CU-CP (over F1) and gNB-CU-UP sends the delay components measured by itself to gNB-CU-CP (over E1). In this case, gNB-CU-CP needs to aggregate the received delay components.

In our view, the gNB-CU-UP centric approach is the simplest since even though Average Packet Delay comprises different individual components, some of which are being measured in gNB-CU-UP and some in the gNB-DU, the overall delay is eventually known by the gNB-CU-UP. As can be seen in TS 38.415 [1], an NG-RAN node (gNB-CU-UP) as part of the QoS monitoring can send to UPF in the UL PDU SESSION INFORMATION (PDU Type 1) frame the DL Delay Result and UL Delay Result IEs:

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 1 UL PDU SESSION INFORMATION (PDU Type 1) Format (TS 38.415)

DL Delay Result IE indicates the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and in the gNB-DU) and the delay over Uu interface in milliseconds for the involved QoS flow. Similarly, the UL Delay Result IE indicates the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow. Even though the purpose of sending the UL PDU SESSION INFORMATION is to send control information elements related to the PDU session from NG-RAN to UPF, this delay information is available to NG-RAN.

Observation 2: The overall uplink delay measurement result, which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU), the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow, is available in the NG-RAN.

Proposal 1: We propose to consider gNB-CU-UP centric approach, where gNB-CU-UP aggregates the overall delay result in the DL and UL and sends it to gNB-CU-CP, similarly to DL Delay Result IE or UL Delay Result IE.

### 2.1.2 Average Throughput UL/DL

The total throughput of a UE in the target node after a handover can be impacted by multiple factors, including congestion, transport delay and radio-related impacts within the target node. In a split architecture, these different types of impacts are visible at different nodes, such as in the gNB-CU-UP or in the gNB-DU. Hence, a throughput measurement in the gNB-CU-UP would be able to measure throughput performance due to congestion while a throughput measurement in the gNB-DU would measure UE throughput performance due to radio or scheduling issues.

We proceed to discuss the different notions of throughput measurements available.

Average UE Throughput UL: One way to measure Average UE throughput in the UL is described in TS 28.558, clause 6.3.1.4.2 where throughput represents the average UE throughput in uplink for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

Average UE Throughput DL: One way to measure Average UE Throughput in the DL is through clause 6.3.1.4.1 in TS 28.558 where throughput measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

Average buffered UE throughput DL: An alternative way to measure throughput is through clause 5.1.1.3.7 of TS 28.552 which provides the average downlink buffered UE throughput per DRB on NRCellCU. For standalone operation, the DRBs are mapped with the same 5QI while for EN-DC with the same QCI. It measures per UE and per bearer throughput independent of traffic patterns and packet size. It supports monitoring in Dual Connectivity scenario and in NSA option3a and 3x. This measurement is measured by the gNB-CU-UP.

In case of split architecture, there are two possible options to measure throughput, namely a) by measuring throughput in the gNB-DU (RLC level throughput) or b) in the gNB-CU-UP (PDCP level throughput). Below, we list a number of observations on these different options of throughput measurements:

- Throughput measurement at the gNB-DU is not end-to-end. It only reflects the effect of the lower layers in the measured UE Performance e.g., throughput after handover. As part of the handover decision, UE measures the different neighbours’ radio quality and a specific target cell is selected for handover considering the radio quality and other resource utilization aspects.  Hence, measuring the RLC level throughput after the handover provides somewhat redundant information as the handover decision itself is made based on the radio quality. On the contrary, measuring the overall throughput at PDCP layer will be more useful since it reflects the overall performance which includes gNB-CU-UP aspects, e.g., related to congestion, F1-U transport issues together with radio level issues. If the throughput UE Performance is then sent to the source node, the latter will be able to identify if a handover decision that was based on radio quality only, also was good in terms of end-to-end throughput.

Using gNB-DU level measurement alone as feedback can result in incorrect evaluation at source node. This is because the PDCP level throughput may be quite different from the RLC-related measurements. For example, RLC level UE Performance can be good if the radio conditions are good, but at the same time the PDCP level UE Performance may be bad, e.g. due to congestion in gNB-CU-UP.  In another case, RLC level UE Performance may be poor if the radio conditions are poor, but PDCP level UE Performance may be good, that is there is no congestion in gNB-CU-UP. Hence, an RLC level measurement alone is not sufficient at the source node for evaluating the impact of its AI/ML based action.

- In NR-DC split-bearer, RLC level measurement is per-leg and can’t be aggregated to derive per-DRB measurement. To illustrate this with an example we consider the scenario where a UE is configured with one or more NR-DC split bearers after the handover to target node. If the throughout measurements are collected at RLC level, even though it is possible to collect the individual MCG and SCG leg level performance these measurements cannot be aggregated per-DRB since there is no common RLC SDU point for MCG and SCG legs. The RLC SDU point for MCG is located in MN while for SCG in SN node. Therefore, for MN, the information on how long and exactly in which time intervals the data is transmitted or kept in the buffer for SCG is hidden. The same holds for SN for MCG leg. Therefore, as neither MN nor SN knows the transmission/buffer status of data in the other leg, it is not possible to aggregate the MCG and SCG throughput per DRB. With PDCP level measurements, it is possible to collect per-bearer (QoS Flow) in all scenarios with or without NR-DC split bearers. A question that arises is whether then it is possible to send the per-leg (MCG/SCG) measurements directly to the source node without aggregation. However, also the source node will be missing required information to interpret the received measurements similarly as the target node (MN, SN). Therefore, such approach will not solve the problem.

Hence, it seems that PDCP level measurement of Average Throughput at gNB-CU-UP is the preferred option for measuring UL/DL Average Throughput.

Proposal 2: Measure Average UE Throughput UL/DL for UE Performance measurement at the gNB-CU-UP.

### 2.1.3 	DL Packet Loss

DL Packets may not be delivered to the UE due to two reasons. One reason is due to packet losses. Those include packets that are transmitted from the sending entity, e.g., gNB-CU-UP in case of DL PDCP SDUs, but are lost on F1 (split architecture) or Xn e.g., NR-DC split bearer or are not acknowledged from the receiving entity, e.g UE. Another reason is due to packets that are being discarded. Those include packets that could not be transmitted from the sending entity e.g., gNB-CU-UP in case of DL PDCP SDUs.

Observation 3: RAN3 agreement on Average Packet Loss DL considers only the packet losses, and not the dropped packets.

For Average Packet Loss DL, the following measurements are defined in TS 28.558/ TS 38.314.

- DL F1-U Packet Loss Rate (measured at DU of the DL packet loss on the F1-U interface) defined in TS 28.558, Clause 6.3.1.3.3: Packet Loss Rate measures the fraction of the PDCP SDU packets which are not successfully received at the gNB-DU. It is measured at gNB-DU.

- Packet Uu Loss Rate in the DL per DRB per UE defined in TS 38.314, Clause 4.2.1.5.1. This measurement measures packets that are lost at Uu transmission.

However, similarly to the case of throughput measurement, Packet Uu Loss Rate measurement at RLC level cannot be aggregated in case of NR-DC split bearer. This is because when the UE has an NR-DC split-bearer, there will be two independent Packet Uu loss rates in MCG and SCG legs. These individual measurements can’t be aggregated per-DRB level.

Different approaches could be considered to address this issue:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce Packet Loss Rate measurement at PDCP level

A measurement at gNB-CU-UP is preferable because it is end-to-end. On the other hand, a measurement at gNB-DU may be useful if the preference is to measure the packet losses in lower layers only.

Proposal 3 : RAN3 to discuss which approach is preferrable:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce a new Packet Loss Rate measurement at PDCP level

# 3	Procedures for the Configuration and Reporting of UE Performance measurements

As discussed in the previous section, we think that the measurement entity for meaningful UE Performance measurements should be the gNB-CU-UP. This means implies that gNB-CU-CP configures the reporting of UE Performance measurements from gNB-CU-UP and the latter reports those measurements to the gNB-CU-CP.

Two questions arise namely,

- Whether to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface or whether to re-use existing procedures e.g., for Bearer Context Setup, over E1 for the configuration of UE Performance measurements.

- How to trigger the UE Performance measurement collection.

We proceed with some observations related to the conditions when a gNB-CU-CP triggers the reporting of UE Performance measurements from gNB-CU-UP.

gNB-CU-CP requests UE Performance measurements from gNB-CU-UP only after it receives a UE Performance measurement collection trigger from a neighbouring node. This is materialised through a handover trigger.The Bearer Context Setup Request or Bearer Context Modification Request messages can be used to configure UE Performance reporting from gNB-CU-UP. This is because gNB-CU-CP can request UE Performance measurements from gNB-CU-UP when it receives a handed over UE. At that time it will know already a) the UE for which UE Performance measurements are requested, b) the corresponding UE Performance measurement configuration as well as c) the CU-UP that will be serving the UE.

In our view, reusing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1would be wasting network resources since those need to be configured between a gNB-CU-CP and all its gNB-CU-UPs in advance even though in the end the corresponding UEs for which UE Performance is requested may be served by a single gNB-CU-UP. This is because before a UE is handed over to a gNB-CU-CP, the CU-CP cannot know which of its CU-UPs will be serving the UE.

Proposal 4: Use the Bearer Context Setup Request or Bearer Context Modification Request messages to configure UE Performance Measurement Collection and Reporting to gNB-CU-UP.

One aspect that remains open is the actual reporting of UE Performance measurements from gNB-CU-UP to gNB-CU-CP. One option is to introduce a new (UE-associated) procedure over E1. In our view, we can introduce the Data Collection Reporting Class 2 procedure over E1 interface to transfer the UE Performance measurements from gNB-CU-UP to gNB-CU-CP. To correlate the Data Collection Reporting procedure to the Bearer Context Setup and Bearer Context Modification procedures, it is sufficient to introduce a Data Collection ID comprising the E1AP IDs corresponding to the gNB-CU-UP and gNB-CU-CP. In this way, the context of UE Performance Configuration provided in Bearer Context Setup and Bearer Context Modification procedures is correlated to the reporting in the Data Collection Reporting procedure.

Proposal 5: Introduce Class 2 Data Collection Reporting procedure over E1 to report UE Performance measurements from gNB-CU-UP to gNB-CU-CP.

# 4	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Some delay components are measured at the gNB-CU-UP while some others in the gNB-DU.

Observation 2: The overall uplink delay measurement result, which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU), the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow, is available in the NG-RAN.

Proposal 1: We propose to consider gNB-CU-UP centric approach, where gNB-CU-UP aggregates the overall delay result in the DL and UL and sends it to gNB-CU-CP, similarly to DL Delay Result IE or UL Delay Result IE.

Proposal 2: Measure Average UE Throughput UL/DL for UE Performance measurement at the gNB-CU-UP.

Observation 3: RAN3 agreement on Average Packet Loss DL considers only the packet losses, and not the dropped packets.

Proposal 3 : RAN3 to discuss which approach is preferrable:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce a new Packet Loss Rate measurement at PDCP level

Proposal 4: Use the Bearer Context Setup Request or Bearer Context Modification Request messages to configure UE Performance Measurement Collection and Reporting to gNB-CU-UP.

Proposal 5: Introduce Class 2 Data Collection Reporting procedure over E1 to report UE Performance measurements from gNB-CU-UP to gNB-CU-CP.

# References

[1]	3GPP TS 38.415	“NG-RAN; PDU Session User Plane Protocol”