3GPP TSG-RAN WG3 Meeting #126	R3-247639

Orlando, USA, 18 - 22 November, 2024

Agenda item:	19.2

Source: 	Huawei

Title: 	Further discussions on SBFD related information exchange

Document for:	Discussion

1. Introduction

In the RAN3#125bis meeting, there were agreements about SBFD related information exchange, listed as follow:

RAN3 agrees to Exchange the following information over Xn interface and F1 interface: - Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP) - Measurement resource configuration - Strongest DL beam information (FFS on the details) - CLI-mitigation request (FFS on the details)

In this contribution, we will further discuss the RAN3 impacts to support CLI mitigation, including SBFD configuration exchange, measurement procedure, and CLI mitigation procedure among gNBs.

2. Discussion

2.1	Exchange of SBFD configuration

In the previous meeting, RAN1 has achieved the following agreements about SBFD time and frequency configuration:

- SBFD subband time-domain location indication Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot SBFD subband frequency-domain location indication Agreement The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS Agreement For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

We think the gNB shall provide the SBFD time and frequency location configuration to other gNB in Xn Setup procedure with XnAP XN SETUP REQUEST message and XnAP XN SETUP RESPONSE message, and NG-RAN node Configuration Update procedure with XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message. Considering the SBFD time and frequency location configuration is cell-specific, similar to the exchange of current TDD UL-DL configuration and cell-specific frequency configuration, the SBFD time and frequency location configuration should also be provided at cell granularity.

Proposal 1: the SBFD time and frequency location configuration with cell granularity shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Currently, the TDD UL-DL Configuration Common NR IE and the Carrier List IE are included in Served Cell Information NR IE to indicate the time and frequency configuration of the cell.

| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |

From above, the TDD UL-DL Configuration Common NR IE is directly referred to the field tdd-UL-DL-ConfigurationCommon in TS 38.331, and the Carrier List IE is explicitly defined with relevant values. For the SBFD time configuration, RAN2 has not decided the signalling to indicate the time location of SBFD symbols, if the SBFD time configuration is extended in tdd-UL-DL-ConfigurationCommon IE in TS 38.331, the current XnAP specification can support the exchange of SBFD time configuration naturally; otherwise, new filed needs to be introduced to Served Cell Information NR IE. For frequency configuration, new fileds need to be introduced to Served Cell Information NR IE. Thus, how to enable the exchange of SBFD time and frequency configuration among gNBs depends on RAN2 work.

One more thing is, since RAN2 is in cc in the LS from RAN1, maybe there is a need for RAN3 to remind RAN2 to work on the IE design of SBFD time and frequency configuration, and from RAN3 perspective, RAN1 should also provide the detailed parameters for the SBFD timing and frequency info.

Proposal 1bis: The SBFD time and frequency configuration should be provided in Served Cell Information NR IE, the detailed sub-IE design is pending to RAN2 and RAN3 to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design.

Besides, the F1AP specification should also support to provide the SBFD time and frequency configuration of the served cell and obtain the SBFD time and frequency configuration of other cells. In the current F1AP specification, the TDD UL-DL Configuration Common NR IE and Carrier List IE are provided from gNB-DU to gNB-CU. Similarly, the SBFD time and frequency location configuration should also be provided from gNB-DU to gNB-CU in Served Cell Information IE, RAN3 can simply use the way of enhancement for Xn interface, the messages to be enhanced should be straight forward, i.e F1 setup and gNB-DU Configuration Update. Similarly, to make gNB-DU know the SBFD configuration of the cell of neighbour gNBs, the F1AP should also be enhanced to provide the time and frequency configuration of other cells from gNB-CU to gNB-DU, the message to be enhanced is also straight forward, since Neighbour Cell Information List is only carried via gNB-DU Configuration Update message.

Proposal 2: It is proposed RAN3 to agree that, in case of split architecture, the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE of F1AP specification.

Proposal 2bis: It is proposed RAN3 to agree to enhance the F1AP specification to support gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-DU Configuration Update message.

2.2	Exchange of measurement configuration

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

Based on this, the measurement resource configuration is required to be exchanged. Currently, the SSB resource configuration has been already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration, which is used by gNB to generate the measurement gaps based on the received SMTC information. While the NZP CSI-RS resource configuration hasn’t be provided in inter-node RRC message Measurement Timing Configuration.

While for NZP CSI-RS resource configuration, it was defined in NZP-CSI-RS-ResourceSet in Measurement Timing Configuration by RAN2, but RAN2 has not included it in the inter-node message. RAN3 could either refer to NZP-CSI-RS-ResourceSet IE directly, or ask RAN2 to include this IE in the inter-node message.

Observation 1: The SSB resource configuration has been already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration, as defined by RAN2; while for NZP CSI-RS resource configuration, RAN2 defined this IE but not included in the inter-node RRC message.

On the other hand, whatever the solution option is to be chosen by RAN1 or RAN2, we need to further consider the concrete IE design of XnAP and F1AP. As we could see that from the IE “Served Cell Information NR”, in which there is sub-IE “Measurement Timing Configuration” which further includes the MeasurementTimingConfiguration inter-node message for the served cell, which has included the SSB resource configuration. If RAN2 agree to introduce the NZP-CSI-RS-ResourceSet in Measurement Timing Configuration, the current XnAP and F1AP specification can support the exchange of SBFD time configuration naturally; otherwise, new filed need to be introduced to Served Cell Information NR IE.

Proposal 3: RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs.

Proposal 3bis: While for the NZP-CSI-RS resource configuration, it should be provided in Served Cell Information NR IE and, RAN3 to discuss the detailed sub-IE design, either asking RAN2 to work on the detailed parameters and IE design, or just refer directly to the IE NZP-CSI-RS-ResourceSet defined by RAN2.

In addition, whether there is need to have a measurement request to trigger the neighbour gNB to start CLI measurement on exactly indicated reference signals and report the measurement result should be considered. We think it is useful for the neighbour gNB to determine when to start measurement and which reference signals to measure, otherwise, the neighboring station needs to measure all reference signals. Since not all the “NZP-CSI-RS” or “SSB” in a cell supporting SBFD are used for gNB-gNB CLI measurement. It is better for gNB to indicate when to start measurement and which reference signals to measure. For this issue, there are three options may be used:

- Option 1: Indicate to perform CLI measurement in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure

- Option 2: Indicate to perform CLI measurement in Resource Status Reporting Initiation

- Option 3: Introduce new procedures to indicate to perform CLI measurement

Option 1 and option 2 are intend to reuse the existing procedure. Among these two options, we think option2 is preferred to trigger gNB-gNB CLI measurement, since the existing framework of Resource Status Reporting Initiation procedures can support controlling CLI measurements with smaller changes, the gNB1 can trigger the gNB2 to perform measurement for the indicated cell and control the report periodicity. While for the option1, the interface management procedures are used to update the cell level information, which are not suitable to trigger gNB-gNB CLI measurement. For option3, we think the similar procedures as option2 can be introduced.

Proposal 4: The gNB needs to send measurement request to trigger the neighbour gNB to perform gNB-gNB CLI measurement on exactly indicated reference signals, RAN3 to discuss which procedure to trigger the neighbour gNB to perform gNB-gNB CLI measurement, with the following options:

- Option 1: Indicate to perform CLI measurement in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure

- Option 2: Indicate to perform CLI measurement in Resource Status Reporting Initiation

- Option 3: Introduce new procedures to indicate to perform CLI measurement

Here two stage 2 TPs to 38.420 and 38.470 of option3 are attached in the Annex part, stage3 TP to TS 38.423 in [3].

## 2.3	Exchange of measurement result

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of strongest DL beam information

Firstly, RAN3 should discuss whether to introduce the new procedure or enhance the existing procedure to support strongest DL beam information exchange. For this issue, there are three options may be used:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures to provide CLI measurement result

Option 1 and option 2 are intend to reuse the existing procedure. Among these two options, we think option2 is preferred to support gNB-gNB CLI measurement, since the existing framework of Resource Status Reporting procedures can support controlling CLI measurements with smaller changes. For option3, we think the similar procedures as option2 can be introduced.

Proposal 5: It is proposed RAN3 discuss which procedure to support the strongest DL beam information exchange, with the following options:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures to provide CLI measurement result

Here two stage 2 TPs to 38.420 and 38.470 of option3 are attached in the Annex part, stage3 TP to TS 38.423 in [3].

2.4	Exchange of CLI-mitigation request

In the LS from RAN1, RAN3 is request to support the exchange of following information:

- Information exchange of CLI-mitigation request

And in the WID, the following requirement is listed:

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

Based on these, we think RAN3 only needs to introduce the signalling of CLI-mitigation request while no need to discuss the node behavior upon reception of request, which basically includes two options:

- Option 1: Introduce an indication IE such as “CLI-mitigation requested” in NG-RAN node Configuration Update procedure

- Option 2: Introduce New procedure to indicate CLI-mitigation request

Proposal 6: It is proposed RAN3 discuss which procedure to be used to support CLI-mitigation request, in the following options:

- Option 1: Introduce an indication IE such as “CLI-mitigation requested” in NG-RAN node Configuration Update procedure

- Option 2: Introduce New procedure to indicate CLI-mitigation request

Here the option 2 is preferred, and two stage 2 TPs to 38.420 and 38.470 and a draft LS are attached in the Annex part, stage3 TP to TS 38.423 in [3].

3. Conclusion

In this contribution, we analyzed the specification impacts of gNB-gNB mitigation. The following observation and proposals were provided:

Exchange of SBFD configuration

Proposal 1: the SBFD time and frequency location configuration with cell granularity shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Proposal 1bis: The SBFD time and frequency configuration should be provided in Served Cell Information NR IE, the detailed sub-IE design is pending to RAN2 and RAN3 to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design.

Proposal 2: It is proposed RAN3 to agree that, in case of split architecture, the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE of F1AP specification.

Proposal 2bis: It is proposed RAN3 to agree to enhance the F1AP specification to support gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-DU Configuration Update message.

Exchange of measurement configuration

Observation 1: The SSB resource configuration has been already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration, as defined by RAN2; while for NZP CSI-RS resource configuration, RAN2 defined this IE but not included in the inter-node RRC message.

Proposal 3: RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs.

Proposal 3bis: While for the NZP-CSI-RS resource configuration, it should be provided in Served Cell Information NR IE and, RAN3 to discuss the detailed sub-IE design, either asking RAN2 to work on the detailed parameters and IE design, or just refer directly to the IE NZP-CSI-RS-ResourceSet defined by RAN2.

Proposal 4: The gNB needs to send measurement request to trigger the neighbour gNB to perform gNB-gNB CLI measurement on exactly indicated reference signals, RAN3 to discuss which procedure to trigger the neighbour gNB to perform gNB-gNB CLI measurement, with the following options:

- Option 1: Indicate to perform CLI measurement in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure

- Option 2: Indicate to perform CLI measurement in Resource Status Reporting Initiation

- Option 3: Introduce new procedures to indicate to perform CLI measurement

Exchange of measurement result

Proposal 5: It is proposed RAN3 discuss which procedure to support the strongest DL beam information exchange, with the following options:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures to provide CLI measurement result

Exchange of CLI-mitigation request

Proposal 6: It is proposed RAN3 discuss which procedure to be used to support CLI-mitigation request, in the following options:

- Option 1: Introduce an indication IE such as “CLI-mitigation requested” in NG-RAN node Configuration Update procedure

- Option 2: Introduce New procedure to indicate CLI-mitigation request

4. Reference

[1]. RP-241614, WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2]. R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation

[3]. R3-247640	BL CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

Annex A: TP to TS 38.420

## 5.2	Functions of Xn-C

### 5.2.1	Xn-C interface management and error handling functions

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 5.2.X	CLI management function

This function allows exchanging CLI measurement configuration and result, and CLI-mitigation request between NG-RAN nodes, such that the NG-RAN node can manage CLI appropriately.

# 6	Xn interface procedures

## 6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

## 6.2	Control plane protocol procedures

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 6.2.X	CLI management procedures

The CLI management procedures are used by NG-RAN nodes to indicate CLI measurement configuration and result, and CLI-mitigation request to each other.

-	CLI measurement Initiation

-	CLI measurement Reporting

-	CLI mitigation request

Annex B: TP to TS 38.470

## 5.2	F1-C functions

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 5.2.X	CLI management function

This function allows exchanging CLI measurement configuration and result, and CLI-mitigation request between the gNB-CU and the gNB-DU.

## 6.1	Control plane procedures

### 6.1.1	Interface Management procedures

The F1 Interface management procedures are listed below:

-	Reset procedure;

-	Error Indication procedure;

-	F1 Setup procedure;

-	gNB-DU Configuration Update procedure;

-	gNB-CU Configuration Update procedure;

-	gNB-DU Resource Coordination procedure;

-	gNB-DU Status Indication procedure;

-	F1 Removal procedure;

-	Network Access Rate Reduction procedure;

-	DU-CU TA Information Transfer procedure;

-	CU-DU TA Information Transfer procedure.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<TEXTS OMITTED>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 6.1.X	CLI management procedures

The CLI management procedures are listed below:

-	CLI measurement Initiation

-	CLI measurement Reporting

-	CLI mitigation request

Annex C: Draft LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Title:	LS to RAN2 on SBFD information exchange among gNBs for CLI mitigation

Release:	Release 19

Work Item:	NR_duplex_evo

Source:	TSG RAN WG3

To:	TSG RAN WG2, TSG RAN WG1

Cc:

Contact person:	Yang Xudong

yangxudong@huawei.com

# 1	Overall description

RAN3 has started to work on the SBFD information exchange among gNBs for CLI mitigation, according to the received LS in R1-2407533 from RAN1.

RAN3 has agreed that SBFD time and frequency configuration info should be exchanged between neighbour gNBs in Served Cell Information NR IE, RAN3 also reached the common understanding that RAN3 needs RAN2 to provide the concrete IE design on SBFD time and frequency configuration info, which should be based on parameters provided by RAN1.

RAN3 has also agreed that measurement resource configuration should be exchanged between neighbour gNBs in Served Cell Information NR IE, RAN3 also reached the common understanding that measurement resource configuration should be provided in inter-node RRC message Measurement Timing Configuration, while such message doesn’t include NZP-CSI-RS-ResourceSet.

With the agreement and common understanding reached in RAN3, RAN3 would like to ask RAN1 to work on the detailed parameters, and ask RAN2 to work on the IE design of SBFD time and frequency configuration info and inter-node RRC message.

# 2	Actions

To TSG RAN1

ACTION: 	RAN3 respectfully asks RAN1 to provide the detailed parameters for SBFD time and frequency configuration info.

To TSG RAN2

ACTION: 	RAN3 respectfully asks RAN2 to work on the IE design of SBFD time and frequency configuration info and inter-node RRC message.

# 3	Dates of next TSG RAN WG3 meetings

RAN3#127	2025-02-17 - 2025-02-21		Athens, GR

RAN3#127-bis	2025-04-07 - 2025-04-11		China, CN