3GPP TSG-RAN WG3 Meeting #126							 R3-247649

Orlando , US, 18th – 22nd Nov 2024

Agenda Item:		12.3

Source:		Baicells

Title:		TP to BL CR for TS 38.300 on NR Femto

Document for:		Discussion and Approval

- Introduction

This TP is based on the agreed BLCR R3-245754 and R3-247648(Discussion on the NR Femto Architecture and Functionalities).

- TP to 38.300

# 4	Overall Architecture and Functional Split

## 4.X	Support of NR Femtos

### 4.X.1	Architecture

============================Start of the change ===============================

An NR Femto shall only connect to a single NR Femto GW at one time when the NR Femto connects via the NR Femto GW:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

### 4.X.2	Functional Split

The NR Femtos and NR Femto GW have the following specifics in case of connection to the NR Femto GW:

-	The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW;

-	NG Flex function shall be hosted by NR Femto GW instead of the NR Femtos.

- 	Selection of an AMF i.e. NAS node selection function (NNSF) at UE registration is hosted by NR Femto GW instead of the NR Femtos:

-	Upon reception of the assistance information used for assisting NR Femto GW to select an AMF (e.g. UE's temporary identifier, GUAMI, slicing information), the NR Femtos shall transfer them to NR Femto GW through NGAP procedure.

- 	NR Femto GW terminates non-UE associated NG application part procedures towards the NR Femto and towards the AMF.

- 	NR Femto GW relays UE-associated NG application part messages between the AMF serving the UE and the NR Femto serving the UE, except INITIAL UE MESSAGE received from the NR Femto with some IEs modificaitons (if any) by the NR Femto GW.