3GPP TSG-RAN WG3 Meeting #126                                                         R3-247700

Orlando, USA, 18th - 22th Nov 2024

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discussion on renewable energy based LBO

Document for:	Discussions & Approval

1	Introduction

In this RAN3 meeting, RAN3 received the LS in [1] from SA5, requesting support for the exposure of renewable energy information within RAN for LBO, such as e.g. carbon emission, carbon emission efficiency and renewable energy usage. In this paper, we discuss how to respond.

2	Discussion

In the LS[1], SA5 describes the use case of renewable energy based LBO, and the requirement of  the exposure of renewable energy information within RAN. see below:

--------------------------------------------------------------------------------------------------------------------------

3GPP SA5 (Management, Orchestration and Charging) has initiated its Release 19 study on energy efficiency and energy saving aspects of 5G networks and services. In this context, 3GPP SA5 identified a use case where neighbor gNBs, which are candidates for distributed LBO, are powered using different types of energy sources, some of which being renewable, some others being not, and where the decision for choosing the target gNB to receive the load transferred from source gNB is taken based on such considerations.

As an illustration, in the figure below, gNB A and gNB C are both candidates to receive traffic load transferred from gNB B, the only difference between gNB A and gNB C being that gNB A is fully powered by renewable energy. With the guidance of renewable energy usage policy, gNB B gives gNB A higher priority and chooses gNB A.

To support this capability, the procedure of Distributed SON LBO would need enhancements to consider new energy related information, such as e.g. carbon emission, carbon emission efficiency and renewable energy usage.

--------------------------------------------------------------------------------------------------------------------------

After review the LS, we agree that renewable energy-based load balancing and status exchange would be valuable enhancements to SON MRO and network energy-saving capabilities. However, we observe that the current scope of NES Work Items from Rel-15 to Rel-19 does not address any renewable energy tracking, exchange, or usage prioritization across RAN nodes.

Furthermore, we note that SA1 has already developed several energy related use cases addressing renewable energy and carbon-awareness in TR 22.882 (Study on Energy Efficiency as a Service Criterion) for Rel-19 study. Specifically:

- Renewable Energy Consumption Information Exposure :enabling network-to-subscriber renewable energy information sharing.

- Carbon-Aware Communication Services: providing carbon emissions insights based on energy consumption and carbon intensity.

- These use cases focus on renewable energy information exposure primarily at the network-to-subscriber level, rather than inter RAN node exchanges. Therefore, we suggest that further development on renewable energy related functionalities in SON should also take SA1's energy efficiency framework and requirements into account to ensure consistency and avoid duplication.

In conclusion, given that renewable energy-based load balancing introduces new features related to renewable energy that go beyond the current NES Work Items, we propose that this enhancement be considered as part of a new feature set in Release 20, such as in a Rel-20 NES Work Item. This timeline will provide sufficient time to incorporate renewable energy-based load balancing and renewable energy source prioritization into SON and NES mechanisms, as well as any other required functionalities from TR 22.882, if needed.

Proposal 1: Renewable energy related feature could be considered as part of a new feature set in Release 20, such as in a Rel-20 NES Work Item. This timeline will provide sufficient time to incorporate renewable energy-based load balancing and renewable energy source prioritization into SON and NES mechanisms, as well as any additional required functionalities from other WGs, such as SA1/SA2 (e.g., in TR 22.882), if needed.

Based on the above proposal, the draft reply LS is prepared in [2].

Reference

- R3-247114, LS to RAN3 on renewable energy based LBO, SA5

- R3-247701, [draft]Reply LS on renewable energy based LBO, ZTE Corporation