3GPP TSG-RAN WG3 #126                                                 R3-247712

Orlando, US, 18th – 22th Nov, 2024

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 37.340) MRO for R18 mobility mechanisms

Document for:	other

Introduction

In last RAN3 meeting, the following agreements are captured:

LTM:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change“successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

MRO for CHO with Candidate SCG(s)

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - MRO for LTM

RACH-less and RACH-based access

First, once UE starts RACH-less access, there would be no fall back from RACH-less to RACH-based access.

UE is triggered to perform LTM via a MAC CE (LTM Cell Switch Command), not legacy handover command. And LTM is specified in subsection 9.2.3.5 in TS38.300, not the subsection 9.2.3.6.

More specifically, the following text describes UE behavior when receiving LTM Cell Switch Command captured in TS38.321.

- 5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2); 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

We can see that UE would perform RACH less LTM cell switch when Timing Advance is configured. Then UE performs RACH-less access until RACH failure occurs, i.e. t304 expires. There is not fallback mechanism in above procedure.

Proposal 1: If valid TA is provided via LTM Cell Switch Command or measured by UE, UE would perform RACH-less LTM cell switch until T304 expires. There is no fall back mechanism after RACH-less procedure start.

UE-based TA related optimization:

For UE-based TA measurement, the description on RACH-less and RACH-based LTM is as below:

In the case where UE-based TA measurement is configured, but no valid TA value is provided in the cell switch command, the UE applies the valid TA value by itself if available. The UE performs RACH-less LTM cell switch upon receiving the cell switch command whenever a valid TA value is available. If no valid TA value is available, the UE performs RACH-based LTM cell switch.

Network could configure UE-based TA measurement. If UE cannot calculate valid TA, UE would perform RACH-based LTM cell switch which may delay handover procedure.

Observation 1: If UE is configured UE-based TA measurement, but UE cannot calculate valid TA, UE would perform RACH-based LTM cell switch.

This case may be seen as a near failure case. Optimization may be needed for RACH-less LTM rather than RACH-based LTM.

From UE perspective, how to calculate TA value is UE implementation for which it is not needed to perform optimization.

From network perspective, network could know UE-based TA measurement is configured but RACH-based LTM is performed in target DU. Next time network may not configure UE-based TA measurement, i.e. network could make optimization by its own.

Proposal 2: For the case of UE-based TA measurement, calculating TA value is UE implementation which it is not needed to perform optimization. Also, the optimization on whether to configure UE-based TA is based on network implementation which has no specification impact.

Network-based TA related optimization:

For the timing of triggering early TA acquisition procedure, if triggering too early, the TA may be outdated when starting LTM cell switch. On the contrary, if triggering too late, the early TA acquisition procedure may be not finished. So, it may be needed to discuss how to trigger early TA acquisition procedure at the correct time.

Early TA acquisition procedure by PDCCH order may periodic triggering. Shorter periodicity may better meet the requirement of TA validity with the cost of impact UE and network performance while longer periodicity may not provide valid TA when triggering LTM cell switch. So, Network may decrease the periodicity step by step until RACH-less handover successful rate is satisfied. As for RACH-less handover successful rate, RACH-less handover failure can be achieved by RLF Report which has been agreed to include access type according to RAN2 agreement:

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

Proposal 3: The optimization on avoiding network based TA outdates can be performed based on network implementation e.g., shorten TA obtain period.

There are some RACH-less LTM failure scenarios discussed in last meeting, and the following scenarios after RACH-less LTM cell switch execution is proposed:

- RACH-less LTM failure followed by successful RACH-based LTM recovery;

- RACH-less LTM failure followed by RACH-based LTM recovery failure;

- RACH-less LTM failure followed by reestablishment (if recovery is not configured).

These similar scenarios have been supported in CHO, and the recovery and reestablishment cell information has been included in RLF Report. LTM may just reuse these IEs to support above scenarios.

Proposal 4: It is proposed to reuse CHO recovery and reestablishment cell information included in RLF Report to support above scenarios.

Frequent and repeated LTM switch

The goal of LTM is to enable a serving cell change via L1/L2 signaling, in order to reduce the latency, overhead and interruption time. UE may perform frequent and repeated handover within candidate cell list. Whether there is Ping-Pong issue during frequent and repeated LTM handover needs consideration. For example, the UE is switched between cell 1 and cell 2 repeat for many times may be normal in LTM but it would be ping-pong in L3 handover.

Proposal 5: It is proposed for RAN3 to discuss whether ping-pong exists in LTM and whether it needs to be optimized by UHI which is recorded by CU.

During the following LTM triggered handover, more than one times of failure or near failure maybe occur. Since the LTM triggered handover may be frequent and quick, network may be not able to fetch SHR or/and RLF Report before the next failure or near failure occurs which may cause overwriting issue. RAN3 may discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. LS may be sent to ask RAN2 to record multiple failures or near failures information in UE, if needed.

Proposal 6: It is proposed for RAN3 to discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. If needed, LS to RAN2 may be sent.

  - MRO for CHO with candidate SCGs

    - Problem definition and detection mechanism

According to the understanding of some companies, problem definition and detection mechanism of CHO with candidate SCGs may reuse those of CHO and CPA/CPC. But there may be some issues if we simply divide CHO with candidate SCGs into independent CHO and CPA/CPC. In other words, optimization on CHO with candidate SCGs is not equal to MRO for CHO + MRO for CPA/CPC.

First, MCG failure + SCG failure is a common case which does not mean MCG failure and SCG failure occurs at the same time. Most of time it may be MCG failure occurs first and then SCG failure occurs, or SCG failure occurs first and then SCG failure occurs. For example, too late failure type, UE moves out of courage of PSCell and PCell gradually without triggering handover. SCG failure occurs first due to the courage of PSCell is smaller than that of PCell most of the time, then MCG failure occurs when UE moves out of courage of PCell. Therefore, MCG failure after SCG failure or SCG failure after MCG failure is a common case should be supported.

Proposal 7: MCG failure + SCG failure is a common case.

For MCG failure + SCG failure case, network may receive SCG failure report and MCG failure report separately. If network does not know these two reports were generated from the same CHO with candidate SCGs and same UE, duplicated optimization on CHO with candidate SCGs (i.e. RLF Report triggered optimization and SCGFailureInformation message triggered optimization) may be performed for one-time mobility procedure.

Moreover, different failure type may be detected by MCG failure and SCG failure. For example, too early failure type for MCG failure and to wrong cell handover failure type for SCG failure is detected.

Observation 2: Issue of duplicated or controversial optimization on one-time CHO with candidate SCGs caused by separate SCG failure report and RLF report should be considered by RAN3.

We also believe it is too difficult to use two failure reports to make optimization. Obviously, to make the optimization on CHO with candidate SCGs simple, using only one failure to detect failure type is feasible even MCG failure + SCG failure occurs.

On how to use only one failure to define failure type, the solution may be:

Solution 1: the first occurred failure may be selected to perform MRO.

Solution 2: MCG failure would be selected because MCG connection is more important than SCG.

Solution 1 may have less specification impact than solution2 because it is hard for solution 2 to handle the case of SCG failure followed by MCG failure. UE and network do not know whether MCG failure may occurs after SCG failure. For the first occurred SCG failure, it is actually 1b, 2b, 3b, 7b, 8b case which should trigger MRO at once when network receiving SCGFailureInformation message. If RLF report indicating MCG failure is received by network later, RLF report has to be enhanced to correlate with the previous SCG failure information in order to withdraw the MRO triggered by SCG failure because solution 2 only perform optimization on MCG failure.

Proposal 8: It is proposed for RAN3 to discuss how to use only one failure (MCG failure or SCG failure) to define the failure type for MRO for MCG failure + SCG failure case e.g., based on the first failure or always MCG failure.

For solution 1, stage 2 description may capture the following text:

CHO with candidate SCGs reuses the problem definition and detecting mechanism of CHO and CPA/CPC. In case of CHO and CPAC fail concurrently, the MRO only consider the first receiving failure report in network, while the second failure report is ignored.

For solution 2, stage 2 description may capture the following text:

CHO with candidate SCGs reuses the problem definition and detecting mechanism of CHO and CPA/CPC. In case of CHO and CPAC fail concurrently, MRO only for CHO base on RLF report, while SCG failure report is ignored.

Proposal 9: It is proposed for RAN3 to discuss the above stage-2 problem definition and detection mechanism.

    - Forwarding mechanism and interface impact

The forwarding mechanism may be updated to capture the following case.

For the SCG failure after successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B, due to PCell 2/PSCell B taking as last serving node, PCell 2 may receive SCGFailureInformation message from UE or third node. After MRO analysis, PCell 1/PSCell A may be informed to perform optimization. Xn interface message may be enhanced to provide SCGFailureInformation message for the same UE from PCell 2 to PCell 1/PSCell A.

Forwarding mechanism may capture the following description:

Target MN receiving the SCGFailureInformation message may inform the source MN by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. If the CPAC is triggered by SN, the source MN may further forward the SCGFailureInformation message to SN.

Proposal 10: It is proposed to capture the above forwarding mechanism for CHO with candidate SCGs.

Proposal 11: It is proposed to enhance XN interface message to transfer SCGFailureInformation message for the same UE from target MN to source MN.

    - Information in RLE Report

For information in RLF Report, the agreement and FFS is listed below:

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

In last RAN2 meeting, the relate agreements are captured below:

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

For the fulfilled events, it has been included in RLF Report and SCGFailureInformation message for CHO only case and CPAC only case as the following text captured in TS38.331.

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]],

[[

choCandidate-r17                        ENUMERATED {true}                                                           OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                             OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                                    OPTIONAL,

firstTriggeredEvent-r17                 ENUMERATED {condFirstEvent, condSecondEvent}                            OPTIONAL

}                                                                                                               OPTIONAL

]],

[[

firstEntering-r18                       ENUMERATED {true}                                                           OPTIONAL

]]

}

As for CHO with candidate SCG, we may just follow CHO and CPAC. But how to record them together depend on RAN2

Proposal 12: How to record fulfilled events of CHO and CPAC together for CHO with candidate SCG is depend on RAN2 design.

    - Others

For case 9, PCell and PSCell execution conditions cannot be met simultaneously. Most of time, it may cause too late handover failure type and execution condition should be optimized to be more easily met in order to trigger handover earlier. But for the following PCell and PSCell coverage illustrated in figure1, it is useless to solve too late handover failure type by just optimizing execution condition, because for any execution conditions, both PCell and PSCell execution conditions could not be satisfied at the same time due to no common coverage for PCell and PSCell.

Figure 1: not suitable to configure CHO with candidate SCG scenario

For the scenario in figure 1, the UE should first perform handover from PCell 1 to PCell 2 and then perform PSCell change from PSCell A to PSCell B. So, it is needed to optimize whether configure CHO with candidate SCG or not rather than CHO with candidate SCG configuration itself.

Proposal 13: For case 9, PCell and PSCell execution conditions cannot be met simultaneously. The cause may be:

- execution conditions is sub-optimized;

- Coverage issue, i.e. no common coverage to meet both PCell and PSCell execution conditions.

According to above understanding, before deciding the detection mechanism on CHO with candidate SCGs failure type, the text on scenario validation may be captured first in stage2 description. The following text may be captured:

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. If no common handover coverage for PCell and PSCell, CHO with candidate SCG(s) should not be configured.

Proposal 14: It is proposed to capture the text on common coverage detection for CHO with candidate SCGs in stage2 description.

In the figure 1 scenario, CHO with candidate SCG may not be triggered. So, legacy handover/PSCell change, CHO/CPC only, or CHO with target SCG should be used instead. If only configure CHO with candidate SCG, RLF may occur because CHO with candidate SCG would not be executed.

Therefore, since we may not able to aware about the scenario ahead of the time, legacy handover/PSCell change, CHO/CPC only, or CHO with target SCG may be configured as a backup together with CHO with candidate SCG in order to avoid RLF.

Proposal 15: It is proposed to support MRO for a case that UE is configured with legacy handover/PSCell change, CHO/CPC only or CHO with target SCG together with CHO with candidate SCGs.

  - MRO for Subsequent CPAC

Forwarding mechanism

In last RAN3 meeting, forwarding mechanism has been discussed and the stage2 description still needs further check.

Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

There are some differences due to the following principles applied to subsequent CPAC captured in TS37.340.

-	For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC).

-	For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC.

-	For both MN and SN initiated inter-SN subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

In case of SN initiated PSCell change, the initiating node may be not the source SN and target SN for current PSCell change when receiving SCGfailureInformation message from UE. For example, SN1 as source SN initiates subsequent CPC, UE perform PSCell change from SN1 to SN2, and then PSCell change from SN2 to SN3. If SCG failure occurs in SN3, SN2 and SN3 as current source SN and target SN may be involved to perform MRO. SN1 as the initiating node which provide the candidate cell list may be also optimized.

Observation 3: For MRO for S-CPAC, the initiating node may be not source SN and target SN for the current PSCell change in case of SN initiated CPC.

For optimization on candidate cell selection, PSCell candidate list is first selected by initiating node, and then selected by target SN.

For MN initiated S-CPAC, MN and target SN should be involved to optimize candidate cell selection.

For SN initiated S-CPAC, it is the initiating SN and target SN to optimize candidate cell selection.

Proposal 16: The initiating node and target SN is responsible for optimize candidate cell selection.

For optimization on execution conditions, for the initial execution of S-CPAC, MN or SN as the initiating node generate the execution conditions.

For the following execution of subsequent S-CPAC, in case of no intra-SN PSCell change, candidate SN generates the execution conditions. Here the candidate SN refers to the source SN of PSCell change.

In case of intra-SN PSCell change, the source SN (or last serving SN) generates the execution conditions.

Proposal 17: For initial CPC execution, initiating node should perform execution conditions optimization. For following CPC, it is source SN to optimize execution conditions. The source SN is related to the latest PSCell change.

With above understanding, we could conclude that the forwarding mechanism would include two steps:

Step 1: optimization on execution conditions optimization

Step 2: optimization on PSCell candidate list

The related nodes to perform execution conditions optimization and candidate PSCell list selection are listed in following table.

|  | Node for execution conditions optimization | Node for candidate PSCell list selection |
| --- | --- | --- |
| Initial S-CPAC initiated by MN | MN(initiating node) | MN, target SN |
| Following S-CPAC initiated by MN | Source SN | MN, target SN |
| Initial S-CPAC initiated by SN | Initiating SN | Initiating SN, target SN |
| Following S-CPAC initiated by SN | Source SN | Initiating SN, target SN |

The forwarding mechanism captured in TS37.340 in last meeting is as below:

For SN initiated CPC or a following S-CPC initiated by MN, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

According to above table, for SN initiated CPC, it is initiating SN to perform candidate PSCell list selection, not source SN. There are three nodes i.e., source SN, initiating SN and candidate/target SN involved for following SN initiated CPC optimization, but now above text only cover source SN and candidate/target SN, without initiating SN.

Observation 4: There are three nodes i.e., source SN, initiating SN and candidate/target SN involved in following SN initiated CPC optimization

Following S-CPAC initiated by MN, it is MN to perform initial candidate PSCell list selection and candidate or target SN select part of them. There are three node i.e., source SN, MN and candidate/target SN involved subsequent CPC optimization, but now it only cover source and candidate/target SN, without MN.

Observation 5: There are three nodes i.e., source SN, MN and candidate/target SN involved for following S-CPAC optimization initiated by MN.

According to above discussion, we update the forwarding mechanism which includes the above related nodes:

For CPA or MN initiated CPC or S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation. For the following S-CPC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the Source SN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to initiating SN, and initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that wrong PSCell selection occurred in the candidate or target SN. MN then sends the SCG Failure Information Report message to the candidate or target SN. If no wrong PSCell selection occurs, MN sends the SCG Failure Information Report message to the source SN to perform the final MRO related optimisation.

We also try to reuse legacy CPAC forwarding mechanism for S-CPAC as below:

For CPA or MN initiated CPC or S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report messageto the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation in case of initial S-CPAC initiated by MN and MN initiated CPAC, or MN sends the SCG Failure Information Report message to source SN performs the final MRO related optimisation in case of following CPAC initiated by MN.

For SN initiated CPC or SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the SN which initiating S-CPAC or CPC, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report messageto the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation in case of initial S-CPAC initiated by SN and SN initiated CPC, or MN sends the SCG Failure Information Report message to source SN for final MRO related optimisation in case of following CPAC initiated by SN.

The above two forwarding mechanisms are also provided in annex for discussion.

Proposal 18: It is proposed for RAN3 to discuss above two forwarding mechanisms in TP for TS37.340.

UHI

There is an issue raised in RAN3 meeting as below:

Support UHI and SCG UE History Information transmission during S-CPAC procedure to avoid the ping-pong or unnecessary PSCell change

To solve the issue on ping-pong or unnecessary PSCell change, there are mainly two steps:

The first step is how to record UHI and detect ping-pong occurrence.

The second step is to optimize configuration to solve the issue.

For the first step, UHI could be updated by MN during S-CPAC procedure. The related text in TS37.340 is as below:

The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

Since MN is aware of the each PSCell selected during S-CPAC procedure according to above text, MN could correlate PCell and PSCell, i.e. update the correlated UHI which includes PCell and PSCell information.

After the analysis of UHI, MN can detect ping-pong issue occurs during S-CPAC procedure. To solve the issue, i.e. the second step, there may be the following solutions:

Solution 1: MN sends UHI to every candidate SN each time UHI is updated.

Solution 2: MN sends UHI to every candidate SN when ping-pong issue is detected by MN.

Solution 3: MN sends UHI to the candidate SN which ping-pong issue is related to when detecting ping-pong issue by MN.

Solution 4: it is not needed to solve ping-pong issue immediately, i.e. the next time configuring S-CPAC, ping-pong issue will be solved according to the previous collected UHI.

Considering above solutions, we think not every candidate SN should be informed because they may be not involved in ping-pong issue, and it is also not needed to inform SN each time UHI is updated because ping-pong issue may not exist. So, solution 1 and 2 is not preferred.

For solution 3, after detecting ping-pong issue, MN informs the related SN about the updated UHI. The candidate cell add/release may be optimized at once which may lead to many interactions between every candidate cells because the change on each candidate cell should be informed to every other candidate cells in S-CPAC procedure.

For solution 4, considering the complexity to perform the modification of S-CPAC configuration, UHI can be used for the next S-CPAC, i.e. not perform optimization for current S-CPAC. Moreover, most of the times MRO is performed based on the statistics, not one time failure. Therefore, it is not needed to perform MRO in a hurry or each time Ping-Pong issue occurs. Comparing with solution 3, solution 4 also has no specification impact. So, solution 4 is preferred.

Proposal 19: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

Information in SCGfailureInformation message

Network could save UE context in MN which may include the initiating node information. But MN may only know the initiating SN node ID and may not know the PSCell ID in the initiating node which is optional present in S-NODE CHANGE REQUIRED message. For the initiating node, it may be not selected as candidate cell and cannot save UE context when UE is not connected to this node. So, it is hard for network to save PSCell information in initiating node in case of SN initiating S-CPAC case.

As we discussed before, the initiating node, specifically, initiating PSCell may be required to perform MRO. Therefore, SCGfailureInformation may be enhanced to include source initiating PSCell ID.

Proposal 20: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Conclusions

MRO for LTM:

Proposal 1: If valid TA is provided via LTM Cell Switch Command or measured by UE, UE would perform RACH-less LTM cell switch until T304 expires. There is no fall back mechanism after RACH-less procedure start.

Observation 1: If UE is configured UE-based TA measurement, but UE cannot calculate valid TA, UE would perform RACH-based LTM cell switch.

Proposal 2: For the case of UE-based TA measurement, calculating TA value is UE implementation which it is not needed to perform optimization. Also, the optimization on whether to configure UE-based TA is based on network implementation which has no specification impact.

Proposal 3: The optimization on avoiding network based TA outdates can be performed based on network implementation e.g., shorten TA obtain period.

Proposal 4: It is proposed to reuse CHO recovery and reestablishment cell information included in RLF Report to support above scenarios.

Proposal 5: It is proposed for RAN3 to discuss whether ping-pong exists in LTM and whether it needs to be optimized by UHI which is recorded by CU.

Proposal 6: It is proposed for RAN3 to discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. If needed, LS to RAN2 may be sent.

MRO for CHO with candidate SCGs:

Proposal 7: MCG failure + SCG failure is a common case.

Observation 2: Issue of duplicated or controversial optimization on one-time CHO with candidate SCGs caused by separate SCG failure report and RLF report should be considered by RAN3.

Proposal 8: It is proposed for RAN3 to discuss how to use only one failure (MCG failure or SCG failure) to define the failure type for MRO for MCG failure + SCG failure case e.g., based on the first failure or always MCG failure.

Proposal 9: It is proposed for RAN3 to discuss the above stage 2 problem definition and detection mechanism.

Proposal 10: It is proposed to capture the above forwarding mechanism for CHO with candidate SCGs.

Proposal 11: It is proposed to enhance XN interface message to transfer SCGFailureInformation message for the same UE from target MN to source MN.

Proposal 12: How to record fulfilled events of CHO and CPAC together for CHO with candidate SCG depend on RAN2 design.

Proposal 13: For case 9, PCell and PSCell execution conditions cannot be met simultaneously. The cause may be:

- execution conditions is sub-optimized;

- Coverage issue, i.e. no common coverage to meet both PCell and PSCell execution conditions.

Proposal 14: It is proposed to capture the text on common coverage detection for CHO with candidate SCGs in stage2 description.

Proposal 15: It is proposed to support MRO for a case that UE is configured with legacy handover/PSCell change, CHO/CPC only or CHO with target SCG together with CHO with candidate SCGs.

MRO for Subsequent CPAC:

Observation 3: For MRO for S-CPAC, the initiating node may be not source SN and target SN for the current PSCell change in case of SN initiated CPC.

Proposal 16: The initiating node and target SN is responsible for optimize candidate cell selection.

Proposal 17: For initial CPC execution, initiating node should perform execution conditions optimization. For following CPC, it is source SN to optimize execution conditions. The source SN is related to the latest PSCell change.

Observation 4: There are three nodes i.e., source SN, initiating SN and candidate/target SN involved in following SN initiated CPC optimization

Observation 5: There are three nodes i.e., source SN, MN and candidate/target SN involved in following S-CPAC optimization initiated by MN.

Proposal 18: It is proposed for RAN3 to discuss above two forwarding mechanisms in TP for TS37.340.

Proposal 19: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

Proposal 20: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Reference

[1]

TP for TS 37.340

- 10.18.x	CHO with candidate SCG(s).

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. If no common handover coverage for PCell and PSCell, CHO with candidate SCG(s) should not be configured.

Problem definition and detecting mechanism:

Solution 1:

CHO with candidate SCGs reuses the problem definition and detecting mechanism of CHO and CPA/CPC. In case of CHO and CPAC fail concurrently, the MRO only consider the first receiving failure report in network, while the second failure report is ignored.

Solution 2:

CHO with candidate SCGs reuses the problem definition and detecting mechanism of CHO and CPA/CPC. In case of CHO and CPAC fail concurrently, MRO only for CHO base on RLF report, while SCG failure report is ignored.

Forwarding mechanism:

Target MN receiving the SCGFailureInformation message may inform the source MN by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. If the CPAC is triggered by SN, the source MN may further forward the SCGFailureInformation message to SN.

### 10.18.3	Conditional PSCell addition or change failure

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

Option 1: Add new paragraph CPAC forwarding mechanism for S-CPAC as below:

For CPA or MN initiated CPC or S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation. For the following S-CPC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the Source SN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to initiating SN, and initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that wrong PSCell selection occurred in the candidate or target SN. MN then sends the SCG Failure Information Report message to the candidate or target SN. If no wrong PSCell selection occurs, MN sends the SCG Failure Information Report message to the source SN to perform the final MRO related optimisation.

Option 2: Reusing legacy CPAC forwarding mechanism for S-CPAC as below:

For CPA or MN initiated CPC or S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report messageto the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation in case of initial S-CPAC initiated by MN and MN initiated CPAC, or MN sends the SCG Failure Information Report message to source SN performs the final MRO related optimisation in case of following CPAC initiated by MN.

For SN initiated CPC or SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the SN which initiating S-CPAC or CPC, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report messageto the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation in case of initial S-CPAC initiated by SN and SN initiated CPC, or MN sends the SCG Failure Information Report message to source SN for final MRO related optimisation in case of following CPAC initiated by SN.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>