3GPP TSG-RAN3 Meeting #126	R3-24xxxx

Orlando, USA, 18th - 22nd Nov 2024

- Title:	(TP to 38.410) AMF selection for NR Femto

- Source:	ZTE Corporation

- Agenda Item:	12.3

- Document for:	Discussion, Decision

# TP for NR femto

## TP for 38.410

5.7 NAS Node Selection function The interconnection of NG-RAN nodes to multiple AMFs is supported in the 5GS architecture. Therefore, a NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account other information (e.g. slicing information, onboarding indication) to determine the AMF. When the NG-RAN node is configured to ensure that the selected AMF serves the country where the UE is located, as described in TS 23.501 [8], the NG-RAN node takes into account UE location information, if available, when determining the AMF. When the NR Femto connects to a Femto GW, selection of an AMF at UE attachment is performed by the NR Femto GW instead of the NR Femto. This functionality is located in the NG-RAN node and enables proper routing via the NG interface. On NG, no specific procedure corresponds to the NAS Node Selection Function.