3GPP TSG-RAN WG3 Meeting #126	R3-247770

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	8.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # 4_N3mb

Document for:	Approval

# Introduction

CB: # 4_N3mb

- Check the details above

- Provide the CRs and reply LS if agreeable

(moderator - Nok)

Summary of offline disc R3-247770

# For the Chairman’s Notes

For NGAP agree R3-247802 revision of 7764 revision of 7252

For E1AP agree R3-247781 revision of 7248

For 38.300 agree R3-7782 revision of 7249

Agree Reply LS to CT4 in 7783 revision of 7250

# Discussion-First round

## NG interface

There are two potential solutions:

- Solution 1: Reuse the existing NG-U Failure indication IE introduced for broadcast which for location dependent can include a list of area session IDs and associated failure indication

- Solution 2: Add a new multicast NG-U failure indication which is for location independent or for one area session indicated at upper level in the multicast session update transfer request.

According to CT4 CR, there is either zero or one area session ID in a multicast session update request.

Therefore, the existing IE with a list of area session IDs is not needed.

Adopt solution 2.

Use R3-247252 as baseline but with updating the tabular to align the encoding with the asn.1/

## E1 CR

There are three options proposed:

- Solution 1: Reuse the existing MC Bearer Context NG-U TNL Info at NG-RAN Request IE

- Solution 2: Add a new codepoint to the MC Bearer Context NG-U TNL Info at NG-RAN Request IE.

- Solution 3: Add a new MC Bearer Context NG-U TNL Info at NG-RAN Replace Request IE in the MC Bearer Context To Modify IE in the MC BEARER CONTEXT MODIFICATION REQUEST message

Solution 1 has backwards compatibility issue because the existing IE is used for new creation of transport address and not replacement, therefore CU UP can fail the request if an existing address already exist.

Solution 2 and 3 are equivalent but solution 3 was found a bit cleaner because can set its own criticality.

Agree solution 3.

Use R3-247248 as baseline with updating the criticality to “reject”.

## Reference to TS 23.527 in TS 38.300

It is acknowledged that TS 23.527 and recovery scenarios are not even referenced in current TS 38.300 and such pointer could be useful.

The draft CR in R3-247249 could be extended to also reference TS 23.527 for the broadcast receovery scenario in the broadcast section.

Use R3-247249 as baseline with updating with broadcast case.

# Conclusion, Recommendations

# References