3GPP TSG-RAN WG3 Meeting #126	R3-247771

Orlando, FL, USA, November 18th – 22nd 2024

Agenda Item:	12.2

Source:	Ericsson

Title:	Reply to SA2 Regarding Additional ULI for WAB-Nodes

Document for:	Agreement

# Introduction

In this paper we discuss the reply to the LS from SA2 in R3-244019, with respect to Issue 2. The LS was received during the SI.

# Discussion

With respect to issue 2, the LS from SA2 states:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

The issue at hand is how to formulate the ULI that is to be reported to the UE’s 5GC in case the WAB-gNB and WAB-MT connect to different PLMNs. In that respect, clause 8.5 of TR 23.700-6 considers 3 alternatives for additional ULI:

>>>>>>>>>>>>>Start of excerpt from TR 23.700-6<<<<<<<<<<<<<<<<

8.5	KI#5: Support of location services for UEs when MWAB(s) is involved Conclusion

The following aspects are taken as way forward during normative work:

-	Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11.

-	The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

-	TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

-	If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase.

-	option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

-	option 2: geo-location of the MWAB.

-	option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

-	The AMF may provide the received Additional ULI to LMF in LCS request.

NOTE:	If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

>>>>>>>>>>>>>End of excerpt from TR 23.700-6<<<<<<<<<<<<<<<<

In Option 1, the TAC and cell ID pertain to the WAB-gNB’s cell serving the UE. This TAC and cell ID may change, depending on the node’s location. The WAB-gNB is configured by the OAM with the mapping between the geo-location and the TAC/cell ID. Based on its location, the WAB-gNB determines the TAC/cell ID that it is supposed to use. The additional ULI format defined for mIAB (consisting of cell ID and TAC) can be reused to communicate the additional ULI to the AMF. However, it should be noted that, for mIAB, the TAC and cell ID pertain to the cell where the mIAB-MT is connected, while, in the WAB case, they pertain to the WAB-gNB’s cell serving the UE. Note that a similar principle for additional ULI has been adopted for NTN, where virtual cell IDs are mapped to physical cell IDs.

With respect to Option 2, we notice that specifying geo-location information of WAB-gNB as the additional ULI may be a substantial deviation from the current framework (which defines additional ULI as the cell ID/TAC), because, as of today, the geo-location information is not used by the LMFs for UE location (it is used for TRP location). Note that, in addition to the AMF, the ULI should be provided to other entities in the CN as well, such as the SMF, charging system etc., meaning that the CN other nodes in addition to the LMF would be impacted. Therefore, this option should not be considered by RAN3.

Further, although Option 3 is identical to the additional ULI format currently used for UEs served by the mIAB-nodes, it is unclear how the UE’s 5GC should interpret the TAC/Cell ID from another PLMN. This format was indeed specified for mIAB, but the RAN WG work only considered intra-PLMN scenarios for mIAB, as opposed to WAB, where the WAB-MT and WAB-gNB may connect to different PLMNs.

Based on the above, we prefer Option 1, given that the additional ULI format from mIAB can be reused, and the impact incurred is limited to the mapping provided to the WAB-gNB by the OAM. We also note that Option 1 is line with the general principle of OAM providing location-dependent configurations to the WAB-gNB.

Proposal: Additional ULI for WAB consists of WAB-gNB cell’s TAC and Cell ID, which are determined by the WAB-gNB based on WAB-node’s geo-location.

# Conclusion

This paper discusses the reply LS on SA2 regarding FS_VMR_Ph2, with respect to Issue 2. The following is proposed:

Proposal: Additional ULI for WAB consists of WAB-gNB cell’s TAC and Cell ID, which are determined by the WAB-gNB based on WAB-node’s geo-location.